3GPP TSG-RAN-WG4 Meeting # 111 	R4-2407171

# Fukuoka, Japan, 20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.101-1 to add new HP-NRCA combinations for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Softbank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_TDD_NR_CADC_SUL_R18 |  |  |  |  |  |  | Date: |  | 2024-05-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following HP-NRCA combinations with single PC2 and PC1.5 UL are added. CA_n1A-n3A-n41A with UL n41A(PC1.5) CA_n1A-n41A-n77A with UL n41A(PC1.5) CA_n1A-n41A-n77(2A) with UL n41A(PC1.5) CA_n1A-n28A-n41A with UL n41A(PC1.5) CA_n3A-n41A-n77A with UL n41A(PC1.5) CA_n3A-n41A-n77(2A) with UL n41A(PC1.5) CA_n3A-n41A-n77(2A) with UL n77A(PC1.5) CA_n3A-n28A-n41A-n77A with UL n77A(PC2 and PC1.5) CA_n1A-n3A-n41A-n77A with UL n41A(PC1.5) CA_n1A-n3A-n41A-n77A with UL n77A(PC1.5) CA_n1A-n3A-n28A-n41A with UL n41A(PC1.5) CA_n1A-n28A-n41A-n77A with UL n41A(PC1.5) CA_n3A-n28A-n77A-n79A with UL n77A(PC2 and PC1.5) Additions of PC2 new UL band combinations as follows. CA_n3A-n28A-n77A-n79A with UL CA_n3A-n77A (PC2) CA_n3A-n28A-n77A-n79A with UL CA_n28A-n77A (PC2) CA_n3A-n28A-n77A-n79A with UL CA_n3A-n79A (PC2) CA_n3A-n28A-n77A-n79A with UL CA_n28A-n79A (PC2) CA_n3A-n28A-n77A-n79A with UL CA_n77A-n79A (PC2) All fallback combinations for CA_n3A-n28A-n77A-n79A with these PC2 UL combinations are already defined in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requested PC2/PC1.5 NR CA combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed CA are not supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3.2 Table 5.5A.3.2-1a 5.5A.3.3 Table 5.5A.3.3-1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected parts omitted]

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: Void

Table 5.5A.3.2-1a

Table 5.5A.3.2-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A | CA_n1A-n3A CA_n1A-n5A CA_n3A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3(2A)-n7A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1(2A)-n3A-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3B-n7A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1(2A)-n3B-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1(2A)-n3(2A)-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3B-n7B | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n8A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n18A | CA_n1A-n3A CA_n1A-n18A CA_n3A-n18A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n18 | 5, 10, 15 |  |
| CA_n1A-n3A-n20A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n26(2A) | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n1A-n3B-n26(2A) | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 201, 301 |  |
| CA_n1A-n3B-n28A | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3B-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3A-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3B-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A)-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A)-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n40A | CA_n1A-n3A CA_n1A-n40A CA_n3A-n40A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n41A | n417,9 CA_n1A-n3A CA_n1A-n41A7 CA_n3A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n67A | CA_n1A-n3A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n77A | n777,9 CA_n1A-n3A CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n77(2A) | n777,9 CA_n1A-n3A CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n77(3A) | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n78C | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3B-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A CA_n3A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3B-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3B-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3B-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3B-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3(2A)-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3(2A)-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3(2A)-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3(2A)-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3A-n105A | CA_n1A-n3A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n5A-n7A | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n5A-n7B | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n5A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  | CA_n1A-n5A CA_n1A-n28A CA_n5A-n28A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n40A | CA_n1A-n5A CA_n1A-n40A CA_n5A-n40A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n78C | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n1A-n5A-n79A | CA_n1A-n5A CA_n1A-n79A CA_n5A-n79A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n8A | CA_n1A-n7A CA_n1A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n26(2A) | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n7B-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n26(2A) | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n28A | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n38A10 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n7A-n38A10 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n7A-n40A | CA_n1A-n7A CA_n1A-n40A CA_n7A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n7A-n67A | CA_n1A-n7A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 901, 100 |  |
|  | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7B-n78A | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n7B-n78(2A) | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n78C | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n7A-n105A | CA_n1A-n7A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n8A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n28 | 10, 15, 20 |  |
| CA_n1A-n8A-n40A | CA_n1A-n8A CA_n1A-n40A CA_n8A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | - | n1 | 5, 10, 15, 20 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n8A-n78C | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n1A-n8A-n78(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
| CA_n1A-n8A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A | CA_n1A-n18A CA_n1A-n28A CA_n18A-n28A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n1A-n18A-n41A | CA_n1A-n18A CA_n1A-n41A CA_n18A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n77A | n777 CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n77(2A) | CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n20A-n67A | CA_n1A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n20A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n28A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n28A-n40A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n40B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n1A-n28A-n41A | n417,9 CA_n1A-n28A CA_n1A-n41A7 CA_n28A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n46A | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n1A-n28A-n46C | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n1A-n28A-n46D | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n1A-n28A-n46(2A) | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n1A-n28A-n75A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n28A-n77A | n777,9 CA_n1A-n28A CA_n1A-n77A7 CA_n28A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n77(2A) | n777,9 CA_n1A-n28A CA_n1A-n77A7 CA_n28A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n28A-n77(3A) | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n28A-n78C | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n102A | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n28A-n102B | CA_n1A-n28A CA_n1A-n102A CA_n1A-n102B CA_n28A-n102A CA_n28A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n28A-n102C | CA_n1A-n28A CA_n1A-n102A CA_n1A-n102C CA_n28A-n102A CA_n28A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n28A-n102D | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n28A-n102E | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n28A-n102(2A) | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40A-n77A | CA_n1A-n40A CA_n1A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40A-n77(2A) | CA_n1A-n40A CA_n1A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n40A-n78A | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n40B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n40A-n105A | CA_n1A-n40A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n41A-n77A | n417,9 n777,9 CA_n1A-n41A7 CA_n1A-n77A7 CA_n41A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n77(2A) | n417,9 n777,9 CA_n1A-n41A7 CA_n1A-n77A7 CA_n41A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n41A-n77(3A) | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n1A-n41A-n79A | CA_n1A-n41A CA_n1A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n46A-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46C-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46D-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46(2A)-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46A-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | 10, 20, 40, 60, 80 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n46C-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n46D-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n46(2A)-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n67A-n78A | CA_n1A-n78A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n67A-n78(2A) | CA_n1A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n77(2A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n77(3A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78A-n79A5 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78A-n102A | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n78A-n102B | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102B CA_n78A-n102A CA_n78A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n78A-n102C | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102C CA_n78A-n102A CA_n78A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n78A-n102D | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n78A-n102E | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n78A-n102(2A) | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n78(2A)-n102A | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n78(2A)-n102B | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102B CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n78(2A)-n102C | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102C CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n78(2A)-n102D | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n78(2A)-n102E | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n78(2A)-n102(2A) | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n78A-n105A | CA_n1A-n78A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n2A-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n41A | CA_n2A_n5A CA_n2A_n41A CA_n5A_n41A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A CA_n48B | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n5A-n48(A-B) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2(2A)-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n5A-n66(3A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n5A-n77A | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77C | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n77(2A) | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n77A | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n77(2A) | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n7A-n12A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2A-n7A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n7A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n7A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n12A-n41A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n2A-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n66(3A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n12A-n71A | CA_n2A-n12A CA_n2A-n71A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n12A-n77A | n777,9 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n77A | n777,9 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n77(2A) | n777,9 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n77(2A) | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n66(3A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n14A-n77A | n777,9 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n77(2A) | n777,9 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n77A | n777,9 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n77(2A) | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n30A | CA_n2A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n29A-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n29A-n66A | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n77A | n777,9 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n77A | n777,9 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n77(2A) | 777,9 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n77(2A) | n777 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2(2A)-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2(2A)-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n30A-n66(3A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n30A-n77A | n777,9 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n77(2A) | n777,9 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n77A | n777,9 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n30A-n77(2A) | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n41A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n41A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(A-B)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48B-n66A | CA_n48B CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | n777,9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A7 CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48(2A)-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77C | n777,9 CA_n48B CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77A | n777,9 CA_n48B CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n77A | n777,9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n66A-n77A | n777, 9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n66A-n77A | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(2A)-n77A | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77C | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n66A-n77(2A) | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n66(2A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n66(2A)-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n66A-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66(2A)-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66(3A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(3A)-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66A-n78A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n2A-n71A-n77A | CA_n2A-n71A CA_n2A-n77A CA_n71A-n77A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n71A-n77(2A) | CA_n2A-n71A CA_n2A-n77A CA_n71A-n77A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n71A-n78A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A-n5A-n7B | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3A-n5A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  | CA_n3A-n5A CA_n3A-n28A CA_n5A-n28A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n79A | CA_n3A-n5A CA_n3A-n79A CA_n5A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n8A | CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
| CA_n3A-n7A-n20A | CA_n3A-n7A CA_n3A-n20A CA_n7A-n20A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n7A-n26(2A) | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n7B-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n26(2A) | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n7A-n26A | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n7A-n26(2A) | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n7B-n26A | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n7B-n26(2A) | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n7A-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3B-n7A-n28A | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3B-n7B-n28A | CA_n7B CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7A-n38A10 | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3B-n7A-n38A10 | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3(2A)-n7A-n38A10 | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n7A-n67A | CA_n3A-n7A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n75A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78C | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n3A-n7B-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7B-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n79A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n79C | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3B-n7A-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3(2A)-n7A-n79A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3(2A)-n7A-n79C | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3A-n7A-n105A | CA_n3A-n7A CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n8A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n77A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n77(2A) | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n79A | CA_n3A-n8A CA_n3A-n79A CA_n8A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n18A-n28A | CA_n3A-n18A CA_n3A-n28A CA_n18A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77A | n777 CA_n3A-n18A CA_n3A-n77A7 CA_n18A-n77A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77(2A) | n777 CA_n3A-n18A CA_n3A-n77A7 CA_n18A-n77A7 | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n20A-n67A | CA_n3A-n20A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n28A | CA_n3A-n20A CA_n3A-n28A CA_n20A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n20A-n78A | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n78(2A) | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A CA_n78(2A) | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3A-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n28A-n38A | - | n3 | 5, 10, 15, 20, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n38 | 5, 10, 15, 20, 30, 40 |  |
| CA_n3A-n28A-n40A | CA_n3A-n28A CA_n3A-n40A CA_n28A-n40A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n40 | 20, 40 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A | n417,9 CA_n3A-n28A CA_n3A-n41A7 CA_n28A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n41B | CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n28A-n77A | n777,9 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n77(2A) | n777,9 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  | CA_n77(2A)7 | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n28A-n77(3A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n78C | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n78(2A) CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n28A-n78(2A) | CA_n78(2A) CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n28A-n79A | n797,9 CA_n3A-n28A CA_n3A-n79A7 CA_n28A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n38A-n40A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n38A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n40A-n78A | CA_n3A-n40A CA_n3A-n78A CA_n40A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n40A-n105A | CA_n3A-n40A CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n77A-n79A4 | n777,9 n797,9 CA_n3A-n77A7 CA_n3A-n79A7 CA_n77A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(2A)-n79A4 | n777,9 n797,9 CA_n3A-n77A7 CA_n3A-n79A7 CA_n77A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(3A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n40A-n41C | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n3A-n40A-n77A | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n40A-n77(2A) | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n41A-n77A | n417,9 n777 CA_n3A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A7 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41B-n77A | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n77(2A) | n417,9 n777,9 CA_n3A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n41A-n77(3A) | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n3A-n41A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n41A-n79A | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41A-n79C | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n41C-n79A | CA_n41C CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41C-n79C | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n67A-n78A | CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n67A-n78(2A) | CA_n78(2A) CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n75A-n78A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n78A-n79A | n787,9 | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n78A-n79C | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3B-n78A-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3B-n78A-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3(2A)-n78A-n79A | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3(2A)-n78A-n79C | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3A-n78A-n105A | CA_n3A-n78A CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n5A-n7A-n25A | CA_n5A-n7A CA_n5A-n25A CA_n7A-n25A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n5A-n7A-n25(2A) | CA_n5A-n7A CA_n5A-n25A CA_n7A-n25A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n25 | CA_n25(2A) |  |
| CA_n5A-n7A-n28A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n5A-n7A-n66A | CA_n5A-n7A CA_n5A-n66A CA_n7A-n66A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n5A-n7A-n77A | n777,9 CA_n5A-n7A CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n7A-n77(2A) | n777,9 CA_n77(2A)7 CA_n5A-n7A CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n5A-n7A-n77(3A) | n777,9 CA_n77(2A)7 CA_n5A-n7A CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n5A-n7A-n78A | CA_n5A-n78A7 CA_n7A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n7B-n78A | CA_n5A-n78A7 CA_n7A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n12A-n77A | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n12A-n77(2A) | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n14A-n77A | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n14A-n77(2A) | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n29A | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n29 | 5, 10 |  |
| CA_n5A-n25A-n41A | CA_n5A-n25A CA_n5A-n41A CA_n25A-n41A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
| CA_n5A-n25(2A)-n41A | CA_n5A-n25A CA_n5A-n41A CA_n25A-n41A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n25 | CA_n25(2A) |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25A-n77A | n777,9 CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  | CA_n5A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n77A7 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n77A | n777,9 CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n77(2A) | n777,9 CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n77(3A) | n777,9 CA_n77(2A) CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5A-n25(2A)-n77(2A) | n777,9 CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n78A | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n78A | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n78(2A) | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n78(2A) | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n28A-n78A | CA_n5A-n28A CA_n5A-n78A CA_n28A-n78A | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n28A-n79A | CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n28A-n105A | CA_n5A-n28A CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n5A-n29A-n66A | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n29A-n77A | n777 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n29A-n77(2A) | n777 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n30A-n66A | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n5A-n30A-n66(2A) | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n5A-n30A-n66(3A) | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n5A-n30A-n77A | n777,9 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n77(2A) | n777,9 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n40A-n78A | CA_n5A-n40A CA_n5A-n78A CA_n40A-n78A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n40 | 58, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |
| CA_n5A-n41A-n66A | CA_n5A-n41A CA_n5A-n66A CA_n41A-n66A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n5A-n41A-n77A | CA_n5A-n41A CA_n5A-n77A CA_n41A-n77A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n41A-n77(2A) | CA_n5A-n41A CA_n5A-n77A CA_n41A-n77A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n77 | CA_n77(2A) |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(A-B)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48B-n66A | CA_n48B CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48B-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C BCS1 |  |
| CA_n5A-n48(2A)-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77A | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77A | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77(2A) | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66(3A)-n77A | n777,9 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(3A)-n77(2A) | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66A-n77C | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77(2A) | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 CA_n77(2A)7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n5A-n66A-n77(3A) | CA_n77(2A) CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n66(2A)-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n78A-n79A | CA_n5A-n78A CA_n5A-n79A CA_n78A-n79A | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n8A-n28A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n7A-n8A-n40A | CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n7A-n8A-n78A | CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n12A-n25A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n12A-n66A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n7A-n12A-n71A | CA_n7A-n12A CA_n7A-n71A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n12A-n77A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n20A-n67A | CA_n7A-n20A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | See n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n20A-n78A | CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n20A-n78(2A) | CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n25A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n25A-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n77(2A) | n777,9 CA_n77(2A)7 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n77(3A) | n777,9 CA_n77(2A)7 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n25(2A)-n77(2A) | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25(2A)-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n77(2A) | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n77(2A) | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n78A | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7(2A)-n25A-n78A | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7A-n25(2A)-n78A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7(2A)-n25(2A)-n78A | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7A-n25A-n78(2A) | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25A-n78(2A) | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n25(2A)-n78(2A) | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7(2A)-n25(2A)-n78(2A) | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n26A-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n26(2A)-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n26(2A)-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7B-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n26A-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n7B CA_n26A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7B-n26(2A)-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n26(2A)-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n28A-n38A11 | - | n7 | 5, 10, 15, 20, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n38 | 5, 10, 15, 20, 30, 40 |  |
| CA_n7A-n28A-n78(2A) | CA_n78(2A) CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n28A-n78A | CA_n7A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n28A-n78A7 | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7A-n28A-n78C | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n7B-n28A-n78A | CA_n7A-n78A7 CA_n28A-n78A7 | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n38A-n78A10 | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n40A-n78A | CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n40A-n105A | CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5,10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n7A-n46A-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46C-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46D-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46(2A)-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46(2A)-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n46A-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n46C-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n46D-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n66A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n66A-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n77(3A) | n777,9 CA_n77(2A)7 CA_n77(2A) 7 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n66(2A)-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66A-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66(2A)-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n78A | n787,9 CA_n7A-n66A CA_n7A-n78A7 CA_n66A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n78(2A) | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66(2A)-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n67A-n78A | CA_n7A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n67A-n78(2A) | CA_n7A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n71A-n77A | n777,9 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n71A-n77(2A) | n777,9 CA_n77(2A)7 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n7A-n71A-n77(3A) | n777,9 CA_n77(2A)7 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n7A-n75A-n78A | - | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n78A-n102A | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n78A-n102B | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102B CA_n78A-n102A CA_n78A-n102B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n78A-n102C | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102C CA_n78A-n102A CA_n78A-n102C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n78A-n102D | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n78A-n102E | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n7A-n78A-n102(2A) | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n78(2A)-n102A | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n78(2A)-n102B | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102B CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n78(2A)-n102C | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102C CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n78(2A)-n102D | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n78(2A)-n102E | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n7A-n78(2A)-n102(2A) | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n78A-n105A | CA_n7A-n78A CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

[Unaffected parts omitted]

5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: Void

Table 5.5A.3.3-1a

Table 5.5A.3.3-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier 4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A-n7A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n7A CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3A-n5A-n7B | CA_n1A-n3A CA_n1A-n5A CA_n1A-n7A CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n5A-n28A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n28A CA_n3A-n5A CA_n3A-n28A CA_n5A-n28A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n5A-n78A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n78A CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n8A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n26A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7A-n26A | CA_n3B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7B-n26A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7B-n26A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n26(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n7A-n26(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n7B-n26(2A) | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n7B-n26(2A) | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
| CA_n1A-n3A-n7B-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  | CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7A-n28A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7B-n28A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3A-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3B-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3B-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A)-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A)-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n7A-n67A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3B-n7A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7B-n78A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7A-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7B-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n7B-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3A-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3A-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3B-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3B-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3B-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3B-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3(2A)-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3(2A)-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3(2A)-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3(2A)-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3A-n7A-n105A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n105A CA_n3A-n7A CA_n3A-n105A CA_n7A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n8A-n78A | CA_n1A-n3A CA_n1A-n8A CA_n1A-n78A CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
| CA_n1A-n3A-n18A-n28A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n28A CA_n3A-n18A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n1A-n3A-n18A-n41A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n41A CA_n3A-n18A CA_n3A-n41A CA_n18A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n18A-n77A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n77A CA_n3A-n18A CA_n3A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n20A-n67A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n1A-n3A-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n28A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n28A-n41A | n416 CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n77A | n775,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | n775 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n77(2A) | n775 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n28A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202,302 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202, 302 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n28A-n78A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n28A-n79A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n77A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n77A CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n105A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n105A CA_n3A-n40A CA_n3A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n41A-n77A | n415,6 n775,6 CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n41A-n77(2A) | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n41A-n79A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n67A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n67A-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n77A-n79A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n77(2A)-n79A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n78A-n105A | CA_n1A-n3A CA_n1A-n78A CA_n1A-n105A CA_n3A-n78A CA_n3A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25,30, 35 |  |
| CA_n1A-n5A-n7A-n78A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n78A CA_n5A-n7A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n7B-n78A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n28A-n78A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n78A CA_n5A-n28A CA_n5A-n78A CA_n28A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n28A-n79A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n40A-n78A | CA_n1A-n5A CA_n1A-n40A CA_n1A-n78A CA_n5A-n40A CA_n5A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n78A-n79A | CA_n1A-n5A CA_n1A-n78A CA_n1A-n79A CA_n5A-n78A CA_n5A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n8A-n40A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n40A CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n7A-n8A-n78A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26A-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n26A-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n1A-n7A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7A-n28A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n7A-n28A-n78A | CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n28A-n78A | CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n38A-n78A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n78A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n78A CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n105A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n7A-n67A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n67A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n78A-n105A | CA_n1A-n7A CA_n1A-n78A CA_n1A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n8A-n40A-n78A | CA_n1A-n8A CA_n1A-n40A CA_n1A-n78A CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n8A-n78A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n8A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A-n41A | CA_n1A-n18A CA_n1A-n28A CA_n1A-n41A CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n28A-n77A | CA_n1A-n18A CA_n1A-n28A CA_n1A-n77A CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n18A-n41A-n77A | CA_n1A-n18A CA_n1A-n41A CA_n1A-n77A CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A-n77A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40B-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n41A-n77A | n415,6 n775,6 CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n41A-n77(2A) | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n28A-n41A-n79A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n75A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n77A-n79A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n78A-n79A | CA_n1A-n28A CA_n1A-n78A CA_n1A-n79A CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n77(2A)-n79A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n40A-n78A-n105A | CA_n1A-n40A CA_n1A-n78A CA_n1A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n41A-n77A-n79A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n41A-n77(2A)-n79A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n30A-n66A | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n30A-n66A | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n30A-n66(2A) | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n5A-n30A-n77A | n775,6 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n30A-n77A | n775 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n30A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n30A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n48A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48B-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48(2A)-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48(A-B)-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48A-n77C | n775,6 CA_n77C CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n775,6 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n48B-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48(2A)-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77A | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n66A-n77A | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30,40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66(2A)-n77A | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n66(2A)-n77(2A) | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n66A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n66A-n77C | n775,6 CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n77A5 CA_n5A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n12A-n30A-n66A | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n30A-n66A | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n30A-n66(2A) | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n30A-n77A | n775,6 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n30A-n77A | n775 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n30A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n12A-n66A-n77A | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n66A-n77A | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n66(2A)-n77A | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n66A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n12A-n66(2A)-n77(2A) | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n66A | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n30A-n66A | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n14A-n30A-n66(2A) | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n77A | n775,6 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n30A-n77A | n775 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n30A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n30A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n66A-n77A | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n66A-n77A | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n66(2A)-n77A | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n66A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n66(2A)-n77(2A) | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n66A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n77A | n775,6 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n30A-n77A | n775 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n30A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n30A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n66A-n77A | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n66A-n77A | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n66(2A)-n77A | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n66A-n77(2A) | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66A-n77(2A) | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n66(2A)-n77(2A) | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A-n77A | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n30A-n66A-n77A | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n66(2A)-n77A | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n66A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66(2A)-n77(2A) | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n66A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n41A-n66A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n48A-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48B-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A-n77C | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n775,6 CA_n77C CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n66A-n71A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A-n77(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |
| CA_n2A-n66A-n71A-n78A | - | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n28A-n78A | CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n5A-n28A-n79A | CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n8A-n78A | CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n7A-n20A-n67A | CA_n3A-n7A CA_n3A-n20A CA_n7A-n20A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n78A | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n78(2A) | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7A-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n3A-n7A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7A-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7B-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n28A-n38A7 | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n7A-n28A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n28A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7B-n28A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7A-n28A-n78A | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n28A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7B-n28A-n78A | CA_n7B CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7A-n38A-n78A7 | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n40A-n105A | CA_n3A-n7A CA_n3A-n40A CA_n3A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n7A-n67A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n67A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7A-n75A-n78A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78A-n105A | CA_n3A-n7A CA_n3A-n78A CA_n3A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n18A-n28A-n41A | CA_n3A-n18A CA_n3A-n28A CA_n3A-n41A CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n28A-n77A | CA_n3A-n18A CA_n3A-n28A CA_n3A-n77A CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n18A-n41A-n77A | CA_n3A-n18A CA_n3A-n41A CA_n3A-n77A CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n67A-n78A | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n67A-n78(2A) | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n28A-n38A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n40A-n77A | CA_n3A-n28A CA_n3A-n40A CA_n3A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77A | n775,6 CA_n3A-n28A CA_n3A-n41A5 CA_n3A-n77A5 CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A5 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77(2A) | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n28A-n41A-n78A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n78(2A) | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n28A-n41A-n79A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n28A-n77A-n79A | n775,6 CA_n3A-n28A CA_n3A-n77A5 CA_n3A-n79A5 CA_n28A-n77A5 CA_n28A-n79A5 CA_n77A-n79A5 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n28A-n77(2A)-n79A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n40A-n78A-n105A | CA_n3A-n40A CA_n3A-n78A CA_n3A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25,30, 35 |  |
| CA_n3A-n41A-n77A-n79A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n41A-n77(2A)-n79A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |