3GPP TSG-RAN WG4 Meeting # 111	R4-2407211

Fukuoka, Japan, 20th - 24th May 2024

Title:	TP for TR38.899 to add new HP-NRCA 1-77-79 with 2UL

Source:	Softbank Corp., LG Electronics

Agenda item:	6.17.2

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR38.899[8] to add the following HP-NRCA combinations.

- CA_n1A-n77A-n79A (PC2 2UL)

2. Reference

[3] 3GPP TS 38.101-1 " NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone; (Release 18)", v18.5.0.

[8] 3GPP TR38.899 “High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s); (Release 18)”, v0.9.0

[9] 3GPP TR38.717-03-02 V17.0.0: “Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL (Release 17)”

Text Proposal

-- Start of TP –

-- Unaffected parts omitted –

## 6.xx	DL CA_n1-n77-n79

### 6.xx.1	Configurations

Table 6.xx.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A7 CA_n1A-n79A7 CA_n77A-n79A7 | n1 | n1 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n1A-n77(2A)-n79A4 | CA_n1A-n77A7 CA_n1A-n79A7 CA_n77A-n79A7 CA_n77A(2A)7 | n1 | n1 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A) BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.xx.2	Maximum output power

Table 6.xx.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n1A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.xx.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.xx.3.1	Power class 2 case a, b, c, d

MSD analysis of ULCA_n77(2A) can be skipped. Because the impact of it has already been investigated in PC3’s fallback combination.

Based on PC3’s co-existence studies of 2UL/2DL CA_n1-n77, CA_n1-n79, and CA_n77-n79 in TR38.717-03-02[9], own Rx impact of the 3rd band is the followings

-3rd, 4th and 5th order IMD generated by dual uplink of CA_n1-n77 may fall into part of own band n79.

=> There is no need to have MSD added because non-simultaneous Rx-Tx operation for CA_n77-n79 is assumed, therefore no MSD is needed for CA_n1-n77.

-5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n77.

=> There is no need to have MSD added because non-simultaneous Rx-Tx operation for CA_n77-n79 is assumed, therefore no MSD is needed for CA_n1-n79.

-3rd, 4th and 5th order IMD generated by dual uplink of CA_n77-n79 may fall into part of own band n1.

=> The MSD values are shown in the following table. This MSD value is the average of the analysis results of the two companies. Also, we define only the highest MSD, and omit the MSDs due to IM4 and IM5. This is described in NOTE2 and NOTE1.

Table 6.xx.3-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | 24.6 | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

### 6.xx.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

-- Unaffected parts omitted –

-- End of TP --