3GPP TSG-RAN WG4 Meeting #111	R4-2407215

Fukuoka, Japan, 20th May 2024 - 24th May 2024

Source:	AT&T

Title:	TP for TR 38.850 Addition of Single UL PC2 FDD for CA_n2-n66

Agenda item:			6.18.2

Document for:	Approval

# 1.	Introduction

A text proposal for TR 38.850 to add single UL PC2 FDD for CA_n2-n66.

# 2.	Discussion

For n2 PC2 with 20 MHz UL channel BW and n66 PC2 with 40 MHz UL channel BW, cross-band isolation MSD analyses were provided in [1] and [2] based on simulation and measurement data.

For single Tx, the following has been proposed based on averaging proposals from the companies.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.7 Skyworks 2.1 QC Avg. 1.9dB | >ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.5 Skyworks 0.8 QC Avg. 0.7dB | >ACLR2 |

For dual Tx, the following has been proposed based on averaging proposals from the companies.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 2.5 Skyworks 4.0 QC Avg. 3.3dB | >ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.7 Skyworks 1.1 QC Avg. 0.9dB | >ACLR2 |

# 3.	Text Proposal

---Start of changes---

## 5.x	CA_n2A-n66A

### 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n66A | n28 n668 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | n28 n668 CA_n2A-n66A | n2 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 		Only single uplink carriers with power class other than PC3 are listed.

### 5.x.2	Reference sensitivity requirements

For n2 PC2 with 20 MHz UL channel BW and n66 PC2 with 40 MHz UL channel BW, cross-band isolation MSD is considered based on simulation and measurement data. This section will propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n2 without TxD

For single Tx PC2 n2, the following MSD is proposed.

Table 5.x.2.1-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.7XX | >ACLR2 |
| NOTE XX: Applicable to single Tx |  |  |  |  |  |  |  |  |  |

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n2 with TxD

For dual Tx PC2 n2, the following MSD is proposed.

Table 5.x.2.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0.9 YY | >ACLR2 |
| NOTE YY: Applicable to dual Tx |  |  |  |  |  |  |  |  |  |

#### 5.x.3.1	Reference sensitivity requirements with PC2 on n66 without TxD

For single Tx PC2 n66, the following MSD is proposed.

Table 5.x.3.1-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.9 XX | >ACLR2 |
| NOTE XX: Applicable to single Tx |  |  |  |  |  |  |  |  |  |

#### 5.x.3.2	Reference sensitivity requirements with PC2 on n66 with TxD

For dual Tx PC2 n66, the following MSD is proposed.

Table 5.x.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 3.3 YY | >ACLR2 |
| NOTE YY: Applicable to dual Tx |  |  |  |  |  |  |  |  |  |

---End of changes---

# 4.	References

[1] R4-2407166: PC2 CA_n2-n66 CA_n25-n66 MSD; Skyworks Solutions Inc.

[2] R4-240xxxx: CA_n25A-n66A and CA_n2A-n66A PC2 and PC3 MSD evaluation; Qualcomm France