3GPP TSG-RAN WG4 Meeting #111	R4-2407216

Fukuoka, Japan, 20 May 2024 - 24 May 2024

Agenda Item:	4.2 BS RF requirements and BS conformance testing [WI code]

Source:	Charter, Ericsson, Nokia

Title:	(NR_newRAT-Core) Discussions on change request for Total power dynamic range (TPDR)

Document for:	Discussion

# 1	Introduction

It was noticed that OTA Total power dynamic range requirement values for some of FR2-1 and FR2-2 channel bandwidths and SCS are rounded not correctly.

As FR2-2 equipment is not ready yet for deployment, it is not too late to correct the numbers for FR2-2 OTA measurements.

Without the correct numbers, the FR2-2 operators cannot validate the Total power dynamic range of the BS. If the MAX power is too small, the BS may not reach the cell edge users when all PRB are used, and if it is too large it may cause interference to the adjacent cells around it. Similarly, the MIN power needs to cover the cell using a single PRB. Therefore, both need to meet the desired specifications.

In this proposal, we propose a corrected table for FR2-2 OTA requirements (see below), for the numbers in TS 38.104 clause 9.4.3.3.

# 2	Discussion on the purpose of this WI

## 2.1	Background

TS 38.104 in Figure 6.3.3.1-1 (see below) shows the DL Resource Blocks map. This diagram shows both the single RB (Resource Block) and the maximum number of RBs, called NRB, which TS 38.104 specifies in clause 5.3.2.

Figure 6.3.3.1-1: Min power includes 1 [RB] while Max power includes NRB [RB]

TPDR measures the BS output power in Max number of resource-blocks (RB), namely NRB, and in Min number of RBs, namely 1 RB.

Clause 9.4.3.1 defines that “The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.” It also defines that “The upper limit of the OTA total power dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.”

TBRS = Max - Min where both are measured in dBm, and TBRS in [dB]. The measurement requires that total power for Max will be at peak power level using all the NRB, while Min is measured with a single RB.

There are two important power levels in the drawing above, the Min and the Max output power levels. The ratio between these two powers should be the same as the ratio between the maximum number of RBs and the minimum number of RBs, which is:   Max [Watt] / Min [Watt] = NRB / 1.

- We propose to modify Table 9.4.3.3-2 with the correct numbers for FR2-2 OTA.

# Conclusion

In this CR proposal we have discussed and provided our view on the remaining issues related to Total power dynamic range (TPDR). Based on the discussions, we have made following proposals:

- We propose to add Table 9.4.3.3-2 for FR2-2 OTA. (See our proposal below.)

The following is a summary of our proposed CR:

--- Start of changes of section 9.4.3.3 ---

#### 9.4.3.3	Minimum requirement for BS type 2-O

OTA total power dynamic range minimum requirement for BS type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.3-1 in FR2-1 and table 9.4.3.3-2 in FR2-2.

Table 9.4.3.3-1: Minimum requirement for BS type 2-O total power dynamic range in FR2-1

| SCS | OTA total power dynamic range (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | 18.1 | 21.2 | 24.2 | N/A |
| 120 | 15.0 | 18.1 | 21.2 | 24.2 |

Table 9.4.3.3-2: Minimum requirement for BS type 2-O total power dynamic range in FR2-2

| SCS | OTA total power dynamic range (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| 120 | 18.1 | 24.2 | N/A | N/A | N/A |
| 480 | N/A | 18.1 | 20.9 | 23.9 | N/A |
| 960 | N/A | 15.1 | 17.9 | 20.9 | 21.7 |

--- End of changes in section 9.4.3.3 ---

# References

3GPP TS 38.104 V18.3.0 (2023-09) “Base Station (BS) radio transmission and reception”

3GPP TS 38.141-1 “Base Station (BS) conformance testing Part 1: Conducted.”

3GPP TS 38.141-2 “Base Station (BS) conformance testing Part 2: Radiated.”

R4-1714432 “TP to TS 38.104 - Output Power Dynamics for FR1 (conducted)”

R4-1714121 “TP to TS 38.104: OTA Output power dynamics (9.4)”