3GPP TSG-RAN WG4 Meeting#111							         			R4-2407231

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	Template for 2 band DL 1or2 band UL inter-band combination TR and TP

Source: 	Skyworks Solutions Inc., Nokia

Agenda Item:	12.3	RAN4 basket WI work plan (according to WF R4-2403721)

Document for:	Approval

# 1	Background

In RAN4#110bis, a way forward captured the result of the discussion on the band combination TR template [1]. Based on this, the contributions [2-5] were approved. These provided templates to cover the detection of all the possible types of two-band DL with up to 2 band UL and up to 3UL CCs. In this contribution, we provide the text proposal including all the templates with some additional improvements on UL configurations, structure and two-band UL with 1CC/UL band IMDs needed for the two-band DL band combination TPs. We then provide an example on the use of the template.

# Discussion

## 2.1 Way forward input

RAN4 has agreed one template [5]. Additional templates have been provided in [6, 7, 8] and should accordingly also be approved.

- R4-2405935 “Improved harmonic related MSDs template”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia.

- R4-2404248 “On simplifying analysis for triple beat products”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia, Murata.

- R4-2404249 “On simplifying analysis for 2DL-1 band intra-band ULCA IMD products”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia, Murata.

- R4-2404250 “On cross-band isolation MSD analysis”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia, Murata.

<Way forward>:

- RAN4 will continue to discuss how to improve the TR templates for R19 basket WI, and try to approve any additional TR templates in May meeting, i.e. RAN4 #111 meeting.

  - To collect the extra info need to be included in the TR templates, such as

      - The detail analysis for the calculation of MSD values.

      - Other aspects to be included.

  - To decide if some info could be removed from the current template.

      - If a simplified rule-based delta T/R could be applied?

      - Other aspects considered to be removed.

  - To analyse if a common TR template is needed for all basket WI.

  - To analyse if some contents in the current TR template could be moved to a common clause.

  - Other aspects are not precluded.

In this contribution we provide a TP for the two band DL template based on:

- Approved templates per UL types in [2-5]

- Addition of a UL configuration table that triggers the related analysis.

- Incremental improvement on the two band UL with one CC per band IMD table

- An improved structure based on UL configurations.

At the conclusion, an example of how to use the tables is provided for CA_n1-n102

This contribution does not address the guidelines and templates for the MSD test point design as this will be covered in a different contribution. Additionally, the Delta T/R aspects should be addressed.

## 2.2 Proposed improvements to the 2DL/1or2UL coexistence analysis

Based on the above description and the per MSD types contributions approved in [2-5] in RAN4#110bis, we make the following proposal for the overall two-band DL TP template. This is similar to what was already discussed in [6].

Proposed enhancements for 2DL/1or2UL bands block approval TP template for Release 19:

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A question related to the support of SimRx/Tx, or otherwise for TDD/TDD cases.

  - A table that sorts the applicable UL configuration and their related MSD studies

- For the 2DL/1UL section:

  - The addition of a specific section for “Co-existence studies for 1UL band with 1CC”

    - UL harmonic and harmonic mixing tables are updated in a matrix form with additional guidelines as approved in [2]

    - A new calculation table for cross-band isolation MSD is added, as approved in [3]

  - The addition of a specific section for “Co-existence studies for 1UL band with 2CC intra-band”

    - The IMD range table is updated and simplified as discussed in [4]

- For this meeting, the delta T/R, REFSENS and OOB exception sections are not covered. However, these may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/2UL section:

  - Slightly updated 2DL 2UL with 1CC/band IMD table, with an analysis and note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and note section, as discussed in [5]

- The band group table used in [3] and [5], is added in Annex A (note that the last band group had an error as the starting frequency is 5150MHz and not 5250MHz).

- The valid UL configurations up to Release 18 are listed in Annex B.

## 2.3 Proposed template for 2DL/1or2UL block approval TPs

In the proposed template the fields to be completed by the proponent in are highlighted in grey. Added parts are designated by blue text for the part proposed in this meeting and in purple text for the part that was approved in RAN4#110bis.

The note part of the table may become redundant for the different TPs captured in the TR. The rapporteur updating the TR may decide to omit the note section of the tables when copying the TPs in the related TR.

************************* Start of template **********************************

## 5.X		CA_nX-nY

### 5.X.1	Common for 1 band UL and 2 bands UL CA

#### 5.X.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.X.1.1-1 below.

Table 5.X.1.1-1: CA band combination constituent bands definition.

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |

#### 5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination.

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_nXA-nYA | CA_nXA-nYA | nX | Channel BWs | X |
|  |  | nY | Channel BWs |  |
| CA_nX(A/B/C(2A)-nY(A/B/C(2A) | CA_nX(A/B/C(2A)-nY(A/B/C(2A) | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |

To determine the coexistence study cases to be analyzed, the following question and UL configuration types table should be completed. The allowable UL configurations are listed in Annex B.

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? XXX

Note: for TDD/TDD band combination supporting SimRx/Tx, MSD analysis is required for one UL band cases but not for two UL band cases.

Table 5.X.1.2-2: Supported UL configurations and required coexistence studies

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 1UL band with 1CC | nX, nY | 3 | One band is FDD or SimRx/Tx TDD/TDD | UL harmonic, harmonic mixing and cross-band MSD should be studied | 5.X.1.3.1-1 5.X.1.3.1-2 |
| 1UL band with 2CC | CA_nXB/C1/(2A) and/or CA_nYB/C1/(2A) | 3 |  | IMDs of the two intra-band UL CCs | 5.X.1.3.2-1 |
| 2UL bands 1CC per band | CA_nXA-nYA | 3 | FDD/FDD Or FDD/TDD | IMDs of the two UL bands | 5.X.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_nXA/B/C-nYA/B/C | 3 |  | Triple beat of the three UL CCs if the two bands are in the same or adjacent band groups | 5.X.2.2.2-1 |
| Note 1: Should be considered as a fallback of CA_nXA/B/C-nYA/B/C |  |  |  |  |  |

#### 5.X.1.3	Co-existence studies

#### 5.X.1.3.1	Co-existence studies for 1UL band with 1CC

Table 5.X.1.3.1-1 summarizes frequency ranges of UL harmonics and/or harmonic mixing collisions that may occur for both UL bands into the other band DL.

Table 5.X.1.3.1-1: UL harmonics and harmonic mixing analysis.

| UL/DL harmonics |  | nX | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nY | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A | X1 | X1 | X1 | X1 | UL Harmonic |
| DL2 | 2*fDLlow | 2*fDLhigh | X1 | N/A | X1,2 | N/A | N/A | Harmonic Mixing |
| DL3 | 3*fDLlow | 3*fDLhigh | X1,3 | X1 | N/A | X1 | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh | X1 | N/A | N/A | N/A | N/A |  |
| DL5 | 5*fDLlow | 5*fDLhigh | X1,3 | X1 | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nX | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A | X1 | X1 | X1 | X1 | UL Harmonic |
| DL2 | 2*fDLlow | 2*fDLhigh | X1 | N/A | X1 | N/A | N/A | Harmonic Mixing |
| DL3 | 3*fDLlow | 3*fDLhigh | X1,3 | X1 | N/A | X1 | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh | X1 | N/A | N/A | N/A | N/A |  |
| DL5 | 5*fDLlow | 5*fDLhigh | X1,3 | X1 | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: When a collision is detected with an overlap >0Hz between the UL(X) with DL(Y) frequency ranges, the UL(X)/DL(Y) cell is marked “D” for direct hit. When the gap between UL(X) and DL(Y) frequency range is from 0Hz to X*MinULCBW, the UL(X)/DL(Y) cell is marked “N” for Near miss. Note 2: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 3: For harmonic mixing, near-miss cases only apply for UL1 and odd DL orders. |  |  |  |  |  |  |  |  |

Table 5.X.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.X.1.3.1-2: Cross-band isolation analysis

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | X | X | X | X |
| fDL (MHz) | X | X | X | X |
| CBW (MHz)2 | X | X | X | X |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | X | X | X | X |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | X | X | X | X |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | X | X | X | X |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | X | X | X | X |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | X | X | X | X |
| Analysis | text |  | text |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in Annex A -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest harmonic 2 frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.X.1.3.2	Co-existence studies for 1UL band with 2CC intra-band

Table 5.X.1.3.2-1 provides the one UL band with intra-band ULCA IMD frequency ranges for the CA_nX/YB/C/(2A) UL configuration for DL CA_nX-nY.

Table 5. X.1.3.2-1: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | fLow/Min | fHigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL | X | X | Order | fLow | fHigh |
| 2CCBW1 | X | X | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | X | X |  | X | X |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | fLow | fHigh |  | X | X |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | X | X |  | X | X |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | X | X | Order | fLow | fHigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | X | X |  | X | X |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | X | X |  | X | X |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | X | X | Order | fLow | fHigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | X | X |  | X | X |
| Analysis | text |  |  |  |  |
| Note 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) Note 2: The close to UL IMD range is the most critical when the victim DL band is in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. Note 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) -IMD2 is not considered assuming CA with 450MHz bands is not considered -IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz -IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz Note 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

*************** Delta T/R and 2DL/1UL REFSENS sections omitted in this version of the proposal ****************

### 5.X.2		Specific for 2 bands UL CA

#### 5.X.2.1	Maximum output power for inter-band CA

Table 5.X.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +2/-3 |
| CA_nX/YA/B/C/(2A) | 23 | +2/-3 |
| CA_nXA/B/C-nXA/B/C | 23 | +2/-3 |

#### 5.X.2.2	UE co-existence

#### 5.X.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.X.2.2.1-1 provides the two UL bands with one CC per band IMD interference analysis.

Table 5.X.2.2-1: Two UL band IMD analysis.

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | X | X | X | X |
| fDL (MHz) | X | X | X | X |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| Analysis | text |  | text |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

#### 5.X.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.X.2.2.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products interference analysis for CA_nXA/B/C-nYA/B/C, with nX/YB/C transmitting with a XXXMHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-1: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | X | X | X | X |
| fDL (MHz) | X | X | X | X |
| 2CCBW (MHz)2 | N/A | N/A | X | X |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in Annex A, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW), respectively. |  |  |  |  |

*********************REFSENS section omitted in this version of the template – end of template*********

## 2.4 proposed annexes for TR and/or TPs

Since the band group definition is used as a criteria for cross-band isolation MSD and Triple beat MSD, it is proposed to have its definition in Annex A, while the valid UL configurations for two-band DL are provided in Annex B.

********************* Start of annex *********

Annex A

For the following MSD issues detection, a criterion applies in terms of band group as defined in Table A:

- For 2DL 1UL/1CC cross-band isolation, and for the Tx noise floor MSD to be considered, the two bands should be part of the same or adjacent band group.

- For 2DL 2UL/3CC triple beat MSD to be considered, the two bands should be part of the same or adjacent band group.

Table A: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex B

For CA_nX-nY two band DL inter band combinations, the following UL configurations are applicable:

- One band UL with one CC: nXA, nYA

- One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A)

- two band UL with one CC per band: CA_nXA-nYA

- two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

********************* End of annex *********

## 2.5 Example of a resulting TP for CA_n1-n102

The selected example is based on TP [7] on CA_n1-n102 and DC_n1-n102. Thisexample contains all of the different types of UL configurations and can be considered to be fairly complete in terms of MSD studies. Although it does not result in an actual MSD specified for all the MSD types, it provides a good example of how the proposed template could function for a concrete case.

Text is color coded with highlights to provide indication on to derive the information to be filled in:

- Yellow highlights correspond to how data related to the bands definitions

- Cyan highlights correspond to data related to the BCS DL, UL, channel bandwidths, and intra-band CA BW from the BCS definitions

- Green highlights correspond to data that can be calculated with the equations provided

- Pink highlights correspond to text that is provided in support of the analysis of the table results.

- When a part of the table is not relevant for a given case, the cell is with a grey background.

Commentary text can be added below each table. In this example, we did not retain the note section of the tables as this will not need duplication in the TR. However, in TPs this information may be a useful reference for the TP reviewers during the flagging process.

************************* Start of example TP **********************************

## 5.XX		CA_n1-n102

### 5.XX.1	Common for 1 band UL and 2 bands UL CA

#### 5.XX.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.XX.1.1-1 below.

Table 5.XX.1.1-1: CA band combination constituent bands definition.

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n102 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD |

#### 5.XX.1.2	Channel bandwidths per operating band for CA

Table 5.XX.1.2-1: Supported bandwidths per CA band combination.

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A CA_n1A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A CA_n1A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

To determine the coexistence study cases to be analyzed, the following question and UL configuration types table should be completed. The allowable UL configurations are listed in Annex B.

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? N-A

Table 5.XX.1.2-2: Supported UL configurations and required coexistence studies.

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 1UL band with 1CC | n1, n102 | 3 3 | One band is FDD Or SimRx/Tx TDD/TDD | UL harmonic, harmonic mixing and cross-band MSD should be studied | 5.XX.1.3.1-1 5.XX.1.3.1-2 |
| 1UL band with 2CC | CA_n102B, CA_n102C in CA_n1A-n102B, CA_n1A-n102C | 3 |  | IMDs of the two intra-band UL CCs | 5.XX.1.3.2-1 |
| 2UL bands 1CC per band | CA_n1A-n102A | 3 | FDD/FDD Or FDD/TDD | IMDs of the two UL bands | 5.XX.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_n1A-n102B CA_n1A-n102C | 3 3 |  | Triple beat of the three UL CCs, if the two bands are in adjacent band groups | 5.XX.2.2.2-1 |

#### 5.XX.1.3	Co-existence studies

#### 5.XX.1.3.1	Co-existence studies for 1UL band with 1CC

Table 5.XX.1.3.1-1 summarizes frequency ranges of UL harmonics and/or harmonic mixing collisions that may occur for CA_n1-n102.

Table 5.XX.1.3.1-1: UL harmonics and harmonic mixing analysis.

| UL/DL harmonics |  | n102 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 5925 | 11850 | 17775 | 23700 | 29625 |  |
| n1 | fLow | fHigh | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| DL1 | 2110 | 2170 | N/A | - | - | - | - | UL Harmonic |
| DL2 | 4220 | 4340 | - | N/A | - | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6510 | D | - | N/A | - | N/A |  |
| DL4 | 8440 | 8680 | - | N/A | N/A | N/A | N/A |  |
| DL5 | 10550 | 10850 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n102 UL1 / n1 DL3 |  |  |  |  |  |
| UL/DL harmonics |  | n1 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n102 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 5925 | 6425 | N/A | - | N | - | - | UL Harmonic |
| DL2 | 11850 | 12850 | - | N/A | - | N/A | N/A | Harmonic Mixing |
| DL3 | 17775 | 19275 | - | - | N/A | - | N/A |  |
| DL4 | 23700 | 25700 | - | N/A | N/A | N/A | N/A |  |
| DL5 | 29625 | 32125 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Near miss n1 UL3 / n102 DL1 (5MHz) |  |  |  |  |  |

Based on the above table:

- n1 3rd UL harmonic falls 5MHz short of the band n102 DL, and thus may be the victim of the ACLR1 range of the third harmonic. Given that n102 is a shared spectrum access band, a REFSENS exclusion range can be defined.

- n1 3rd DL harmonic mixing falls on the band n102 UL, thus harmonic mixing MSD should be considered.

Table 5.XX.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for CA_n1-n102.

Table 5.XX.1.3.1-2: Cross-band isolation analysis.

| Bands | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1920 | 1980 | 5925 | 6425 |
| fDL (MHz) | 2110 | 2170 | 5925 | 6425 |
| CBW (MHz) | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2020 | 5825 | 6525 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2070 | 5725 | 6625 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2120 | 5625 | 6725 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2170 | 5525 | 6825 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2220 | 5425 | 6925 |
| Analysis | There is no overlap of the band n1 ACLR range with the band n102 DL up to order 5 and there is sufficient rejection of the transmitter noise floor at band n102 so no cross-band MSD is needed. |  | There is no overlap of the band n102 ACLR range with the band n1 DL up to order 5 and there is sufficient rejection of the transmitter noise floor at band n1 so no cross-band MSD is needed. |  |

Based on the above table: There is no case with the UL band ACLR range overlapping with the DL range and the band 1 and band n102 UL filters. This, together with the diplexer isolation, provides sufficient rejection of the other band DL frequency range, and justifies the conclusion to ignore the UL noise floor impact.

5.XX.1.3.2	Co-existence studies for 1UL band with 2CC intra-band

Table 5.XX.1.3.2-1 provides the one UL band with intra-band ULCA IMD frequency ranges for the CA_n102B and CA_n102C UL configurations, which are part of CA_n1-n102B and CA_n1-n102C requested cases.

Table 5.XX.1.3.2-1: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | fLow/Min | fHigh/Max | BB IMD range |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL | 5925 | 6425 | Order | fLow | fHigh |
| 2CCBW1 | 0 | 160 | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | 2110 | 2170 |  | 0 | 160 |
| Close to UL IMD range |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | fLow | fHigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 5765 | 6585 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range |  |  |
|  | 5605 | 6745 | Order | fLow | fHigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 5445 | 6905 |  | 11690 | 13010 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 5285 | 7065 |  | 11530 | 13170 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range |  |  |
|  | 5125 | 7225 | Order | fLow | fHigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 4965 | 7385 |  | 17615 | 19435 |
| Analysis | The DL victim band is lower than the TDD intra-band ULCA band, which is contiguous ULCA. Thus, only the close to CA_n102C UL IMD3 to 9 should be considered, and there is no overlap with the DL band n1. |  |  |  |  |

Since this is a contiguous ULCA TDD case, and the UL band is above the DL band, only up to IMD9 is considered for close to UL products, while BB and close to harmonic products can be ignored. Based on the above table: There is no case with the 2CC intra-band IMD range overlapping with the DL range of band 1.

********* Delta T/R, 2DL/1UL REFSENS and OOB exception sections omitted in this version of the example *********

### 5.XX.2		Specific for 2 bands UL CA

#### 5.XX.2.1	Maximum output power for inter-band CA

Table 5.XX.2.1-1: UE Power Class for uplink inter-band CA.

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n102A | 23 | +2/-3 |
| CA_n1A-n102B | 23 | +2/-3 |
| CA_n1A-n102C | 23 | +2/-3 |

#### 5.XX.2.2		UE co-existence

#### 5.XX.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.XX.2.2.1-1 provides the two UL bands, with one CC per band IMD interference analysis for CA_ n1A-n102A.

Table 5.XX.2.2-1: Two UL band IMD analysis.

| Bands | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | 1920 | 1980 | 5925 | 6425 |
| fDL (MHz) | 2110 | 2170 | 5925 | 6425 |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | 3945 | 4505 | 7845 | 8405 |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | 2585 | 1965 | 9870 | 10930 |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | 9765 | 10385 | 13770 | 14830 |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | 665 | 15 | 15795 | 17355 |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | 9010 | 7890 | 15690 | 16810 |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | 11685 | 12365 | 19695 | 21255 |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | 23780 | 21720 | 1995 | 1255 |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | 15435 | 13815 | 5910 | 7090 |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | 25620 | 27680 | 13605 | 14345 |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | 21615 | 23235 | 17610 | 18790 |
| Analysis | IMD3 fall in FDD band n1 DL: MSD may occur |  | IMD5 fall in TDD band n102 DL: no MSD possible |  |

Based on the table 5.XX.2.2.1-1, one 3rd order intermodulation product fall inside the DL band of n1.

#### 5.XX.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.XX.2.2.2-1 provides the two UL bands with one band, with a 2CC intra-band uplink CA triple beat products interference analysis is provided for CA_n1A-n102C including n102C transmitting with a 160 MHz maximum instantaneous bandwidth.

Table 5.XX.2.2.2-1: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | n1 |  | CA_n102C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1920 | 1980 | 5925 | 6425 |
| fDL (MHz) | 2110 | 2170 | 5925 | 6425 |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 160 |
| IMD3 products | fx_low-max2CCBW | fx_low | fx_high | fx_high+max2CCBW |
| IMD3 (MHz) | 1760 | 1920 | 1980 | 2140 |
| Analysis | There is a triple beat product of n1 UL with n102C 2CC UL falling in n1 DL. However, n1 and n102 are not part of adjacent band groups and thus MSD is not required. |  |  |  |

The uplinks of n1A and n102C have a 1st order triple beat falling inside n1 DL. As stated in Annex A, the triple beat case is not eligible for MSD if the two UL bands aren’t part of the same or adjacent band group.

*********************REFSENS section omitted in this version of the example *********

************************* End of example TP **********************************

# Conclusion

In this contribution, we presented an example of improved template for the block approval TPs for two band DL cases and have the following proposals.

Proposed enhancements for 2DL/1or2UL bands block approval TP template for Release 19:

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A question related to the support of SimRx/Tx, or otherwise for TDD/TDD cases.

  - A table that sorts the applicable UL configuration and their related MSD studies

- For the 2DL/1UL section:

  - The addition of a specific section for “Co-existence studies for 1UL band with 1CC”

    - UL harmonic and harmonic mixing tables are updated in a matrix form with additional guidelines as approved in [2]

    - A new calculation table for cross-band isolation MSD is added, as approved in [3]

  - The addition of a specific section for “Co-existence studies for 1UL band with 2CC intra-band”

    - The IMD range table is updated and simplified as discussed in [4]

- For this meeting, the delta T/R, REFSENS and OOB exception sections are not covered. However, these may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/2UL section:

  - Slightly updated 2DL 2UL with 1CC/band IMD table, with an analysis and note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and note section, as discussed in [5]

- The band group table used in [3] and [5], is added in Annex A (note that the last band group had an error as the starting frequency is 5150MHz and not 5250MHz).

- The valid UL configurations up to Release 18 are listed in Annex B.

# References

[1] R4-2406685 WF on band combination TR template, ZTE, Nokia, Skyworks, Ericsson, RAN4#110bis

[2] R4-2404243 Improved harmonic related MSDs template, Skyworks Solutions Inc., Nokia, RAN4#110bis

[3] R4-2404250 On cross band isolation MSD analysis, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

[4] R4-2404249 On simplifying analysis for 2DL-1 band intra-band ULCA IMD products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

[5] R4-2404248 On simplifying analysis for triple beat products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

[6] R4-2405554 Template example for 2 band DL with 1 or 2 band UL up to 3 UL CCs, Skyworks Solutions Inc., RAN4#110bis