3GPP TSG-RAN WG4 Meeting#111							        	 		R4-2407232

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	Template for 3 band DL 2 band UL inter-band combination TR and TP

Source: 	Skyworks Solutions Inc., Nokia

Agenda Item:	12.3	RAN4 basket WI work plan (according to WF R4-2403721)

Document for:	Approval

# 1	Background

In RAN4#110bis, a way forward captured the result of the discussion on band combination TR template [1]. As a result, the contribution [2] on the triple beat template was approved. In this contribution, we provide the text proposal including all the templates with some additional improvements on UL configurations, structure needed for the three band DL with 2 band UL band combination TPs which can be derived from the 2band DL, 2 band UL cases in [3] and is formalized in [4] for this meeting. We then provide an example of the use of the template.

# Discussion

## 2.1 Way forward input

RAN4 has agreed one template [5]. Additional templates have been provided in [6, 7, 8] and should accordingly also be approved.

- R4-2405935 “Improved harmonic related MSDs template”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia.

- R4-2404248 “On simplifying analysis for triple beat products”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia, Murata.

- R4-2404249 “On simplifying analysis for 2DL-1 band intra-band ULCA IMD products”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia, Murata.

- R4-2404250 “On cross-band isolation MSD analysis”, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th - 19th 2024, Skyworks Solutions Inc., Nokia, Murata.

<Way forward>:

- RAN4 will continue to discuss how to improve the TR templates for R19 basket WI, and try to approve any additional TR templates in May meeting, i.e. RAN4 #111 meeting.

  - To collect the extra info need to be included in the TR templates, such as

      - The detail analysis for the calculation of MSD values.

      - Other aspects to be included.

  - To decide if some info could be removed from the current template.

      - If a simplified rule-based delta T/R could be applied?

      - Other aspects considered to be removed.

  - To analyse if a common TR template is needed for all basket WI.

  - To analyse if some contents in the current TR template could be moved to a common clause.

  - Other aspects are not precluded.

Since the two-band UL cases are also valid for issues in the third band of a three-band DL case, in this contribution we provide a TP for the three band DL template based on:

- Approved templates for triple beat in [2]

- Addition of a UL configuration table that triggers the related analysis.

- Incremental improvement on the IMD table for 2 band UL with 1CC per band.

- An improved structure based on UL configurations.

At the conclusion, an example of how to use the tables is provided for CA_n1-n77-n102

This contribution does not address the guidelines and templates for the MSD test point design as this will be covered in a different contribution. Additionally, the Delta T/R aspects should be addressed.

## 2.2 Proposed improvements to the 3DL/2UL coexistence analysis

Based on the above description and the triple beat MSD types contributions approved in [2] in RAN4#110bis, along with this meeting contribution on the 2 band DL template [4], we make the following proposal for the overall three band DL TP template.

Proposed enhancements for 3DL/2UL bands block approval TP template for Release 19:

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A table that sorts the applicable UL configuration and their related MSD studies

- For this meeting, the delta T/R and REFSENS sections are not covered. However, these sections may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/2UL section:

  - Slightly updated 3DL 2UL with 1CC/band IMD table, including an analysis and note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and note section, as discussed in [2]

- The band group table used in [2], is added in Annex A (note that the highest band group had an error, as the starting frequency is 5150MHz and not 5250MHz).

- The valid UL configurations up to Release 18 are listed in Annex B.

## 2.3 Proposed template for 2DL/1or2UL block approval TPs

In the proposed template the fields to be completed by the proponent in are highlighted in grey, added parts are designated by blue text for the part proposed in this meeting and in purple text for the part that was approved in RAN4#110bis.

The note part of the table may become redundant for the different TPs captured in the TR. The rapporteur updating the TR may decide to omit the note section of the tables when copying the TPs in the related TR.

************************* Start of template **********************************

## 5.X		CA_nX-nY-nZ

### 5.X.1	Common for 1 band UL and 2 bands UL CA

#### 5.X.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.XX.1.1-1 below.

Table 5.X.1.1-1: CA band combination constituent bands definition.

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nZ | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |

#### 5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination.

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_nXA-nYA-nZA | CA_nXA-nYA | nX | Channel BWs | X |
|  | CA_nXA-nZA | nY | Channel BWs |  |
|  | CA_nYA-nZA | nZ | Channel BWs |  |
| CA_nXA/B/C(2A)-nYA/B/C/(2A)- nZA/B/C/(2A) | CA_nXA/B/C-nYA/B/C CA_nXA/B/C-nZA/B/C CA_nYA/B/C-nZA/B/C | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |
|  |  | nZ | Channel BW or CA BCS |  |

To determine the coexistence study cases to be analyzed, the UL configuration types table should be completed. The allowable UL configurations are listed in Annex B.

Table 5.X.1.2-2: Supported UL configurations and required coexistence studies.

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 2UL bands 1CC per band | CA_nXA-nYA CA_nXA-nZA CA_nYA-nZA | 3 | Third band is simultaneous Rx with the other two bands Tx | IMDs of the two UL bands falling into third band DL | 5.X.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_nXA/B/C-nYA/B/C CA_nXA/B/C-nZA/B/C CA_nYA/B/C-nZA/B/C | 3 |  | Triple beat into the third band DL of the three UL CCs if the third bands is in the same or adjacent band group than one of the UL | 5.X.2.2.2-1 |

#### 5.X.1.3	∆TIB,c and ∆RIB,c values

*************** Delta T/R section is omitted in this version of the proposal ****************

### 5.X.2		Specific for 2 bands UL CA

#### 5.X.2.1	UE co-existence

#### 5.X.2.2.1	Co-existence studies for 2UL band with 1CC per band

Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced.

Table 5.X.2.2.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_nXA-nYA-nZA with UL CA_nXA-nYA.

Table 5.X.2.2.1-1: Two UL bands IMD analysis.

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | X | X | X | X |
| nZ fDL (MHz) | X | X | N/A | N/A |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.X.2.2.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_nXA-nYA-nZA with UL CA_nXA-nZA.

Table 5.X.2.2.1-2: Two UL bands IMD analysis.

| Bands | nX |  | nZ |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | X | X | X | X |
| nY fDL (MHz) | X | X | N/A | N/A |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.X.2.2.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_nXA-nYA-nZA with UL CA_nYA-nZA.

Table 5.X.2.2.1-3: Two UL bands IMD analysis.

| Bands | nY |  | nZ |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | X | X | X | X |
| nX fDL (MHz) | X | X | N/A | N/A |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

#### 5.X.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Note: Since the triple beat tables have already been calculated for the different two-band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two-band TP tables. Nonetheless, the triple beat issues should be stated, and the related TPs referenced.

Table 5.X.2.2.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band nZ interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXXMHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-1: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | X | X | X | X |
| nZ fDL (MHz) | X | X | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | X | X |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note 1: If the third band is not part of the same or adjacent band groups of one of the UL bands as defined in Annex A, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.X.2.2.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band nY interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXXMHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-2: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | nX |  | CA_nZB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | X | X | X | X |
| nY fDL (MHz) | X | X | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | X | X |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note 1: If the third band is not part of the same or adjacent band groups of one of the UL bands as defined in Annex A, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.X.2.2.2-3 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band nX interference analysis for CA_nYA/B/C-nYA/B/C with nX/YB/C transmitting with a XXXMHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-3: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | nY |  | CA_nZB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | X | X | X | X |
| nX fDL (MHz) | X | X | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | X | X |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | X | X | X | X |
| Analysis | text |  |  |  |
| Note 1: If the third band is not part of the same or adjacent band groups of one of the UL bands as defined in Annex A, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

*********************REFSENS section omitted in this version of the template – end of template*********

## 2.4 proposed annexes for TR and/or TPs

Since the band group definition is used as the criteria for Triple beat MSD, it is proposed to have its definition in Annex A, while the valid two UL bands configurations for three-band DL are provided in Annex B.

********************* Start of annex *********

Annex A

For three DL and 2UL/3CC triple beat MSD to be considered, the two bands should be part of the same or adjacent band group as defined in Table A.

Table A: Band group definition for same or adjacent band-group criterion.

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex B

For CA_nX-nY-nZ three band DL inter band combinations with two UL bands, the following UL configurations are applicable:

- two band UL with one CC per band: CA_nXA-nYA

- two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

********************* End of annex *********

## 2.5 Example of a resulting TP for CA_n1-n77-n104

The selected example is based on CA_n1-n77-n102. This combination is not requested or specified, but instead is selected as it contains several of the different types of UL configurations, and can be considered to be fairly complete in terms of MSD studies. Although it does not result in an actual MSD specified for all the MSD types, it provides a good example of how the proposed template could function for a concrete case.

Text is color coded with highlights, this provides indication on how to derive the information to be filled in:

- Yellow highlights correspond to data related to the bands definitions

- Cyan highlights correspond to data related to the BCS DL, UL, channel bandwidths, and intra-band CA BW from the BCS definitions

- Green highlights correspond to data that can be calculated with the equations provided

- Pink highlights correspond to text that is provided in support of the analysis of the table results.

- When a part of the table is not relevant for a given case, the cell is with a grey background.

Commentary text can be added below each table. In this example, we did not retain the note section of the tables as this will not need duplication in the TR. However, in TPs, this information may be a useful reference for TP reviewers during the flagging process.

************************* Start of example TP **********************************

## 5.XX		CA_n1-n77-n102

### 5.X.1	Common for 1 band UL and 2 bands UL CA

#### 5.X.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination constituent bands definition.

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n102 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD |

#### 5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination.

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n77A-n102A | CA_n1A-n102A CA_n1A-n77A CA_n77A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n77C-n102A | CA_n1A-n77C CA_n77C-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n77A-n102C | CA_n1A-n102C CA_n77A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |

To determine the coexistence study cases to be analyzed, the UL configuration types table should be completed. The allowable UL configurations are listed in Annex B.

Table 5.X.1.2-2: Supported UL configurations and required coexistence studies.

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 2UL bands 1CC per band | CA_n1A-n102A CA_n1A-n77A CA_n77A-n102A | 3 | Third band is simultaneous RX with the other two bands Tx | IMDs of the two UL bands | 5.X.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_n1A-n77C CA_n77C-n102A CA_n1A-n102C CA_n77A-n102C | 3 3 |  | Triple beat of the three UL CCs, if the two bands are in adjacent band groups | 5.X.2.2.2-1 |

********* Delta T/R section is omitted in this version of the example *********

### 5.X.2		Specific for 2 bands UL CA

#### 5.X.2.1	UE co-existence

#### 5.X.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.X.2.2.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n77A-n102A with UL CA_n1A-n77A.

Table 5.X.2.2.1-1: Two UL bands IMD analysis.

| Bands | n1 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | 1920 | 1980 | 3300 | 4200 |
| n102 fDL (MHz) | 5925 | 6425 | N/A | N/A |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | 1320 | 2280 | 5220 | 6180 |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | -360 | 660 | 4620 | 6480 |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | 7140 | 8160 | 8520 | 10380 |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | 1560 | 2640 | 7920 | 10680 |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | 4560 | 2640 | 10440 | 12360 |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | 9060 | 10140 | 11820 | 14580 |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | 14880 | 11220 | 4620 | 3480 |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | 8760 | 5940 | 660 | 2640 |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | 15120 | 18780 | 10980 | 12120 |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | 13740 | 16560 | 12360 | 14340 |
| Analysis | IMD2, IMD3 and IMD5 fall in band n102 DL: MSD may occur and should be studied for IMD2 and IMD3. |  |  |  |

Table 5.X.2.2.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n77A-n102A with UL CA_n1A-n102A.

Table 5.X.2.2.1-2: Two UL bands IMD analysis.

| Bands | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | 1920 | 1980 | 5925 | 6425 |
| n77 fDL (MHz) | 3300 | 4200 | N/A | N/A |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | 3945 | 4505 | 7845 | 8405 |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | 2585 | 1965 | 9870 | 10930 |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | 9765 | 10385 | 13770 | 14830 |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | 665 | 15 | 15795 | 17355 |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | 9010 | 7890 | 15690 | 16810 |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | 11685 | 12365 | 19695 | 21255 |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | 23780 | 21720 | 1995 | 1255 |
| IMD5 products | ¦2*fxULlow – 3*fyULhigh¦ | ¦2*fxULhigh – 3*fyULlow¦ | ¦2*fyULlow – 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | 15435 | 13815 | 5910 | 7090 |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | 25620 | 27680 | 13605 | 14345 |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | 21615 | 23235 | 17610 | 18790 |
| Analysis | IMD3 fall in band n77 DL: MSD may occur. |  |  |  |

Table 5.X.2.2.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n77A-n102A with UL CA_n77A-n102A.

Table 5.X.2.2.1-3: Two UL bands IMD analysis.

| Bands | n77 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | 3300 | 4200 | 5925 | 6425 |
| n1 fDL (MHz) | 2110 | 2170 | N/A | N/A |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | 1725 | 3125 | 9225 | 10625 |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | 175 | 2475 | 7650 | 9550 |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | 12525 | 14825 | 15150 | 17050 |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | 3475 | 6675 | 13575 | 15975 |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | 6250 | 3450 | 18450 | 21250 |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | 15825 | 19025 | 21075 | 23475 |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | 22400 | 19500 | 10875 | 6775 |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | 12675 | 9375 | -750 | 2950 |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | 27000 | 29900 | 19125 | 23225 |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | 24375 | 27675 | 21750 | 25450 |
| Analysis | IMD2, IMD3 and IMD5 fall in band n1 DL: MSD may occur and should be studied for IMD2 and IMD3. |  |  |  |

#### 5.X.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.X.2.2.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n102 interference analysis for CA_n1A-n77C, with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-1: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | n1 |  | CA_n77C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1920 | 1980 | 3300 | 4200 |
| n102 fDL (MHz) | 5925 | 6425 | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 200 |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | 1720 | 1920 | 1980 | 2180 |
| Analysis | There is no triple beat product overlapping with n102. |  |  |  |

Table 5.X.2.2.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n77 interference analysis for CA_n1A-n102C, with n102C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-2: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | n1 |  | CA_n102C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1920 | 1980 | 5925 | 6425 |
| n77 fDL (MHz) | 3300 | 4200 | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 200 |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | 1720 | 1920 | 1980 | 2180 |
| Analysis | There is no triple beat product overlapping with n77. |  |  |  |

Table 5.X.2.2.2-3 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n77A-n102C with n102C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-3: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | n77 |  | CA_n102C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 3300 | 4200 | 5925 | 6425 |
| n1 fDL (MHz) | 2110 | 2170 | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 200 |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | 3100 | 3300 | 4200 | 4400 |
| Analysis | Since band n1 is no part of the same or adjacent band group to n77 and n102, the analysis can be ignored. However, there is no triple beat product overlapping with n1. |  |  |  |

Table 5.X.2.2.2-4 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n77C-n102A with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.X.2.2.2-4: Two UL band with intra-band ULCA Triple beat IMD analysis.

| Band / CA1 | n102 |  | CA_n77C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 5925 | 6425 | 3300 | 4200 |
| n1 fDL (MHz) | 2110 | 2170 | N/A | N/A |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 200 |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) | 5725 | 5925 | 6425 | 6625 |
| Analysis | Since band n1 is no part of the same or adjacent band group to n77 and n102, the analysis can be ignored. However, there is no triple beat product overlapping with n1. |  |  |  |

*********************REFSENS section omitted in this version of the example *********

************************* End of example TP **********************************

# Conclusion

In this contribution, we presented an example of improved template for the block approval TPs for three band DL cases and have the following proposals.

Proposed enhancements for 3DL/2UL bands block approval TP template for Release 19:

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A table that sorts the applicable UL configuration and their related MSD studies

- For this meeting, the delta T/R and REFSENS sections are not covered. However, these sections may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/2UL section:

  - Slightly updated 3DL 2UL with 1CC/band IMD table, including an analysis and note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and note section, as discussed in [2]

- The band group table used in [2], is added in Annex A (note that the highest band group had an error, as the starting frequency is 5150MHz and not 5250MHz).

- The valid UL configurations up to Release 18 are listed in Annex B.

# References

[1] R4-2406685 WF on band combination TR template, ZTE, Nokia, Skyworks, Ericsson, RAN4#110bis

[2] R4-2404248 On simplifying analysis for triple beat products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

[3] R4-2405554 Template example for 2 band DL with 1 or 2 band UL up to 3 UL CCs, Skyworks Solutions Inc. , RAN4#110bis

[4] R4-2407231 Template for 2 band DL 1or2 band UL inter-band combination TR and , Skyworks Solutions Inc., Nokia, RAN4#111