# 3GPP TSG-RAN WG4 Meeting #111	R4-2407249

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.11.2.2

Source:	Apple

Title:	Simulation results for MU-MIMO with advanced receiver

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108 RAN4 concluded the Phase 1/ study on advanced receiver for MU-MIMO and the details of the study were captured in TR 38.878. In RAN4#110bis the parameters for defining requirements in phase 2 were discussed and WF [1] was agreed. In this contribution we present our simulation results for Phase 2 of this WI.

# 2. Simulation Results

In [1] we agreed on some simulation assumptions for further evaluation for defining requirements for MU-MIMO with advanced receiver.

## Simulation results when MO is signaled (DCI index 1~5)

For the agreed simulation assumptions in [1], in Tables 1 and 2 we provide the simulation results with co-scheduled UE modulation order signaled for FDD and TDD modes respectively.

Table 1: Results with MO signaled for FDD

| Case Number | Rank Combo | MIMO | Channel Model | Antenna config/ corr | MCS | MO of co- UE | SNR @ 70% Max TP (R-ML) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Alignment | Impairment |
| 1 | 1+1 | 2T2R | TDLC300-100 | 2x2 ULA medium | MCS 13 | QPSK | 14.0 | 16.0 |
| 2 |  | 2T4R |  |  |  |  | 13.6 | 15.6 |
| 3 | 2+2 | 4T4R | TDLA30-10 | ULA Low | MCS 17 | 16QAM | 16.8 | 19.1 |

Table 2: Results with MO signaled for TDD

| Case Number | Rank Combo | MIMO | Channel Model | Antenna config/ corr | MCS | MO of co- UE | SNR @ 70% Max TP (R-ML) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Alignment | Impairment |
| 4 | 1+1 | 2T2R | TDLC300-100 | 2x2 ULA medium | MCS 13 | QPSK | 14.0 | 16.0 |
| 5 |  | 2T4R |  |  |  |  | 13.5 | 15.5 |
| 6 | 2+2 | 4T4R | TDLA30-10 | ULA Low | MCS 17 | 16QAM | 16.7 | 19.0 |

## Simulation results when MO is not signaled (DCI index 6)

For the simulation assumptions agreed in [1] we provide results with blind modulation order detection for FDD and TDD in tables below.

Table 3: Results with blind detection of MO for FDD

| Case Number | Rank Combo | MIMO | Channel Model | Antenna correlation | MCS | MO of co- UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | R-ML (Alignment) | MMSE-IRC | R-ML (Impairment) |
| 7 | 1+1 | 2T2R | TDLC300-100 | ULA medium | MCS 13 | QPSK | 17.6 | 20.5 | 20.1 |
| 8 |  | 2T4R |  |  |  |  | 17.3 | 19.8 | 19.8 |
| 9 |  | 2T2R |  |  | MCS 17 | 16QAM | 22.3 | N/A | 24.8 |
| 10 |  | 2T4R |  |  |  |  | 21.7 | N/A | 24.2 |
| 11 |  | 2T2R |  | ULA Low | MCS 13 | QPSK | 12.1 | 12.6 | 14.6 |
| 12 |  | 2T4R |  |  |  |  | 5.9 | 6.2 | 8.4 |
| 13 | 2+2 | 4T4R | TDLA30-10 | ULA Low | MCS 13 | QPSK | 12.4 | 12.9 | 14.9 |
| 14 |  |  |  | XP medium |  |  | 14.0 | 14.5 | 16.5 |

Table 4: Results with blind detection of MO for TDD

| Case Number | Rank Combo | MIMO | Channel Model | Antenna correlation | MCS | MO of co- UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | R-ML (Alignment) | MMSE-IRC | R-ML (Impairment) |
| 15 | 1+1 | 2T2R | TDLC300-100 | ULA medium | MCS 13 | QPSK | 17.8 | 21.3 | 20.3 |
| 16 |  | 2T4R |  |  |  |  | 17.5 | 20.7 | 20.0 |
| 17 |  | 2T2R |  |  | MCS 17 | 16QAM | 22.6 | N/A | 25.1 |
| 18 |  | 2T4R |  |  |  |  | 22.0 | N/A | 24.5 |
| 19 |  | 2T2R |  | ULA Low | MCS 13 | QPSK | 12.3 | 12.8 | 14.8 |
| 20 |  | 2T4R |  |  |  |  | 6.1 | 6.4 | 8.6 |
| 21 | 2+2 | 4T4R | TDLA30-10 | ULA Low | MCS 13 | QPSK | 12.3 | 12.9 | 14.8 |
| 22 |  |  |  | XP medium |  |  | 13.9 | 14.5 | 16.4 |

Observations:

- For 1+1 with TDLC channel, Low antenna correlation, the performance with R-ML is very close to MMSE-IRC

- For 2+2 performance with R-ML with BD-MO is not significantly better than MMSE-IRC

For 1+1 with TDLC channel, Low antenna correlation, the performance with R-ML is very close to MMSE-IRC

For 2+2 performance with R-ML with BD-MO is not significantly better than MMSE-IRC

# 3. Conclusion

In this paper, we provide simulation results for requirements with advanced receiver for MU-MIMO. Our observations are captured below:

- For 1+1 with TDLC channel, Low antenna correlation, the performance with R-ML is very close to MMSE-IRC

For 2+2 performance with R-ML with BD-MO is not significantly better than MMSE-IRC

# Reference

- R4-2406114, “Way forward on [110bis][324] NR_demod_enh3_Part1”, China Telecom.