3GPP TSG-RAN WG4 Meeting # 111                               							 R4-24xxxxx

Fukuoka, Japan, 20th – 24th, April 2024

Source: 	Apple

Title: 	RF requirements in case of multiple contiguous LTE CCs

Agenda Item:	10.7.2.3

Document for:	Discussion

Introduction

There was a discussion on how to address multiple LTE CCs for inter-band non-collocated EN-DC with overlapping DL frequency and a WF was agreed in [1]. It is suggested to capture the following note to address multiple LTE CCs in non-collocated EN-DC scenario.

“NOTE 6:      For Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, REFSENS in this table equals to 5MHz REFSENS+10*log(aggregated BW(MHz)/5) of all the contiguous E-UTRA CCs of the wanted band. BWwanted and BWanother represent the aggregated BWs of all the CCs of the wanted and another wanted band, respectively. The maximum power spectral density imbalance between the contiguous E-UTRA CCs in one band, is within 6 dB.”

While after a further consideration of this note, we think it is not enough to solve the issue. This paper intends to present an updated proposal to address the issue.

Discussion

Terminology change, currently the wanted carrier and another wanted carrier are used as the terminology for description of the concerned CCs. If we want to consider multiple CCs for LTE mode, the terminologies need to be changed to wanted carrer (s) and other wanted carrier (s).

Proposal: it is proposed to make terminology change for power imbalance requirement to take into account multiple CCs in LTE mode.

- Wanted carrier.  wanted carrier (s)

- Another wanted carrier  other wanted carrier (s)

- BWanother  BWohter

Table 7.10B.3-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship (Center of BWother Relative to edge of BWwanted) |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier (s) | REFSENS + 1 | BWwanted ≤ BWohter | < max (5/2* BWohter, 50MHz) |
|  | Other wanted carrier (s) with overlapping DL bands | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier (s) | REFSENS + 1 | BWwanted > BWohter |  |
|  | Other wanted carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier (s) | REFSENS + 1 | NA | ≥ max (5/2* BWother, 50MHz) |
|  | Other wanted carrier (s) with overlapping DL bands | Power of wanted carrier + 25 |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 24dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 3: BWwanted is the channel bandwidth of wanted carrier. BWohter is the channel bandwidth of another wanted carrier with overlapping DL bands NOTE 4: REFSENS is the reference sensitivity level or two antenna port in Table 7.3.2-1a or in Table 7.3.2-1b of TS 38.101-1, or in Table 7.3.1-1 of TS 36.101. NOTE 5: For TDD-TDD combination, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between E-UTRA and NR carriers for the EN-DC combinations in Table 7.10B.3-2. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. |  |  |  |  |

For the agreed NOTE 6 wording, it’s better to consider a little bit re-organization as below:

For Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, BWwanted and BWother for E-UTRA with multiple CCs represent the aggregated BWs of all the CCs. If E-UTRA with multiple CCs are wanted carriers, REFSENS equals to 5MHz REFSENS+10*log(aggregated BW(MHz)/5) of all the contiguous CCs. if E-UTRA with multiple CCs are other wanted carriers, the calculated Rx power are total power for all component CCs. The maximum power spectral density imbalance between the contiguous E-UTRA CCs in one band, is within 6 dB.

Proposal 2: It is proposed to agree the modified NOTE 6 wording as below:

- NOTE 6: For Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, BWwanted and BWother for E-UTRA with multiple CCs represent the aggregated BWs of all the CCs. If E-UTRA with multiple CCs are wanted carriers, REFSENS equals to 5MHz REFSENS+10*log(aggregated BW(MHz)/5) of all the contiguous CCs. if E-UTRA with multiple CCs are other wanted carriers, the calculated Rx power are total power for all component CCs. The maximum power spectral density imbalance between the contiguous E-UTRA CCs in one band, is within 6 dB.

Summary

This contribution presented our consideration on RF requirements in case of multiple contiguous LTE CCs. The following proposals are concluded.

Proposal: it is proposed to make terminology change for power imbalance requirement to take into account multiple CCs in LTE mode.

- Wanted carrier.  wanted carrier (s)

- Another wanted carrier  other wanted carrier (s)

- BWanother  BWohter

Proposal 2: It is proposed to agree the modified NOTE 6 wording as below:

- NOTE 6: For Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, BWwanted and BWother for E-UTRA with multiple CCs represent the aggregated BWs of all the CCs. If E-UTRA with multiple CCs are wanted carriers, REFSENS equals to 5MHz REFSENS+10*log(aggregated BW(MHz)/5) of all the contiguous CCs. if E-UTRA with multiple CCs are other wanted carriers, the calculated Rx power are total power for all component CCs. The maximum power spectral density imbalance between the contiguous E-UTRA CCs in one band, is within 6 dB.”

- References

[1] RP-2406628, WF on clarification of RF requirements in case of contiguous LTE CCs in non-collocated scenario, Huawei, RAN4#110bis