3GPP TSG-RAN WG4 Meeting #111	R4-2407377

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	10.14.4

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In the last meeting, Rel-19 LP-WUS/WUR discussion was started and WFs were summarized in [1]. This document proposes RRM core requirements for LP-WUS/WUR based on the WFs.

- Discussion

During last meeting, some companies propose entry/exit condition for LP-WUS monitoring but it was not discussed due to time limitation. Entry/exit metric should be discussed by other WGs, but the interruption due to MR/LP-WUR switching should be discussed as a RAN4 RRM requirements. For IDLE/INACTIVE states, since MR enters/exits ultra-deep sleep, interruption will be occurred during transition periods.

Proposal 1: The interruption requirements for MR measurement during entry/exit transition periods should be defined. For LP-WUR, the necessity of interruption requirements needs further discussion.

On the other hand, for CONNECTED state, since MR does not enter ultra-deep state according to WID and the enhanced MR functionality is discontinuous PDCCH monitoring triggered by LP-WUS. Thus, no additional RRM requirements are needed so far.

Proposal 2: For CONNECTED state, no additional RRM requirements are needed unless other functionality is defined by other WGs.

- Conclusion

In this contribution, we propose possible RRM core requirements for LP-WUS/WUR

Proposal 1: The interruption requirements for MR measurement during entry/exit transition periods should be defined. For LP-WUR, the necessity of interruption requirements needs further discussion.

Proposal 2: For CONNECTED state, no additional RRM requirements are needed unless other functionality is defined by other WGs.

References

[1] 3GPP, R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo