3GPP TSG-RAN WG4 Meeting #111	R4-2407380

Fukuoka City, Japan 20th – 24th May 2024

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FWA PC1 B106 A-MPR

Agenda Item: 			6.14.2

Document for:	Approval

# 1	Introduction

We study the A-MPR to protect bands 5 and 26 from emissions due to PC1 B106.

# 2	Simulation assumptions and scenario

- 3 MHz LTE channel centered at 899 MHz

- Power class PC1

- Baseband filtering

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only band 5 and 26 protection is simulated. Other gating factors (ACLR, SEM, etc.) are excluded.

Table 1: Spectrum emission limit to protect bands 5 and 26.

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 869 ≤ f ≤ 894 | -30 | 1 MHz |

# 3	Simulation results and discussion

According to our simulations no A-MPR is needed to protect bands 5 and 26.

Proposal 1: RAN4 shall consider defining no A-MPR for PC1 B106 to protect bands 5 and 26.

# 4	Conclusions

We have studied the A-MPR to protect bands 5 and 26 from emissions due to PC1 B106. Based on the simulation results we have made the following proposal.

Proposal 1: RAN4 shall consider defining no A-MPR for PC1 B106 to protect bands 5 and 26.