3GPP TSG-RAN WG4 Meeting #111	R4-2407395

# Fukuoka, Japan, 20th - 24th, May 2024

Source: 	KDDI, Samsung

Title: 	Discussion on B28 andn28 Full band duplexer

Agenda Item:	6.19.2

Document for:	Discussion

- 1. Introduction

In RAN#110-bis meeting, the WF on a new RAN4 spectrum WI for B28 and n28 full band duplexer architecture was discussed [1]. We describe our observations and proposals for it in this paper.

- 2. Discussion

2.1 Background

Based on agreements from RAN4 #110 [2], RAN4 discussed proposals on how to specify PC2 operation in band n28 [3]-[6]. The industry demand to enable full-band duplexer operation in band n28 is recognized. And then, the WF [1] recommended the following for RAN plenary’s consideration during the RAN#104 meeting:

- To conclude the Rel-18 FDD HPUE work without specifying PC2 requirements for band n28 - To initiate new Rel-19 spectrum effort to enable the band 28/n28 full-band duplexer architecture in NR PC2/PC3 and LTE PC3 requirements - TBD whether this is done in one or two work items - This work will include effort to specify new NR CBWs for band n28 - Efforts shall be made to introduce requirements accommodating dual duplexer and full band duplexer - RAN4 to discuss whether a single set of requirements for 1Tx NR PC2 is feasible - Release independence of NR requirements to [Rel-15] is anticipated

With that, the WF [1] proposes a new Rel-19 spectrum WI to enable the band28/n28 full-band duplexer architecture in NR PC3 and LTE PC3 requirements.

Observation 1: The WF R4-2406708 proposes a new Rel-19 spectrum WI to enable the B28 and n28 full-band duplexer architecture in NR PC3 and LTE PC3 requirements.

2.1 Existing B28/n28 network deployment

Currently, there are not any A-MPR requirement for B28/n28 PC3 NS_17 in RAN4 specifications (i.e. A-MPR=0). Those current specs are based on only dual band duplexer architecture. Generally speaking, a bottle neck of a total link-budget is UL link-budget. Therefore, existing Japanese operators has already deployed their B28/n28 network based on UL link-budget with A-MPR=0 for PC3.

Observation 2: Existing Japanese operators has already deployed their B28/n28 network based on UL link-budget with A-MPR=0 for PC3.

2.2 Impacts on B28/n28 PC3 A-MPR for NS_17

If RAN4 agree with adding new requirement A-MPR>0 for PC3, legacy UE should meet the current requirement A-MPR=0. On the other hand, a new UE will need to meet the new requirement A-MPR>0. This means that a new UE’s performance/coverage will degrade in B28/n28 networks already deployed based on a link-budget with A-MPR=0 for PC3 compared to legacy UE.

Proposal 1: Avoid causing UE’s performance/coverage degradation due to adding a new PC3 A-MPR>0 for NS_17 from an aspect of MNO operating PC3 B28/n28 with A-MPR=0.

With that, newly adding A-MPR=0 will give operators’ networks serious impacts, therefore RAN4 needs to separate PC2 and PC3.

Proposal 2: Separate PC2 and PC3 A-MPR for NS_17 on full band duplexer discussions.

2.3 New Rel-19 spectrum WI discussions

Actually, in previous RAN4 meetings, the industry demand to enable full-band duplexer operation in band n28 is recognized shown as [2]-[6]. On the other hand, WF [1] proposes new WI(s) to RAN Plenary. Generally speaking, RAN4 doesn’t technically need any WF/recommendation/conclusions on this topic, because new Rel-19 WI(s) for RAN4 spectrum should be discussed and concluded in only RAN Plenary.

Proposal 3: RAN4 doesn’t technically need any WF/recommendation/conclusions on this topic.

Proposal 4: New Rel-19 WI(s) should be discussed and concluded in only RAN Plenary.

- 3. Conclusion

Observation 1: The WF R4-2406708 proposes a new Rel-19 spectrum WI to enable the B28 and n28 full-band duplexer architecture in NR PC3 and LTE PC3 requirements.

Observation 2: Existing Japanese operators has already deployed their B28/n28 network based on UL link-budget with A-MPR=0 for PC3.

Proposal 1: Avoid causing UE’s performance/coverage degradation due to adding a new PC3 A-MPR>0 for NS_17 from an aspect of MNO operating PC3 B28/n28 with A-MPR=0.

Proposal 2: Separate PC2 and PC3 A-MPR for NS_17 on full band duplexer discussions.

Proposal 3: RAN4 doesn’t technically need any WF/recommendation/conclusions on this topic.

Proposal 4: New Rel-19 WI(s) should be discussed and concluded in only RAN Plenary.

- 4. Reference

[1] 	R4-2406708, “WF on band n28 full band duplexer”, Apple, R4#110-bis

[2]	R4-2403629, “WF on HPUE for FDD bands,” China Unicom, Skyworks, R4#110

[3]	R4-2404193, “On HPUE for NR FDD bands,” Apple, R4#110-bis

[4]	R4-2404878, “Remaining issues for FDD HPUE A-MPR,” Huawei, HiSilicon, R4#110-bis

[5]	R4-2405712, “PC2 A-MPR for bands n7 and n28,” Qualcomm, R4#110-bis

[6]	R4-2405962, “n28 NS_17 PC2,” Skyworks, R4#110-bis