3GPP TSG-RAN WG4 Meeting # 111	R4-2407483

# Fukuoka, JP, May 20 – 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDCCH ordered Random Access delay for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In PDCCH ordered Random Access delay for LTM, the following should be further revised - The value of TSSB should be further clarified. - Except for cases where TSSB = 0, there are still a case where TSSB has no requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce further clarification for the value of TSSB in clause 6.2.2C.2 and remove FFS. - Revised the description of cases other than TSSB = 0 and capture the exception case in clause 6.2.2C.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDCCH ordered RACH delay requirements for LTM would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) 6.2.2C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 6.2.2C.2	PDCCH ordered Random Access delay

UE shall complete the RACH transmission as defined in TS38.213 [3] clause 8.1, in which TSSB and ∆RF/BB preparation are defined as the following:

-	TSSB is the time for T/F tracking

-	TSSB = 0 if the following conditions are met.

-	The target LTM neighbor cell is on an FR1 carrier whose TCI state associated with SSB indicated in the PDCCH-order is activated and the time gap between receiving the MAC-CE activating the target TCI state and PDCCH order is larger than [TBD], and

-  L1-RSRP measurement period of candidate cell is not larger than 160ms or

-	The time between receiving the MAC-CE activating the target TCI state and PDCCH order is not larger than 160ms.

-	 TSSB is as follows if the target LTM neighbor cell is on an FR1 carrier whose TCI state associated with SSB indicated in the PDCCH-order is not activated and the target LTM neighbor cell is on an FR2 carrier.

-	TSSB = Tfirst-SSB_RACH + TSSB-proc, where [TSSB

- Tfirst-SSB_RACH is the time to first SSB transmission after PDCCH-order RACH command is decoded by the UE when SSB is within active BWP

- Tfirst-SSB_RACH is the time to first SSB transmission overlapped with MGL after PDCCH-order RACH command is decoded by the UE when SSB is outside active BWP

- TSSB-proc = 2 ms;

-	TSSB

-  [The requirements does not apply to the RACH transmission if the target LTM neighbor cell on an FR1 carrier and

-  activation command of the TCI state associated with SSB of the neighbor cell indicated in the PDCCH-order is received, and the time gap between receiving the MAC-CE activating the target TCI state and PDCCH order is larger than [TBD], and L1-RSRP measurement period for the SSB is larger than 160 ms.]

-	∆RF/BB preparation is the time for RF and baseband preparation. The following apply:

-	If PRACH bandwidth is outside active UL BWP of the SpCell but within one of configured UL BWPs of other active serving cell, the ∆RF/BB preparation is the DCI based BWP switching delay specified in clause 8.6.

-	If PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB preparation equals to capability value reported in [FG 39-5: RF/BB preparation time for PDCCH-order RACH].

-	Otherwise, ∆RF/BB preparation = 0.

<End of Change 1>