3GPP TSG-RAN WG4 Meeting # 111	R4-2407485

# Fukuoka, JP, May 20 – 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test cases for intra-f RACH based PSCell Cell switch from FR1 to FR1 for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The interruption requirements are also tested by using the parameters in Table A.6.3.y.1.2-2 and Table A.6.3.y.1.2-3. - The wording “Cell 1 and Cell 2 on radio channel 1 are powered on” is misunderstood as “Cell 1 and Cell 2 are not powered on before T1” - The procedure “UE establishes a connection with the Cell 1” should be done before T1. - The test configurations for PCell and two PSCells for LTM should be further clarified. - Some parameters are missing or not accurate. - Some typos need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the following test case in clause A.6.3.y.1 in TS 38.133 for LTM: - Add ‘interruption length is tested by using the parameters’. - Remove the following sentences in clause A.6.3.y.1.2: - Cell 1 and Cell 2 on radio channel 1 are powered on. - UE establishes a connection with the Cell 1 - The test configurations for intra-frequency PSCell cell switch are further clarified and become complete in Table A.6.3.y.1.2-1 in clause A.6.3.y.1.2. - Remove Candidate UL TCI.State.1 in Table A.6.3.y.1.2-2. - Add and correct some parameters in clause A.6.3.y.1.2. - Correct the typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for intra-f RACH based PSCell Cell switch from FR1 to FR1 for LTM would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) A.6.3.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.3.y	LTM PSCell Switch

A.6.3.y.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1

A.6.3.y.1.1	Test Purpose and Environment

This test is to verify the intra-frequency RACH based LTM PSCell switch requirements from NR FR1 to NR FR1 specified in clause 8.20 for both with and without early TCI state activation.

A.6.3.y.1.2	Test Parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. Test configurations are given in Table A.6.3.y.1.2-1. Both cell switch delay and interruption length is tested by using the parameters in Table A.6.3.y.1.2-2 and A.6.3.y.1.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 3, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 3 to Cell 2.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 3

-	Joint TCI state configuration as defined in Table A.6.3.y.1.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.6.3.y.1.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 3) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 3.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 3.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the instant when the last TTI containing LTM cell switch command MAC CE is sent by Cell 2 to the UE.

-	In the cell switch command, Cell 3 is the target cell for PSCell switch. Contention-Free Random -Access Resources are indicated and the field of Timing Advance Command is set to FFF.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	T4 ends upon the reception of PRACH at Cell 3.

Table A.6.3.y.1.2-1: Intra-frequency cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Source PSCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PSCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Source PSCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PSCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | PCell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Source PSCell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PSCell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.y.1.2-2: General test parameters for Intra-frequency cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cells |  | Cell 1 (PCell), Cell 2 (PSCell) |  |  |  |  |
|  | Neighbouring cell |  | Cell 3 |  |  |  | Cell 3 is the candidate cell |
| Final condition | Active cells |  | Cell 1 (PCell), Cell 3 (PSCell) |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cell 2 and cell 3 |  |  | 2 s |  |  |  | RTD between cell 2 and cell 3 is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 3) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.6.3.y.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra-frequency cell switch test case

| Parameter |  |  |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 ~ T4 | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  |  |  | 1 | 2 | 2 |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |  |
|  |  |  |  | Config 2, 3 |  | TDD |  |  |
| TDD configuration |  |  |  | Config 1 |  | Not Applicable |  |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |
|  |  |  |  | Config 2 | MHz | 10: NRB,c = 52 |  |  |
|  |  |  |  | Config 3 | MHz | 40: NRB,c = 106 |  |  |
| BWP BW |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |
|  |  |  |  | Config 2 | MHz | 10: NRB,c = 52 |  |  |
|  |  |  |  | Config 3 | MHz | 40: NRB,c = 106 |  |  |
| PDSCH Reference |  |  |  | Config 1 |  | SR.1.1 FDD |  |  |
|  |  |  |  | Config 2 |  | SR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD |  |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | CR2.1 TDD |  |  |
| CP length |  |  |  | Config 1, 2, 3 |  | Normal |  |  |
| TRS configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |  |  |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  | Config 1, 2, 3 |  | OP.1 |  |  |
| SMTC Configuration |  |  |  | Config 1, 2, 3 |  | SMTC.1 |  |  |
| SSB Configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 |  |  |
|  |  |  |  | Config 3 | kHz | 30 |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 |  |  |
|  |  |  |  | Config 3 | kHz | 30 |  |  |
| PRACH configuration |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuration |  |  | Initial DL BWP | Config 1, 2, 3 |  | DLBWP.0.1 |  |  |
|  |  |  | Dedicated DL BWP | Config 1, 2, 3 |  | DLBWP.1.1 |  |  |
|  |  |  | Initial UL BWP | Config 1, 2, 3 |  | ULBWP.0.1 |  |  |
|  |  |  | Dedicated UL BWP | Config 1, 2, 3 |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1, 2, 3 | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  | Config 1, 2, 3 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  | Config 1, 2, 3 |  |  |  |  |
| Note2 |  |  |  | Config 1, 2, 3 | dBm/15kHz | -98 |  |  |
| Note2 |  |  |  | Config 1, 2 | dBm/SCS | -98 |  |  |
|  |  |  |  | Config 3 |  | -95 |  |  |
|  |  |  |  | Config 1, 2, 3 | dB | 11 | 0.64 | 0.64 |
|  |  |  |  | Config 1, 2, 3 | dB | 11 | [TBD] | [TBD] |
| SSB_RP |  |  |  | Config 1, 2 | dBm/SCS | -87 | -90 | -90 |
|  |  |  |  | Config 3 | dBm/SCS | -84 | -87 | -87 |
| IoNote3 |  |  |  | Config 1, 2 | dBm/ 9.36MHz | -58.7 | -58.7 | -58.7 |
|  |  |  |  | Config 3 | dBm/ 38.16MHz | -52.62 | -52.62 | -56.6 |
| Propagation condition |  |  |  | Config 1, 2, 3 | - | AWGN | AWGN | AWGN |
|  |  | Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.3.y.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 3 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switch observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (=Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU_=20ms

-	TLTM-RRC-processing =10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing =10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing =15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing =20 ms if the UE does not support [faster LTM processing] capability.

<End of Change 1>