3GPP RAN WG4 Meeting #111	R4-2407490

Fukuoka, Japan, May 20 – May 24, 2024

Title: 	Discussion on Core requirements of Sidelink positioning and CPP

Source: 	CATT

Agenda item:	7.12.1.2

Document for:	Discussion

Introduction

In RAN4#110bis meeting, fruitful progress regarding the core requirements was achieved which are captured in the WF [1]. In this paper, we will further discuss the open issues related to core requirements of sidelink positioning and CPP.

Discussion

Sidelink positioning

Issue 1-1: Sync source change at Tx side

Regarding the synchronization source change at Tx side, there is no agreement in last meeting, and we will further discuss it in this contribution. Some options are listed in [1] for reference.

Proposals - Option 1a: - When the synchronization reference source change occurs during the measurement period at Tx side, e.g., UE receives an updated sl-RTD-Info, the measuring UE shall restart the SL PRS-based timing measurements (SL Rx-Tx, SL RSTD, and SL RTOA). - Option 1b: - To ensure that the restart of the SL measurement due to the synch source change is not too much delayed, RAN4 specifies the time until which the UE shall report the restarted measurement (e.g., as in LTE) which is extended by the measurement period with each restart (K): TSL RSTD/Rx-Tx/RTOA,restart=K+1*TSL RSTD/Rx-Tx/RTOA, Total . - No need to specify the maximum allowed value for the number K of restarts. - Option 2: - For synchronization reference source change occurs at Tx side, measurement accuracy requirements do not apply and no specific UE behaviour is defined. - Option 3: - RAN4 to send an LS to RAN1 for getting clarification on how the measuring UE is informed about the synchronization reference source change at the Tx UE side. - Option 4: - If a UE receives an updated sl-RTD-Info while performing SL RSTD or SL RTOA measurements, the UE is allowed to restart the measurements and the measurement period can be longer. - Option 5: - If the UE receives an indication of the synchronization source change at a transmitting anchor UE by SL-RTD-Info [37], while the UE is performing the [TBD] measurements, then the measuring UE shall restart the SL RTOA measurement after the synchronization reference source change, in which case the UE is allowed to extend the TSL RTOA,total measurement period.

In our understanding, when UE is indicated of the synchronization source change at anchor UE via SL-RTD-Info during the positioning measurements, the UE shall restart the measurements after the synchronization source change. Otherwise, the synchronization source change may be unknown to the measuring UE, and UE can continue the positioning measurements, while the measurement results may not satisfy the accuracy requirements which shall not apply in such cases.

Proposal 1: When UE is indicated of the synchronization source change at anchor UE via SL-RTD-Info during the positioning measurements, the UE shall restart the measurements after the synchronization source change.

Carrier phase positioning

Issue 2-1: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD

An open issue in CPP is shown below for further discussion:

FFS: - For UE configured for CPP measurement with legacy measurement with multiple PFLs, When LMF does not configure measurement time window(s) for a PFL or UE does not support FG 41-2-3, - Carrier phase positioning measurement is only performed on single PFL. - FFS which PFL is measured. - Existing requirements for RSTD/UE Rx-Tx without time window apply. - For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and does not support FG 41-2-8/41-2-9, - Option 1: - Existing requirements for RSTD/UE Rx-Tx without time window apply. - Option 2: - existing requirements without time window apply for legacy measurements, but UE is not required to report CP if the resource set(s) associated to the window have occasions outside the time window. - Option 3: - Not to define requirements for this case.

For the 1st bullet, since the UE is not indicated the time window and the PFL on which the CPP is performed, it, in our opinion, can be decided by UE implementation.

For the 2nd bullet, we prefer option 2 since the legacy measurements should be ensured and, if possible, UE can perform CPP measurements on the PRS resources within indicated time window(s). But for the PRS resources outside the time window, UE is not required to report the carrier phase positioning measurements on them.

Proposal 2: When multiple PFLs are configured and UE is requested to perform CPP with time window(s) not indicated or FG 41-2-3 not supported, on which PFL the CPP is performed depends on UE implementation.

Proposal 3: When multiple PFLs and time window(s) are configured and UE is requested to perform CPP with FG 41-2-8/9 not supported, existing requirements without time window apply for legacy measurements, but UE is not required to report the carrier phase measurements on the PRS resources outside the time window.

Conclusions

This paper discussed the core requirements for Sidelink positioning and CPP, and the following proposals are provided:

SL positioning:

Proposal 1: When UE is indicated of the synchronization source change at anchor UE via SL-RTD-Info during the positioning measurements, the UE shall restart the measurements after the synchronization source change.

CPP:

Proposal 2: When multiple PFLs are configured and UE is requested to perform CPP with time window(s) not indicated or FG 41-2-3 not supported, on which PFL the CPP is performed depends on UE implementation.

Proposal 3: When multiple PFLs and time window(s) are configured and UE is requested to perform CPP with FG 41-2-8/9 not supported, existing requirements without time window apply for legacy measurements, but UE is not required to report the carrier phase measurements on the PRS resources outside the time window.

References

[1] R4-2406384, WF on SL positioning and carrier phase positioning, CATT, RAN4#110bis.