3GPP RAN WG4 Meeting #111	R4-2407494

Fukuoka, Japan, May 20 – May 24, 2024

Title: 	Discussion on Performance requirements of PRS BW aggregation

Source: 	CATT

Agenda item:	7.12.2.5

Document for:	Discussion

Introduction

In RAN4#110bis meeting, fruitful progress regarding the performance requirements was achieved which are captured in the WF [1]. In this paper, we will further discuss a few open issues related to performance requirements.

Discussion

Bandwidth for PRS aggregation accuracy requirements

In RAN4#110bis meeting, an agreement on bandwidth for PRS aggregation accuracy requirements is achieved. Related agreements are copied below for information:

Agreements: - RAN4#110 - For PRS BW aggregation for RSTD and UE Rx-Tx, the requirements are to be defined at least for the following per-PFL BWs: - 15 kHz SCS: 104 PRBs (20 MHz) - 30 kHz SCS: 132 PRBs (50 MHz), 272 (100 MHz) - 60 kHz SCS FR1: 64 PRBs (50 MHz), 132 (100 MHz) - 60 kHz SCS FR2: 64 PRBs (50 MHz), 132 (100 MHz) - 120 kHz SCS: 64 PRBs (100 MHz), 128 (200 MHz) - Note: The aggregated PFLs will have the same SCS. - FFS: Other BWs. Agreement: - RAN4#110bis FFS: For RSTD and UE Rx-Tx measurements based on PRS aggregation, in addition to the agreed per-PFL BWs, requirements are also defined for per-PFL BW 400 MHz (272 PRBs, 120 kHz SCS) in FR2.

Based on the above agreements, we have the following proposal for the accuracy table for RSTD measurements with BW aggregation in AWGN channel with 4 samples as examples for alignment of accuracy tables:

Table 1: RSTD measurements with 2 PFLs aggregation in AWGN in FR1 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | PRB | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 104 | ≥ 208 |
| TBD |  | 30 | 132 | ≥ 264 |
| TBD |  |  | 272 | ≥ 544 |
| TBD |  | 60 | 64 | ≥ 128 |
| TBD |  |  | 132 | ≥ 264 |

Table 2: RSTD measurements with 3 PFLs aggregation in AWGN in FR1 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | PRB | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 104 | ≥ 312 |
| TBD |  | 30 | 132 | ≥ 396 |
| TBD |  |  | 272 | ≥ 816 |
| TBD |  | 60 | 64 | ≥ 192 |
| TBD |  |  | 132 | ≥ 396 |

Table 3: RSTD measurements with 2 PFLs aggregation in AWGN in FR2 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 64 | ≥ 128 |
| TBD |  |  | 132 | ≥ 264 |
| TBD |  | 120 | 64 | ≥ 128 |
| TBD |  |  | 128 | ≥ 256 |
| TBD |  |  | 272 | ≥ 544 |

Table 4: RSTD measurements with 3 PFLs aggregation in AWGN in FR2 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 64 | ≥ 192 |
| TBD |  |  | 132 | ≥ 396 |
| TBD |  | 120 | 64 | ≥ 192 |
| TBD |  |  | 128 | ≥ 384 |
| TBD |  |  | 272 | ≥ 816 |

Proposal 1: Companies to align the structure of accuracy tables in different clauses, and the following tables are proposed as examples:

Table 1: RSTD measurements with 2 PFLs aggregation in AWGN in FR1 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | PRB | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 104 | ≥ 208 |
| TBD |  | 30 | 132 | ≥ 264 |
| TBD |  |  | 272 | ≥ 544 |
| TBD |  | 60 | 64 | ≥ 128 |
| TBD |  |  | 132 | ≥ 264 |

Table 2: RSTD measurements with 3 PFLs aggregation in AWGN in FR1 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | PRB | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 104 | ≥ 312 |
| TBD |  | 30 | 132 | ≥ 396 |
| TBD |  |  | 272 | ≥ 816 |
| TBD |  | 60 | 64 | ≥ 192 |
| TBD |  |  | 132 | ≥ 396 |

Table 3: RSTD measurements with 2 PFLs aggregation in AWGN in FR2 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 64 | ≥ 128 |
| TBD |  |  | 132 | ≥ 264 |
| TBD |  | 120 | 64 | ≥ 128 |
| TBD |  |  | 128 | ≥ 256 |
| TBD |  |  | 272 | ≥ 544 |

Table 4: RSTD measurements with 3 PFLs aggregation in AWGN in FR2 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 64 | ≥ 192 |
| TBD |  |  | 132 | ≥ 396 |
| TBD |  | 120 | 64 | ≥ 192 |
| TBD |  |  | 128 | ≥ 384 |
| TBD |  |  | 272 | ≥ 816 |

Accuracy requirements for PRS-RSRP and PRS-RSRPP measurements based on aggregated PRS resources

Another open issue is how to apply the existing PRS-RSRP/PRS-RSRPP accuracy requirements to measurements with BW aggregation, and two options are provided for down-selection.

Accuracy requirements for PRS-RSRP and PRS-RSRPP measurements based on aggregated PRS resources Agreements: FFS: - Option 1: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement for the largest PRS BW among the aggregated PFLs applies to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation. - Option 2: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

In our opinion, which option is reasonable depends on how the PRS-RSRP/PRS-RSRPP measurements are performed. If PRS-RSRP(P) measurements are based on one of the aggregated PFLs, then the option 1 is preferred. If they are based on all aggregated PFLs, option 2 is preferred. In RAN1#115 meeting, an agreement, which can be used as reference to resolve this issue, is copied below:

Agreement – RAN1#115 If the UE/gNB reports aggregated timing measurement, the single reported RSRP/RSRPP (if reported) is based on aggregated PRS/SRS resources across aggregated PFLs/carriers. - Note1: it is up to RAN4 whether to define a corresponding requirement - Note2: for UL, measured SRS signals refer to aggregated SRS resources. For DL, measured PRS signals refer to aggregated PRS resources.

According to the agreement, the single reported RSRP/RSRPP (if reported) is based on aggregated PRS/SRS resources across aggregated PFLs/carriers when they are reported with aggregated timing measurements. Similarly, PRS-RSRP/ RSRPP measurements can also be performed based on aggregated PRS resources when they are configured to be measured without timing measurements. Therefore the option 2 is more reasonable.

Proposal 2: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/ RSRPP measurements in case of PRS BW aggregation.

Considerations for performance requirements for PRS aggregation

In last meeting, an FFS is that the measurement accuracy requirement for RSTD/UE Rx-Tx time difference should depend on the baseband sampling rate as well as the number of PFLs.

Agreements: - The number of samples in accuracy requirements for PRS bandwidth aggregation: - Option 1: both reduced number of samples and 4 samples. - FFS: The measurement accuracy requirement for RSTD/UE Rx-Tx time difference should depend on the baseband sampling rate as well as the number of PFLs.

In regard to the number of PFLs, we think the issue has been settled that the measurement accuracy requirements are defined separately for 2 PFLs and 3 PFLs. For the baseband sampling rate, we do not think it is necessary to include it into accuracy tables, since they are not in the tables in legacy requirements. We prefer to stick to the same rules to keep the consistency of measurement accuracy requirements defined in different releases.

Proposal 3: Keep the consistency of measurement accuracy requirements and do not include baseband sampling rate into measurement accuracy tables.

Conclusions

This contribution discussed the performance requirements for BW aggregation positioning, and the following proposals are provided:

Proposal 1: Companies to align the structure of accuracy tables in different clauses, and the following tables are proposed as examples:

Table 1: RSTD measurements with 2 PFLs aggregation in AWGN in FR1 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | PRB | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 104 | ≥ 208 |
| TBD |  | 30 | 132 | ≥ 264 |
| TBD |  |  | 272 | ≥ 544 |
| TBD |  | 60 | 64 | ≥ 128 |
| TBD |  |  | 132 | ≥ 264 |

Table 2: RSTD measurements with 3 PFLs aggregation in AWGN in FR1 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | PRB | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 104 | ≥ 312 |
| TBD |  | 30 | 132 | ≥ 396 |
| TBD |  |  | 272 | ≥ 816 |
| TBD |  | 60 | 64 | ≥ 192 |
| TBD |  |  | 132 | ≥ 396 |

Table 3: RSTD measurements with 2 PFLs aggregation in AWGN in FR2 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 64 | ≥ 128 |
| TBD |  |  | 132 | ≥ 264 |
| TBD |  | 120 | 64 | ≥ 128 |
| TBD |  |  | 128 | ≥ 256 |
| TBD |  |  | 272 | ≥ 544 |

Table 4: RSTD measurements with 3 PFLs aggregation in AWGN in FR2 with 4 samples

| Accuracy | PRS Ês/Iot | PRS SCS | BW per PFL | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 64 | ≥ 192 |
| TBD |  |  | 132 | ≥ 396 |
| TBD |  | 120 | 64 | ≥ 192 |
| TBD |  |  | 128 | ≥ 384 |
| TBD |  |  | 272 | ≥ 816 |

Proposal 2: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/ RSRPP measurements in case of PRS BW aggregation.

Proposal 3: Keep the consistency of measurement accuracy requirements and do not include baseband sampling rate into measurement accuracy tables.

References

[1] R4-2406383, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson, RAN4#110bis.