3GPP TSG-RAN WG4 Meeting #111	R4-2407711

# Fukuoka, Japan, May 20-24, 2024

Source:	T-Mobile USA, Qualcomm, Skyworks Solutions, Inc.

Title:	TP for TR38.850: FDD PC2 n25 and PC2 n66 with DL CA_n25A-n66A

Agenda item:	6.18.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce MSD for PC2 n25 and PC2 n66 with DL CA_n25A-n66A. PC2 n25 and PC2 n66 with DL CA_n25A-n66A were previously completed based on there being no MSD for PC3 for these combinations [1].

Since then, there have been discussions that there should be cross-band isolation for the wider uplink channel bandwidths of up to 45 MHz for n66 and up to 40 MHz for n25 which have been introduced since CA_n25A-n66A was standardized.

This TP is a work in progress. It is anticipated that the proposed values will be updated during RAN4#111.

- Discussion

Offline discussions were held between RAN4#110bis and RAN4#111.

For n25 PC2 with 40 MHz UL channel BW and n66 PC2 with 45 MHz UL channel BW, cross-band isolation MSD analyses were provided in [1] and [2] based on simulation and measurement data.

For cross-band isolation MSD, It is agreed that the MSD will be around 0.3 dB and will not be added.

For PC2 single Tx for n66, it is also agreed that no MSD will be added.

For single Tx, the following has been proposed based on averaging proposals from the companies.

PC2 with 1Tx:

| UL band | DL band |  | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n66 |  | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.5 Skyworks 0.8 QC Avg. 0.7dB | >ACLR2 |
| n66 | n25 |  | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.9 Skyworks 2.5 QC Avg. 2.2 9dB | >ACLR2 |

For 2 Tx n66 PC2 MSD and 1 and 2 Tx n25 PC, the following has been proposed based in averaging proposals from various companies:

PC2 with 2Tx:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.7 SKW 1.1 QC Avg. 0.9 | >ACLR2 |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 2.8 SKW 4.7 QC Avg. 3.8 | >ACLR2 |

- Reference

[1] R4-2402463 “Draft CR for 38.101-1: T-Mobile USA PC2 FDD with no MSD,” T-Mobile USA

[2] R4-2407166: PC2 CA_n2-n66 CA_n25-n66 MSD; Skyworks Solutions Inc.

[3] R4-240xxxx: CA_n25A-n66A and CA_n2A-n66A PC2 and PC3 MSD evaluation; Qualcomm France

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n25A-n66A

5.x.1	UE maximum output power

The addition of the PC2 notes for n25 and n66 have already been added to 38.101-1 as follows:

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier10 |  | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n66A |  | n25A8 n66A8 CA_n25A-n66A |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 30, 40 |  |  |  |
|  |  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |  |
|  |  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

### 5.x.2	Cross-band isolation reference sensitivity requirements

#### 5.x.2.0	General

For PC3, CA_ n25A-n66A has no MSD for UL n25 or UL n66 currently. This may be updated based on wider channel bandwidths.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n25 without TxD

For n25 PC2 with 40 MHz UL channel BW, cross-band isolation MSD is considered based on simulation and measurement data. For single Tx PC2 n25 the following MSD is proposed:

Table 5.x.2.1-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.74 | >ACLR2 |

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n25 with TxD

For dual Tx PC2 n25 the following MSD is proposed:

Table 5.x.2.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.95 | >ACLR2 |

#### 5.x.3.1	Reference sensitivity requirements with PC2 on n66 without TxD

For CA_n25-n66, there is no cross-band isolation MSD for UL n66 with PC3 based on channel bandwidths up to 20 MHz. Re-examining the MSD based on up to 45 MHz UL channel BW on n66 with PC3 resulted in around 0.3 dB MSD and was decided it was not worth adding.

For n66 PC2 with 45 MHz UL channel BW, cross-band isolation MSD is considered based on simulation and measurement data. For single Tx PC2 n66 the following MSD is proposed:

Table 5.x.3.1-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 |  | n25 |  | 1757.5 | 45 |  | 15 |  | 240 (RBstart=2) |  | 1932.5 |  | 5 |  | 2.24 | >ACLR2 |

#### 5.x.3.2	Reference sensitivity requirements with PC2 on n66 with TxD

For dual Tx PC2 n66 the following MSD is proposed:

Table 5.x.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 3.85 | >ACLR2 |