3GPP TSG-RAN WG4 Meeting#111		R4-2407715

Fukuoka City, Fukuoka, Japan, May 20th -24th, 2024

Agenda item:	10.13.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on deployment scenarios and spectrum usage for ambient IoT

Document for:	Discussion

Introduction

In RAN4#110-bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

- Topic 2-1: Deployment scenario Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement: - RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2. Issue 2-1-2: deployment scenarios for D2T2 Option 1-1: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is only allowed outdoor. Option 1-2: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is indoor. Agreement: - For D2T2 co-existence evaluation, Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. - Consider option 1-1 and option 1-2 as the starting point

- Topic 2-2: Spectrum usage Issue 2-2-1: Spectrum usage for R2D in D1T1 Option 1: FDD DL spectrum for R2D Option 2: FDD UL spectrum for R2D Agreement: - FFS on whether to prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation. Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation - CW transmits in either UL or DL spectrum - FFS on inside topology and outside topology. Issue 2-2-4: Spectrum usage for R2D in D2T2 Agreement: - Use FDD UL spectrum for R2D in D2T2. Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering - Use UL spectrum as the starting point for co-existence evaluation. - It won’t preclude the use of DL for backscattering transmission. - FFS on the minimum distance between the intermediate UE and A-IoT device

Discussion

Deployment scenarios for D1T1

According to the way forward, it is reasonable to prioritize evaluate option 1-1 and option 1-2, taking into account that there is no 900MHz deployment indoors. However, legacy NR BSs are co-located with A-IoT reader and CW, all of NR and A-IoT BS/UE/Reader/Device/CW are indoors need to be further studied if the A-IoT device doesn’t only support 900MHz, but also supports 2GHz due to 2GHz with indoor deployment. Regarding whether to share hardware, it is understood that sharing hardware can save costs to a certain extent, and the interference is relatively serious. On the whole, sharing hardware is recommended to be given priority.

Proposal 1: option 2-1 needs to be prioritized for evaluation if the spectrum with indoor deployment.

Spectrum usage

Spectrum usage for R2D in D1T1

Figure1: D1T1-A1, R2D in DL spectrum                                                                                                Figure 2: D1T1-A2, R2D in DL spectrum

With regard to D1T1, R2D is a signal transmitted from the BS to the device. According to Section 5.2 in TS 38.101-1, the downlink operating band is BS transmit and UE receive. In order to meet the spectrum regulation, it is reasonable to use DL spectrum for R2D. At the same time, higher power can be transmitted by BS if R2D uses DL spectrum. In addition, BS could use existing hardware if R2D is in DL spectrum.

Proposal 2: prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation.

Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Figure1: D1T1-A1, CW in UL spectrum                                   Figure 2: D1T1-A2, CW in UL spectrum                                                            Figure3: D1T1-B, CW in UL spectrum

With regard to D1T1, CW2D is a signal transmitted from the BS to the device for the inside topology. In view of device 1 without the ability of large frequency, D2R backscattering is transmitted in the same carrier as CW for D2R backscattering. Similarly, the uplink operating band is BS receive and UE transmit. In order to meet the spectrum regulation, D2R should be in UL spectrum, but CW2D should be in DL spectrum for inside topology. It is obvious that this cannot be achieved. In general, co-existence simulation needs to simulate the worst case. There is more interference for CW in DL spectrum compared with CW in UL spectrum for inside topology. As to outside topology, it is could be achieved that CW2D in UL spectrum and D2R in UL spectrum to meet the spectrum regulation.

Proposal 3: case 1-1 and case 1-4 could be prioritized to evaluate co-existence.

Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

With regard to D2T2, CW2D is a signal transmitted from the intermediate UE to the device for the inside topology. In view of device 1 without the ability of large frequency, D2R backscattering is transmitted in the same carrier as CW for D2R backscattering. Similarly, the downlink operating band is UE receive and BS transmit. In order to meet the spectrum regulation, D2R should be in DL spectrum and CW2D in UL spectrum for outside topology.

Proposal 4: case 2-2 and case 2-3 could be prioritized to evaluate co-existence.

Conclusions

In this paper, we provide our views on deployment scenarios and spectrum usage, we have made the following proposals.

Proposal 1: option 2-1 needs to be prioritized for evaluation if the spectrum with indoor deployment.

Proposal 2: prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation.

Proposal 3: case 1-1 and case 1-4 could be prioritized to evaluate co-existence.

Proposal 4: case 2-2 and case 2-3 could be prioritized to evaluate co-existence.

Reference

- R4-2406714, Way forward on Ambient IoT in NR, CMCC. TSG-RAN4#110-bis meeting