3GPP TSG-RAN WG4 Meeting #111	R4-2407755

Fukuoka City, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	On MIMO_evo UE demodulation performance and CSI requirements

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

In RAN4#110bis agreements were made on test setup and simulation assumptions for TypeII Doppler, TypeII for CJT and Rel-18 DMRS with a few remaining open issues mainly on TypeII Doppler.

In the following, we will provide Nokia’s view on the remaining open issues as well as make observations and proposals where needed.

Discussion

Test setup and simulation assumptions for TypeII Doppler

Propagation channel for both FDD and TDD cases

In RAN110-bis the doppler of the propagation channel as still kept open, as it was not clear which doppler settings would be most optional based on the available simulation results (see [1]):

Issue 1-1-1: Propagation channel for both FDD and TDD cases For Rel18 Doppler codebook requirements, decide the propagation channel from below options based on the updated simulation results on the May meeting For FR1 FDD: - Option 1: TDLA30-30 - Option 2: TDLA30-20 For FR1 TDD: - Option 1: TDLA30-20 - Option 2: TDLA30-10

The selection of the doppler for the propagation channel should be made to achieve the highest gamma for the PMI requirements. With the available results for FDD for N4=1, the doppler with the best performance increase compared to random PMI is 20Hz (i.e. TDLA30-20). For TDD RAN4#110bis did not provide enough simulation results to determine the most optimal doppler.

Test metric of TypeII-Doppler-r18 codebook

In RAN110-bis the decision of the test metric for TypeII-Doppler-18 codebook was moved to RAN4#111 under the expectation that more simulation results would be available due to the agreed configuration setup in RAN4#110bis (see [1]):

Issue 1-1-4: Test metric of TypeII-Doppler-r18 codebook Decide the gamma value based on the updated simulation results on the May meeting

In RAN4#110bis there were not enough results provided for the agreed N4=1 configuration to enable requirement definition. Further alignment must be done based on new/updated simulation results provided for RAN4#111.

Timing mismatch between the prediction reference and precoder usage in PDSCH transmission

In RAN4#110bis it was brought up, that there is a timing mismatch between the prediction reference and precoder usage in PDSCH transmission (see [1]) with a request for interested companies to provide draft LS to RAN1 (and RAN2).

Issue 1-1-7: timing mismatch between the prediction reference and precoder usage in PDSCH transmission Interested companies are welcomed to provide draft LS to RAN1 and RAN2 on next meeting for this mismatch problem.

The relevant part of the specification related to above issue is as follows (see [2]):

typeII-Doppler-r18 SEQUENCE { n1-n2-codebookSubsetRestriction-r18 CBSR-r18, paramCombination-Doppler-r18 INTEGER (1..9), td-dd-config-r18 TD-DD-Config-r18, numberOfPMI-SubbandsPerCQI-Subband-r18 INTEGER(1..2), predictionDelay-r18 ENUMERATED {m0,n0,n1,n2 }, typeII-RI-Restriction-r18 BIT STRING (SIZE (4)) predictionDelay Prediction delay for Doppler and Doppler port selection codebooks see TS 38.214 [19], Clause 5.2.1.4. The first value m0 means that the first slot for which the CSI corresponds to is the slot where the CSI reference resource is located at. For the other three candidate values (n0, n1, n2), then the first slot for which the CSI corresponds to is given by l= n+delta, where delta can take on values of 0, 1, 2 and n the slot in which CSI is reported.

As was highlighted in RAN4#110bis, the delta values are limited to values of n0, n1 and n2. During the RAN4 discussions in RAN4#110bis, it was highlighted that there can easily be configurations where none of the current delta values will match the configuration, i.e. the highest delta value n2 is not enough to match the NW configuration used.

If the network can only indicate a delta of up to 2 (n2) and if the actual delta for the network configuration is higher, the UE will not be able to optimize the prediction for the used configuration, hence performance is likely not optimal. Based on this, we see it can possible be useful for the UE prediction and provide more flexibility from NW perspective to have additional (higher) delta values to select from.

We do not see it easy to modify the existing agreed values in the ENUM list however adding additional values to the list could be an option if it is ensured, the additional values do not increase the size of the IE “CodebookConfig-r18” significantly. Since the existing ENUM list already contains 4 values, an extension to 8-10 values could be considered.

The currently available delta values in the EMUM list of “typeII-Doppler-r18 -> predictionDelay-r18” does not always enable the NW to provide the UE with high enough delta values matching the used delay (i.e. higher than n2), hence the prediction might not be optimal to the actual configuration used by the NW.

Based on our observations, we suggest sending an LS to RAN1 for RAN1 to consider increasing the number of values for the “predictionDelay-r18” from 4 to 8-10, i.e. introduce predictionDelay-r18 to be ENUMERATED {m0, n0, n1, n2, n(x1), n(x2), …, n(x6)}.

RAN4 to send LS to RAN1 requesting RAN1 to consider increasing the number of values for the predictionDelay-r18. As a starting point 4-6 additional values can be suggested to be added, i.e. from ENUMERATED {m0, n0, n1, n2} to {m0, n0, n1, n2, n(x1), n(x2), …, n(x6)}.

A proposal for a LS has been provided in [4].

Test setup and simulation assumptions for TypeII for CJT

Test metric of TypeII-CJT-r18 codebook

Issue 1-2-7: Test metric of TypeII-CJT-r18 codebook Decide the gamma value based on the updated simulation results on the May meeting

Based on the provided simulation results from RAN4#110bis and Nokia’s updated simulation results, we see a gamma value of [1.8]dB to be reasonable for both 2Rx and 4Rx with MCS20 and Rank 2.

Based on the provided simulation results from RAN4#110bis a gamma value of [1.8]dB seems reasonable, however if updated/additional simulation results are provided in RAN4#111 the value must be reconsidered.

If updated simulation results are provided in RAN4#111, the gamma value must be decided based on the updated results. If no updated results are provided, set gamma to [1.8]dB for both 2Rx and 4Rx cases.

Conclusion

We have presented Nokia's view on the open issues with relation to the remaining open issues for MIMO Evo Demodulation performance.

We have the following observations and proposals:

Timing mismatch between the prediction reference and precoder usage in PDSCH transmission

- The currently available delta values in the EMUM list of “typeII-Doppler-r18 -> predictionDelay-r18” does not always enable the NW to provide the UE with high enough delta values matching the used delay (i.e. higher than n2), hence the prediction might not be optimal to the actual configuration used by the NW.

- RAN4 to send LS to RAN1 requesting RAN1 to consider increasing the number of values for the predictionDelay-r18. As a starting point 4-6 additional values can be suggested to be added, i.e. from ENUMERATED {m0, n0, n1, n2} to {m0, n0, n1, n2, n(x1), n(x2), …, n(x6)}.

Test metric of TypeII-CJT-r18 codebook

Based on the provided simulation results from RAN4#110bis a gamma value of [1.8]dB seems reasonable, however if updated/additional simulation results are provided in RAN4#111 the value must be reconsidered.

If updated simulation results are provided in RAN4#111, the gamma value must be decided based on the updated results. If no updated results are provided, set gamma to [1.8]dB for both 2Rx and 4Rx cases.

- References

- R4-2406111	Way Forward for [110bis][328] NR_MIMO_evo_DL_UL_demod

- TS38.331 	NR; Radio Resource Control (RRC); Protocol specification

- R4-2408521	On MIMO_evo UE demodulation performance and CSI requirements - sim

- R4-2409457	LS on delta paramater option extension for predicted PMI (Mediatek, Nokia)