3GPP TSG-RAN WG4 Meeting #111	R4-2407825

# Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	10.14.2.2

Source: 	Xiaomi

Title: 	Discussion on Rx requirements of REFSENS, ASCS and ACS for LP-WUS

Document for:	Discussion

Introduction

In last RAN4 #111 meeting, The Rx requirements were discussed and some agreements were approved in WF [1].

This contribution further discuss on the REFSENS, ASCS and ACS requirements for LP-WUR.

Discussion

In last RAN4 meeting, the performance metric for LP-WUS RF requirements was agreed using X% missed detection rate as the starting point, and whether to have false alarm rate is FFS.

Firstly, in my understanding, missed detection are different from false alarm. Missed detection rate (MDR) should be

MDR=1-AB

Where A is the number of correctly decoded bits and B is the number of transmitted bits. False alarm rate is the conditional total probability of erroneous detection when input is only noise and related to the setting of the demodulation threshold. Therefore, false alarm rate is a demodulation requirement. Therefore, whether to have false alarm rate should discuss in demodulation part not in FR part.

Proposal 1: False alarm rate is a demodulation requirement related to the setting of the demodulation threshold. Whether to have false alarm rate should discuss in demodulation part not in FR part.

Proposal 2: using 1% missed detection rate as the starting point in the LLS.

And the WF has agreed that for REFSENS:

- Reuse legacy approach to derive REFSENS, further discuss SNR, NF, IM

  - FFS whether to design REFSENS requirements or other requirements to ensure LP-WUR meet the coverage target

  - Side condition for REFSENS test: DL test signal will only have LP-WUS signal.

  - After concluding WUS design in RAN1, the SNR to specify REFSENS requirements should be decided in RAN4

  - Encourage companies input on NF analysis for different LP-WUR types

RAN4 need first discuss the possible NF to derive REFSENS based on different LP-WUR types.

In SI state, RAN4 agreed that will derive RF requirement based on NF which is feasible from coverage and implementation perspective

In SID, RAN1 research different NR for different architectures based on different waveform, and the related conclusion in TR 38.869 were excerpted below:

For OOK-1/2/4:

- RF envelope detection

  - The relative power consumption for ON state is in the range of 0.01~0.2.

  - The noise figure is in the range of 12~22 dB.

- Heterodyne architecture with IF envelope detection

  - The relative power consumption for ON state is in the range of 0.1~4.

  - The noise figure is in the range of 9~15 dB.

- Homodyne/zero-IF architecture with baseband envelope detection

  - The relative power consumption for ON state is in the range of 0.05~4.

  - The noise figure is in the range of 10~16 dB

For OFDM based

- Time-domain correlation (without FFT)

  - The relative power consumption for ON state is in the range of 0.15~30.

  - The noise figure is in the range of 7~25 dB.

- Frequency-domain correlation (with FFT)

  - The relative power consumption for ON state is in the range of 1~30.

  - The noise figure is in the range of 7~12 dB

In RAN1 #116bis meeting, RAN1 using follow assumption for NF to research the SNR:

- Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately.

Therefore, RAN4 can use similar method to define the NF for LP-WUS.

Proposal 3: RAN4 can use the same assumption that +2dB, +5dB, +8dB on top of NF of MR (9dB) for LR’s NF as the starting point to evalue the feasible from coverage and implementation perspective.

Regarding the ACS, ASCS and related guard RB, the WF in last meeting has agreed that ACS and ASCS reuqirements and related guard RB number can be evaluated by link level simulation:

For ASCS:

  - The same level PSD for LP-WUS and NR signals is assumed

  - RF impairments can be considered and aligned for ASCS simulation

For ACS:

  - The same interference level as for main radio is assumed for LP-WUR

For other simulation parameter will be discussed in this meeting using SI assumption as a starting point, and the RF impairments can first consider CFO as using in SI state, whether ADC considered for RF impairments can further discuss:

Table 2.1-1 Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700MHz/2.6GHz |
| gNB CBW | 20/100MHz |
| SCS of OFDM generator for NR signal | 30kHz |
| Channel Model | TDL-C 300 NLOS |
| LP-WUR Waveform | OOK(OOK1 or OOK4) with or without overlaid OFDM sequence(s) |
| LP-WUR Channel bandwidth | 11RB (30kHz SCS)) |
| LP-WUR Rx number | 1 Rx |
| WUS placement within NR channel | case 1: Center; |
| LP-WUR ASCS interferer | 5MHz |
| WUS duration | 28 symbols |
| NR power level | Same PSD with WUS signal |
| Filter characteristic | 5th order Butterworth |
| Filter passband BW | Same as WUS bandwidth (number of RBs) |
| Guard RB | On each side of LP-WUS bandwidth |
| RF impairments | CFO: 100ppm and 200 ppm |
| Performance metric | 1% missed detection rate |

Proposal 4: Using link level simulation assumptions in Table 2-1 as the starting point, whether ADC considered for RF impairments can further discuss.

Conclusion

In this contribution, we discussed the REFSENS, ASCS and ACS requirements for LP-WUR and proposed:

Proposal 1: False alarm rate is a demodulation requirement related to the setting of the demodulation threshold. Whether to have false alarm rate should discuss in demodulation part not in FR part.

Proposal 2: using 1% missed detection rate as the starting point in the LLS.

Proposal 3: RAN4 can use the same assumption that +2dB, +5dB, +8dB on top of NF of MR (9dB) for LR’s NF as the starting point to evalue the feasible from coverage and implementation perspective.

Proposal 4: Using link level simulation assumptions in Table 2-1 as the starting point, whether ADC considered for RF impairments can further discuss.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700MHz/2.6GHz/4GHz |
| gNB CBW | 20/100MHz |
| SCS of OFDM generator for NR signal | 30kHz |
| Channel Model | TDL-C 300 NLOS |
| LP-WUR Waveform | OOK(OOK1 or OOK4) with or without overlaid OFDM sequence(s) |
| LP-WUR Channel bandwidth | 11RB (30kHz SCS)) |
| LP-WUR Rx number | 1 Rx |
| WUS placement within NR channel | case 1: Center; |
| LP-WUR ASCS interferer | 5MHz |
| WUS duration | 28 symbols |
| NR power level | Same PSD with WUS signal |
| Filter characteristic | 5th order Butterworth |
| Filter passband BW | Same as WUS bandwidth (number of RBs) |
| Guard RB | On each side of LP-WUS bandwidth |
| RF impairments | CFO: 100ppm and 200 ppm |
| Performance metric | 1% missed detection rate |

Reference

[1] R4-2406619, WF on NR LP-WUS UE requirements, vivo, RAN4 #110bis.