3GPP TSG-RAN WG4 Meeting #111				                                                                                     R4-2407836

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussion on testability and interoperability for AI positioning

Source: 	Xiaomi

Agenda item:	10.11.3

Document for:	Discussion

Introduction

In Rel-18 RAN4 studied the testability and interoperability of NR AI/ML and the conclusions are included in TR 38.843 [1]. A new WI in Rel19 on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting [2] with the following RAN4 objectives :

- Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. - Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

And in last RAN4 meeting the discussion on this objective were initialized and WF was approved in [3].  In this contribution we will provide some considerations on the open issues under AI positioning scenario.

- Measurement requirement for the different AI Pos cases

- Measurements for model input

- AI Model monitoring hypothesis

Discussion

Requirements for the different AI Pos cases

Firstly, regarding to RAN1’s agreements on the use cases for AI Pos below, RAN4 can discuss whether the measurement testing requirements shall be defined for these using cases.

- Case 1: 	      UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For case 1, and case 3a/3b, in the previous meeting RAN4 agreed that [3][4]:

R4#110b discussion Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced R4#110 discussion Agreement: RAN4 will not define positioning accuracy requirements for case 3a/3b

However, as highlighted below in AI study item[1,TR38.843] it was agreed that UE’s monitoring can fall back to the non-AI models. In such case, the legacy positioning requirements shall be applied for UE or NW device in Rel19 positioning also.  Otherwise the inconsistence behaviour will be observed.

Type2 performance monitoring (UE-side performance monitoring): - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - If it is for UE-side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Indication from NW to UE to do LCM operation - UE reporting of beam measurement(s) based on a set of beams indicated by gNB - Signalling, e.g., RRC-based, L1-based - Note: Performance and UE complexity, power consumption should be considered - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

Observation 1: If no requirements defined for AI Pos cases, it would lead some inconsistent behaviour and requirements on PRS measurements when AI based positioning fallback to non-AI mode.

Especially, in Rel16 with NG-RAN node assisted positioning, the accuracy requirements can be defined as in TS38.133 Clause 13[5].  At same time the UE node assisted positioning, the accuracy requirements can also be defined as in TS38.133 Clause 10.

Therefore, we can propose that:

Proposal 1: In order to guarantee the consistent UE behaviour and requirements, RAN4 should define accuracy requirements for AI Pos case 1 and case 3a/3b.

Measurements for model input

In RAN1 using sample-based or path-based measurements as the model inputs is still under discussion.

RAN1#116 Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

From RAN4 perspective, the legacy requirements defined in Rel16/17 are dependent on path-based measurements. For an example, PRS RSRPP accuracy below in existing spec [5] is based on the measurement reporting of first path and additional paths.

10.1.38.2.1 Absolute PRS RSRPP accuracy The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.38.2.1-3 and Table 10.1.38.2.1-4 apply for the UE supporting supportedDL-PRS-ProcessingSamples [34]. Note: The requirements in this clause are derived based on two-tap channel defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). Note: The requirements in this clause are derived based on the difference between the estimated PRS-RSRPP compared to the ideal PRS-RSRPP defined as RSRPPp∝kHkexpj2πDpkNIFFT2 Where: Hk is the effective channel frequency response (over REs occupied by PRS) measured without receiver noise. Dp is the exact delay of the p-th path in the channel model.

Observation 2: The legacy PRS measurement requirements up to Rel18[5] are based on the path based measurement.

That is if the sample-based measurement as model input, from RAN4 perspective the existing requirements for Pos shall be revisited. For an instance, the margin due to quantization error maybe needed not since the AI training input can be assumed as error free.

Proposal 2: If RAN1 using sample-based measurement as model input for AI Pos, the existing accuracy requirements (e.g. PRS RSRPP) shall be restudied at least.

Model monitoring

In RAN4#110, how to handle the post deployment was discussed with the following agreements [4]

Issue 1-2: Post deployment handling Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded

In principle we support Option 2 since the dynamically monitoring AI performance can guarantee UE positioning performance.  Option 1 only is not sufficient for AI Pos because the wireless realistic deployment scenarios must be quite diverse with the scenarios for model training. It is impossible to rely on the initial testing before the deployment only. But on the other hand, Option 1 is redundant if the AI monitoring procedure needed during AI positioning procedure.

Therefore, we can propose that:
Proposal 3: AI performance monitoring can be used for the post deployment verification directly.

At same time, we noted that RAN1’s triggered batch of discussion on AI monitoring for positioning. One of important issues for AI monitoring is the necessary performance hypothesis which needs RAN4’s investigation.

R1#116bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation

For all AI positioning cases, no matter whether the target UE or LMF performing monitoring metric calculation (Option A or Option B), the evaluation on the validity of current AI models is dependent with AI pos performance threshold. For an instance, if the output error based the known data with label is larger than a specific value (e.g. 10%) the monitoring entity could trigger one report of performance outrage. Particularly, such works related to performance could be studied and decided by RAN4.

Observation 3: For AI model performance monitoring, the proper threshold used to justify AI model validity can be studied by RAN4.

For Case 1, if the monitoring performed by UE, RAN4 can further discuss the acceptable evaluation period to avoid the long time performance outrage because of invalid AI modelling. And such requirements can be also defined in TS38.133.

Observation 4: For Case 1 if AI model performance monitoring handled by UE, the requirements on model evaluation period can be defined in RAN4.

Therefore, we can propose that:

Proposal 4: RAN4 shall study the following necessary requirements for AI pos model monitoring. The specification impacts on TS38.133 or other 3GPP spec can be FFS also.

- Monitoring hypothesis and threshold

- Performance evaluation period

Conclusion

This contribution provided some initial analysis on the testability for AI positioning in Rel19

Observation 1: If no requirements defined for AI Pos cases, it would lead some inconsistent behaviour and requirements on PRS measurements when AI based positioning fallback to non-AI mode.

Proposal 1: In order to guarantee the consistent UE behaviour and requirements, RAN4 should define accuracy requirements for AI Pos case 1 and case 3a/3b.

Observation 2: The legacy PRS measurement requirements up to Rel18[5] are based on the path based measurement.

Proposal 2: If RAN1 using sample-based measurement as model input for AI Pos, the existing accuracy requirements (e.g. PRS RSRPP) shall be restudied at least.

Proposal 3: AI performance monitoring can be used for the post deployment verification directly.

Observation 3: For AI model performance monitoring, the proper threshold used to justify AI model validity can be studied by RAN4.

Observation 4: For Case 1 if AI model performance monitoring handled by UE, the requirements on model evaluation period can be defined in RAN4.

Proposal 4: RAN4 shall study the following necessary requirements for AI pos model monitoring. The specification impacts on TS38.133 or other 3GPP spec can be FFS also.

- Monitoring hypothesis and threshold

- Performance evaluation period

References

- 3GPP TR 38.843 Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface

- RP-221348 SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 2022

- R4-2406617

- R4-2403712

- 3GPP TS 38.133 v18.5.0