3GPP TSG-RAN WG4 Meeting #111														 R4-2407864

Fukuoka City, Japan, 20th – 24th May, 2024

Agenda item:	7.14.1.1

Source:	OPPO

Title:	Discussion on L1/L2 based inter-cell mobility

Document for:	Discussion

- Introduction

Based on the approved WF in last meeting [1], there are still some remaining issues. In this contribution, we provide further discussion on some open issues.

- Discussion

Issue 2-1-1: L1-RSRP measurement on intra-f neighbor cell of deactivated SCC <Way Forward >: Further discuss the following options: - Option 1 (Apple, MTK): LTM intra-frequency L1-RSRP requirements defined in R18 are not applicable to neighbor cell on deactivated SCC. - Option 1a (Apple): not define requirements in R18 - Option 1b (Apple): allow similar measurement relaxation as L3 measurement (e.g. following measCycleSCell) - Option 2 (Ericsson, QC): If network configures cell on deactivate SCell frequency as a LTM candidate cell, UE should measure that cell using LTM L1-RSRP measurement period.

In our view, any cell configured in candidate LTM cell list is equal to be measured, expect that any priority rule is defined. Even though legacy L3 RRM measurement on deactivated SCell is relaxed with measCycleSCell configured by NW, no such relaxation scheme is introduced for LTM L1-RSRP measurement in R18. Thus, we prefer option 2 to take deactivated SCell equally to other candidate LTM cell and measure using LTM L1-RSRP measurement period.

Proposal 1: Prefer option 2 to take deactivated SCell equally to other candidate LTM cell and measure using LTM L1-RSRP measurement period in R18.

Issue 2-2-2: Measurement period of serving cell L1-RSRP measurement <Way Forward >: Further discuss the following option: - introduce NLayer in serving cell L1 RSRP measurement requirement and clarify that it is for UE capable of RTD>CP configured with L1 RSRP measurement on neighbour cell.

Generally, we agree with the principle that the assumption of L1 RSRP measurements on Serving cell and intra-frequency neighbour cell should be kept aligned. But it’s quite hard to understand why we agreed to use NLayer  to scale the measurement delay for intra-frequency layers of LTM based L1-RSPP measurement. If possible, we would like to keep current L1 RSRP measurement requirement as it is, and consider to revisit the agreements for L1 RSRP measurement on neighbour cell.

Compared to intra-frequency L3 measurements without gaps, the measurement delay is scaled by CSSFintra, where zthe measurement occasions on PCC and SCC are not equally shared.

Observation 1: The principle to define the delay requirements of L1 RSRP measurements on Serving cell and intra-frequency neighbour cell should be kept aligned.

Proposal 2: Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement.

- Conclusion

In this contribution, we provide our observations and proposals as follows:

Proposal 1: Prefer option 2 to take deactivated SCell equally to other candidate LTM cell and measure using LTM L1-RSRP measurement period in R18.

Observation 1: The principle to define the delay requirements of L1 RSRP measurements on Serving cell and intra-frequency neighbour cell should be kept aligned.

Proposal 2: Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement.

- References

- R4-2406502, WF on NR mobility enhancements (part 1), MediaTek Inc.