3GPP TSG-RAN WG4 Meeting # 111		R4-2407936

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda Item:	8.3.2

Source: 	CMCC

Title: 			 (IoT_NTN_enh-Perf) Discussion on RRM core maintenance for IOT NTN enhancement

Document for:	Discussion

- Introduction

In RAN4#110bis meeting, some leftover maintenance issues are discussed, however, no consensus achieved. The WF has been approved in [1].

In this contribution, we provide our views on part of maintenance issues.

- Discussion

Issue 1-1-1: Skipping serving cell measurement before t-service

<WayForward>

Further discuss

- Option 1: Proposal 1 with wording refinement.

  - Proposal 1 (Nokia): When a UE starts intra-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), the UE skips the serving cell measurements when the following side conditions are met:

    - a. When skipping serving cell measurements reduces the value of Ksatellite (i.e. no neighbor cell is configured for measurements in the same satellite as the serving cell)

    - b. The S-Criterion is still met by the serving cell, according to the most recent measurements on the serving cell.

    - c. If those conditions are met, the UE is allowed to use the last value for the serving cell measurements for cell reselection purpose until t-service is reached.

- Option 2: no change is needed.

Issue 1-1-2: Measurement on inter-frequency neighbor cells associated to the same satellite

- No agreement

Based on our understanding of the intention of the positive proposals under issue 1-1-1 and 1-1-2, since the serving satellite will be out of service after t-service, then for the short time period before t-service, it seems meaningless to keep monitoring the cells on the serving satellite, UE will not re-select to these cells anyway after t-service.

However, if UE directly stop to monitor the serving cell and use the last value of the serving cell measurement for ranking and so on, it may cause the situation that the serving cell is always the best cell, cell re-selection will not be triggered, although the serving cell quality is not good anymore in fact. The mobility will be impacted.

In order to avoid the aforementioned drawback, we propose to introduce the definition of  ‘expected serving time’ for cell re-selection. For one cell, ‘expected serving time’ is the time period between the current time to the ‘t-service’ for this cell.

Specifically, the UE shall perform ranking of all cells that fulfil the cell selection criterion S firstly, and then UE perform cell re-selection to the cell with the longest ‘expected serving time’ among the cells whose R value is within XdB of the R value of the highest ranked cell. The XdB can be configured by network.

By such way, when the time is near to t-service and UE start to measure the neighbour cells, UE can re-select to a new cell with both good quality and long serving time, even if the serving cell is still good at current time. It decreases the cell re-selection delay around t-service, and avoids frequent cell-reselection actions since the target re-selection cell has the longest expected serving time. Meanwhile, the UE power saving gain can also be reached, after UE re-selection to the target cell, the cell-reselection measurement will be suspended, since the new serving cell has good quality, for example, Srxlev > SIntraSearchP.

Proposal 1: For earth-fixed scenario, introduce the ‘expected serving time’ as an evaluation factor for cell re-selection. For one cell, ‘expected serving time’ is the time period between the current time to the ‘t-service’ for this cell. Specifically:

- UE shall perform ranking of all cells that fulfil the cell selection criterion S

- UE perform cell re-selection to the cell with the longest ‘expected serving time’ among the cells whose R value is within XdB of the R value of the highest ranked cell.

  - The XdB can be configured by network.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For earth-fixed scenario, introduce the ‘expected serving time’ as an evaluation factor for cell re-selection. For one cell, ‘expected serving time’ is the time period between the current time to the ‘t-service’ for this cell. Specifically:

- UE shall perform ranking of all cells that fulfil the cell selection criterion S

- UE perform cell re-selection to the cell with the longest ‘expected serving time’ among the cells whose R value is within XdB of the R value of the highest ranked cell.

  - The XdB can be configured by network.

- Reference

- R4-2406314, WF on R18 IoT NTN RRM requirements, MediaTek inc.