3GPP TSG-RAN4 Meeting #111	R4-2407939

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_enh-Perf) draftCR to TS 36.133 Introduce of cell re-selection test cases for Cat-M1 UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Based on the endorsed draftCR R4-2406316, the applicability rule of test configuration should be further added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Further supplement the applicability rule of test configuration based on the endorsed draftCR R4-2406316. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.1.1.5 - A.14.1.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draft CR is based on the endorsed draftCR R4-2406316 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage

A.14.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD inter frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.3.

The supported test configurations are provided in Table A.14.1.1.5.1-1.

Table A.14.1.1.5.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

The test scenario comprises of 2 E-UTRA FDD cells on 2 different carriers as given in tables A.14.1.1.5.1-2 and A.14.1.1.5.1-3. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.14.1.1.5.1-2: General test parameters for FDD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell2 | UE shall be forced to cell 2 in the initialisation phase, so that reselection to cell 1 occurs during the first T1 phase |
| T1 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cell |  | Cell2 |  |
| Final condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| PRACH configuration |  |  | PRACH_2CE | See table in A.3.16 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 15 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 75 | T3 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.5.1-3: Cell specific test parameters for FDD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| Satellite information |  | 1 | NSC.1 | SSC.1 | SSC.1 |  | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | NSC.2 | SSC.2 | SSC.2 |  | SSC.2 | NSC.2 | NSC.2 |
| E-UTRA RF Channel number |  | 1, 2 | 1 |  |  |  | 2 |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  | 1.4 |  |  |
| OCNG Patterns defined in A.3.2.1.6 |  | 1, 2 | OP.7 FDD |  |  |  | OP.7 FDD |  |  |
| PBCH_RA | dB | 1, 2 | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| MPDCCH_RA | dB |  | 1, 2 |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 |  |  |  | -140 |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |  |
|  | dB | 1, 2 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
|  | dB | 1, 2 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
| RSRP Note 3 | dBm/15 KHz | 1, 2 | -84 | -84 | -84 |  | -102 | -infinity | -86 |
| TreselectionEUTRAN | s | 1, 2 | 0 |  |  |  | 0 |  |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2 | 48 |  |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  |  | 1x1 |  |  |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

A.14.1.1.5.2	Test Requirements

The cell reselection delay to higher priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to higher priority shall be less than 68 s.

The cell reselection delay to lower priority is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to lower priority shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC , and to lower priority cell can be expressed as: Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC,

Where:

Thigher_priority_search	See clause 4.7A.2.1.3

Tevaluate, E-UTRAN_Inter_NC	See Table 4.7A.2.1.3-1 in clause 4.7A.2.1.3

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 67.68 s for higher priority cell search and 7.68 s for lower priority cell search, allow 68 s for higher priority cell and 8 s for lower priority cell in the test case.

A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage

A.14.1.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.3.

The supported test configurations are provided in Table A.14.1.1.6.1-1.

Table A.14.1.1.6.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

The test scenario comprises of 2 E-UTRA carriers and 2 cells as given in tables A.14.1.1.6.1-2 and A.14.1.1.6.1-3. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.14.1.1.6.1-2: General test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell2 | UE shall be forced to cell 2 in the initialisation phase, so that reselection to cell 1 occurs during the first T1 phase |
| T1 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cell |  | Cell2 |  |
| Final condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| PRACH configuration |  |  | PRACH_2CE | See table in A.3.16 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 15 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 75 | T3 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.6.1-3: Cell specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | 1 | NSC.1 | SSC.1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | NSC.2 | SSC.2 | SSC.2 | SSC.2 | NSC.2 | NSC.2 |
| E-UTRA RF Channel number |  | 1, 2 | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  | 1.4 |  |  |
| OCNG Patterns defined in A.3.2.1.6 |  | 1, 2 | OP.7 FDD |  |  | OP.7 FDD |  |  |
| PBCH_RA | dB | 1, 2 | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  | 1, 2 |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 |  |  | -140 |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |
|  | dB | 1, 2 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  | dB | 1, 2 | 14 | 14 | 14 | -4 | -infinity | 12 |
| RSRP Note 3 | dBm/15 KHz | 1, 2 | -84 | -84 | -84 | -102 | -infinity | -86 |
| TreselectionEUTRAN | s | 1, 2 | 0 |  |  | 0 |  |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.14.1.1.6.2	Test Requirements

The cell reselection delay to higher priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to higher priority shall be less than 68 s.

The cell reselection delay to lower priority is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to lower priority shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC , and to lower priority cell can be expressed as: Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC,

Where:

Thigher_priority_search	See clause 4.7A.2.1.3

Tevaluate, E-UTRAN_Inter_NC	See Table 4.7A.2.1.3-1 in clause 4.7A.2.1.3

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 67.68 s for higher priority cell search and 7.68 s for lower priority cell search, allow 68 s for higher priority cell and 8 s for lower priority cell in the test case.

A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering

A.14.1.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD intra frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.2.

The supported test configurations are provided in Table A.14.1.1.7.1-1.

Table A.14.1.1.7.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

The test scenario comprises of 1 E-UTRA FDD carrier and 2 cells as given in tables A.14.1.1.7.1-2 and A.14.1.1.7.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Cell 1 is already identified by the UE prior to the start of the test, i.e. Cell 2 is not identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

T-Service broadcasted in SIB3 of Cell 1 is set to the time point that is 36s after start of T2.

Table A.14.1.1.7.1-2: General test parameters for FDD intra frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one FDD carrier frequency is used. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Configuration |  |  | PRACH_2CE | Refer to A.3.16 |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.7.1-3: Cell specific test parameters for FDD intra frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |  |
| Satellite information |  |  |  | 1 |  | SSC.1 |  | SSC.1 |  | NSC.1 |  | NSC.1 |  |
|  |  |  |  | 2 |  | SSC.2 |  | SSC.2 |  | NSC.1 |  | NSC.2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | RSRP |  | RSRP |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | 16 | 12 | -infinity | 16 |
|  | dB | 1, 2 | 16 | -4.11 | -infinity | 3.73 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -82 | -86 | -infinity | -82 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |
| Sintrasearch | dB | 1, 2 | Not sent |  | Not sent |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.14.1.1.7.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,EUTRAN_Intra_NC + TSI-EUTRA-M1-NC.

Where:

Tdetect,EUTRAN_Intra_NC	See Table 4.7A.2.1.2-1 in clause 4.7A.2.1.2

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB3 is scheduled with 20ms period, SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell in the test case.

A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering

A.14.1.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD intra frequency cell reselection requirements for Cat-M1 UE in enhanced coverage for satellite access specified in clause 4.7A.2.2.2.

The supported test configurations are provided in Table A.14.1.1.8.1-1.

Table A.14.1.1.8.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

The test scenario comprises of 1 E-UTRA carrier and 2 cells as given in tables A.14.1.1.8.1-2 and A.14.1.1.8.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Cell 1 is already identified by the UE prior to the start of the test, i.e. Cell 2 is not identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

T-Service broadcasted in SIB3 of Cell 1 is set to the time point that is 36s after start of T2.

TableA.14.1.1.8.1-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one carrier frequency is used. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Parameters |  |  | PRACH_4CE | Refer to A.3.16 |
| DRX cycle length |  | s | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | ≤340 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.8.1-3: Cell specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | SSC.2 | SSC.2 | NSC.1 | NSC.2 |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 1, 2 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | RSRP |  | RSRP |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | -7 | -12 | -infinity | -7 |
|  | dB | 1, 2 | -7 | -12.79 | -infinity | -7.27 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -105 | -110 | -infinity | -105 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |
| Sintrasearch | dB | 1, 2 | Not sent |  | Not sent |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.14.1.1.8.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 338 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,EUTRAN_Intra_EC + TSI-EUTRA-M1-EC.

Where:

Tdetect,EUTRAN_Intra_EC	See Table 4.7A.2.2.2-1 in clause 4.7A.2.2.2.

TSI-EUTRA-M1-EC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 6400 ms is assumed in this test case provided that SIB3 is scheduled with 20ms period, SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 337.36 s, allow 338 s for the cell re-selection delay to a newly detectable cell in the test case.

A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering

A.14.1.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD inter frequency cell reselection requirements for category M1 UE in enhanced coverage for satellite access specified in clause 4.7A.2.2.3.

The supported test configurations are provided in Table A.14.1.1.9.1-1.

Table A.14.1.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

The test scenario comprises of 2 E-UTRA FDD cells on 2 different carriers as given in tables A.14.1.1.9.1-2 and A.14.1.1.9.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Cell 1 is already identified by the UE prior to the start of the test, i.e. Cell 2 is not identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the referencelocation broadcasted in SIB31 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

Table A.14.1.1.9.1-2: General test parameters for FD-FDD inter frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Parameters |  |  | PRACH_4CE | Refer to A.3.16 |
| DRX cycle length |  | s | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | ≤340 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.9.1-3: Cell specific test parameters for FD-FDD inter frequency cell reselection test case for Cat-M1 UE in enhanced coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | SSC.2 | SSC.2 | NSC.1 | NSC.2 |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | 2 |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | -5 | -12 | -infinity | -5 |
|  | dB | 1, 2 | -5 | -12 | -infinity | -5 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -103 | -110 | -infinity | -103 |
| TreselectionEUTRAN | s | 1, 2 | 0 | 0 | 0 | 0 |
| Snonintrasearch | dB | 1, 2 | 50 |  | Not sent |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.14.1.1.9.2	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 337 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,EUTRAN_Inter_EC + TSI-EUTRA-M1-EC.

Where:

Tdetect,EUTRAN_Inter_EC	See Table 4.7A.2.2.3-1 in clause 4.7A.2.2.3

TSI-EUTRA-M1-EC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 6400 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 336.64 s, allow 337 s for the cell re-selection delay to a newly detectable cell in the test case.

A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering

A.14.1.1.10.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency cell reselection requirements for category M1 UE in normal coverage for satellite access specified in clause 4.7A.2.1.3.

The supported test configurations are provided in Table A.14.1.1.10.1-1.

Table A.14.1.1.10.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FDD-FDD duplex mode |
| 2 | NGSO, FDD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

The test scenario comprises of 2 E-UTRA carriers and 2 cells as given in tables Table A.14.1.1.10.1-2 and A.14.1.1.10.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of lower priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the referencelocation broadcasted in SIB31 of Cell 2 is exceeded by the configured value in distanceThresh plus 50m.

Table A.14.1.1.10.1-2: General test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two FDD carrier frequencies are used. |
| PRACH configuration |  |  | PRACH_2CE | See table in A.3.16 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 |  |
| T2 |  | s | 15 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.1.10.1-3: Cell specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-M1 UE in normal coverage

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1 | SSC.1 | SSC.1 | NSC.1 | NSC.1 |
|  |  | 2 | SSC.2 | SSC.2 | NSC.1 | NSC.2 |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | 2 |  |
| BWchannel | MHz | 1, 2 | 1.4 |  |  |  |
| OCNG Patterns |  | 1, 2 | OP.7 FDD |  | OP.7 FDD |  |
| PBCH_RA | dB | 1, 2 | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | 1, 2 | -140 | -140 | -140 | -140 |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | 14 | -4 | -4 | 14 |
|  | dB | 1, 2 | 14 | -4 | -4 | 14 |
| RSRP Note3 | dBm/15 kHz | 1, 2 | -84 | -102 | -102 | -84 |
| TreselectionEUTRAN | s | 1, 2 | 0 | 0 | 0 | 0 |
| Snonintrasearch | dB | 1, 2 | 50 |  | Not sent |  |
| Threshx, high | dB | 1, 2 | 48 |  | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  | 44 |  |
| Threshx, low | dB |  | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1, 2 | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | ms | 1, 2 | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.14.1.1.10.2	Test Requirements

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to lower priority shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: Tevaluate, E-UTRAN_Inter_NC + TSI-EUTRA-M1-NC.

Where:

Tevaluate, E-UTRAN_Inter_NC	See Table 4.7A.2.1.3-1 in clause 4.7A.2.1.3

TSI-EUTRA-M1-NC	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB31 and SIB33 are scheduled with 80 ms period.

This gives a total of 7.68 s for lower priority cell search, allow 8 s for lower priority cell in the test case.

<End of change>