3GPP TSG-RAN WG4 Meeting # 111	R4-2407966

# Fukuoka, Japan, 20 – 24 May, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for LP-WUS/WUR

Agenda item:	10.14.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues of RRM requirements for NR LP-WUS/WUR based on listed issues in approved WF [1].

Discussion

General aspects

Core requirements to be specified for LP-WUR measurement

RAN4 agreed to define measurement requirements for two type of signals such as LP-SS and existing PSS/SSS for LP-WUR.

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS

For LP-SS, RAN1 is still discussing LP-SS sequences and PRB size depending on SCS, so RAN4 needs to wait for conclusion of RAN1 discussion to evaluate for LP-SS based measurement requirements in RAN4. For exsiting PSS/SSS based measurement requirements, it is unclear the LP-WUR architecture, so noise figure and other RF impairment should be discussed.

- Proposal 1: RAN4 needs to wait for conclusion of LP-SS design from RAN1 and noise figure/other RF impairements for LP-WUR from RF session to evaluation measurement requirements for LP-WUR.

Core requirements to be specified for MR RRM relaxation

Based on companies’ view in the last meeting, MR RRM measurement relaxation can be considered for serving cell and/or neighbour cells. One thing to clarify is that MR based RRM measurement relaxation for serving cell and/or neigbhor cells should be considered for UE supporting LP-WUR.

- Proposal 2: Further MR based RRM measurement relaxation for serving cell and/or neighbour cells should be applied for UE supporting LP-WUR.

Criteria (entry/exit conditions) for LP-WUR measurement

For entry/exit condition for LP-WUS monitoring, in SI phase, RAN2 has discussed and concluded the condition(s) could be at least based on the serving cell measurement using LR and/or MR. However, further discussion for the conditions is also needed from a RAN4 perspective.

- Proposal 3: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective

Criteria (entry/exit conditions) for MR RRM measurement relaxation

The discussion and decision for the criteria of entry/exit conditions for MR RRM measurement relaxation are RAN2 work. However, RAN4 can provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation.

- Proposal 4: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation

LP-WUR requirements at RRC_IDLE/INACTIVE state

Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode

As mentioned in above, RAN1 is still discussing LP-SS design such as sequences and PRB size depending on SCS, so RAN4 needs to wait for further conclusion from RAN1 discussion. RAN4 agreed to define both LP-SS and existing PSS/SSS based measurement requirements, and since SNR/SINR target based on outcome of RAN1’s study is used as the starting point for RAN4 evaluation as below, RAN4 can discuss and evaluate exsiting PSS/SSS based LP-WUR measurement requirements.

Agreement on the target SNR for RAN4 simulation: - The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study. - Aspects not considered in RAN1 (if any) can be further discussed in RAN4.

- Proposal 5: RAN4 needs to wait further conclusion of LP-SS design from RAN1

Measurement accuracy requirements

In the last meeting, RAN4 discussed whther/how to define the accuracy requirements for LP-WUR in Idle/Inactive state and some agreements and information were made below.

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. For information only: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded.

Basically, in Idle/Inactive state, there is no measurement reporting, so core requirement for Idle/Inactive state is measurement delay based on the number of samples to meet measurement accuracy at certain side condition. Therefore, no dedicated accuracy requirement is needed.

- Proposal 6: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section

MR RRM relaxation

MR RRM relaxation factor for serving cell/neighbour cell

As mentioned above section 2.1, MR based measurement relaxation for serving cell and/or nighbor cells should be considered for UE supporting LP-WUR. Detailed criterion for measurement relaxation would be discussed in RAN2, but RAN4 can discuss relaxation factor for measurements mentioned as Proposal 4 above.

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS

Entry/exit condition for LP-WUS monitoring

As mentioned above section 2.1, basic entry/exit condition for LP-WUS monitoring is at least based on the serving cell measurement using LR and/or MR, and RAN2 is the main working group that determines the condition. If Entry/exit condition for LP-WUS monitoring means including measurement offloading from MR to LP-WUR, entry/exit condition should take into account UE mobility (high/low speed) in addition to serving cell RRM measurement since neighbour cell measurement should be performed based on MR for high mobility UE. if there are other conditions that should be considered, RAN4 needs to provide input to RAN2.

- Proposal 7: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2.

LP-WUR CONNECTED mode

LP-WUR at CONNECTED mode

Based on the objectives for LP-WUS/WUR, RAN4 needs to clarify the overall scope of RRM core requirements. LP-WUS design commonly applicable to both IDLE/INCATIVE and CONNECTED modes, but RAN4 needs to only focus on IDLE/INACTIVE mode related core requirements.

- Proposal 8: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

- Proposal 1: RAN4 needs to wait for conclusion of LP-SS design from RAN1 and noise figure/other RF impairements for LP-WUR from RF session to evaluation measurement requirements for LP-WUR.

- Proposal 2: Further MR based RRM measurement relaxation for serving cell and/or neighbour cells should be applied for UE supporting LP-WUR.

- Proposal 3: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective

- Proposal 4: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation

- Proposal 5: RAN4 needs to wait further conclusion of LP-SS design from RAN1

- Proposal 6: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section

- Proposal 7: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2.

- Proposal 8: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19

Reference

[1] R4-2406367, “WF for RRM requirements for LP-WUSWUR,” Moderator (vivo)