3GPP TSG-RAN WG4 Meeting#111		R4-2407992

Fukuoka City, Fukuoka, Japan, May 20th -24th, 2024

Agenda item:	10.1.1.4.1

Source: 	Spreadtrum Communications

Title: 	Discussion on reference sensitivity requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#110-bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for specifying ΔRIB,6R value

Way forward: The following proposals can be endorsed. - Proposal 1: Define one delta value for all CBWs for each band. - Proposal 2: Consider different requirements for different bands. Further discuss the following proposals: - Proposal 3: Study whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case. - Proposal 4: RF isolation at HB and UHB frequencies and antenna placement should be well studied and understood because of their impact on REFSENS for 6Rx handheld UEs. - Proposal 5: Existing ΔRIB requirements for 4Rx and 8Rx can be used as a starting point to study the performance of ΔRIB for 6Rx considering the form factor of handheld and FWA UE and other impact factors.

As we all known, the antenna length is inversely proportional to the frequency that is length= c/2f. According to the practical experience, HB and UHB frequency use different antennas, but n104 use the same antennas with n77/n78/n79.In our understanding, n104 should be included in the high band (n77, n78, n79) category for 6RX case.

Proposal 1: n104 should be included in the high band (n77, n78, n79) category for 6RX case.

ΔRIB,6R value for handheld UE and FWA

Way Forward: Further discuss if the same value is used for handheld UE and FWA.

Handheld UE has more severe impact of coupling and antenna correlation which can be compared with form factor for FWA due to limited size. Unless it is a folding smart phone, the implantation of 6RX is a challenge for handheld UE. Theoretically, we should distinguish handheld UE and FWA for ΔRIB,6R value.

Proposal 2: Different values are used for handheld UE and FWA.

In addition, the number of antenna for 6RX is three times that of 2RX, thus the diversity gain of 6RX is 10log3=4.77 dB higher than for 2RX. In theory, the diversity gain of 6RX is 10log1.5=1.76dB for 4RX and the diversity gain of 8RX is 10log1.33=1.25dB for 6RX. But practically, we should consider the impact of coupling and antenna correlation, which will be more severe than 4RX but less severe than 8RX. Based on the current specification in TS 38.101-1 table 7.3.2-2 and Table 7.3.2-2a, in our understanding, ΔRIB,6R  of FWA/CPE/vehicle/industrial devices is -3.2dB for n77/n78/n79/n104.But n41 has lower frequencies than n77/n78/n79, we could separately consider this question. ΔRIB,6R is -3.6dB for n41. Thus we think that handheld UE could be lower 0.3dB compared with CPE/FWA/vehicle/industrial devices. In view of the above, we could take the table 1 into consideration.

Proposal 3: Adopt the value of ΔRIB,6R  in Table1 for n41/n77/n78/n79/n104 for handheld UE and FWA.

Table1: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.61,-3.32 |
| n77, n78, n79,n104 | -3.21,-3.02 |
| NOTE 1: When 6 Rx operation is supported by FWA form factor. NOTE 2: When 6 Rx operation is supported by handheld UE. |  |

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: n104 should be included in the high band (n77, n78, n79) category for 6RX case.

Proposal 2: Different values are used for handheld UE and FWA.

Proposal 3: Adopt the value of ΔRIB,6R  in Table1 for n41/n77/n78/n79/n104 for handheld UE and FWA.

Table1: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.61,-3.32 |
| n77, n78, n79,n104 | -3.21,-3.02 |
| NOTE 1: When 6 Rx operation is supported by FWA form factor. NOTE 2: When 6 Rx operation is supported by handheld UE. |  |

- Reference

- R4-2406585, WF on requirements for 6RX, AT&T.TSG-RAN4#110-bis meeting.