3GPP TSG RAN WG4 #111 meeting 	 	R4-2408028

Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	10.1.1.2.1

Source:	Meta Ireland

Title:	High power UE RF requirements for intra-band CA in TN

Document for:	Approval

- Introduction

In RAN4 #110bis meeting, RAN4 agreed the WF on HP UE RF enhancements for UL CA and EN-DC in Rel-19 [1]. In the WF, RAN4 only decided MOP tolerance, Pcmax tolerance and ACLR with 31dB.

Major SAR solution and RF architecture for high power (PC1.5) for intra-band contiguous CA/non-contiguous CA will be further discussed to downselect the candidate options.

The detail FFS issues are list up based on WF [1] as follow:

PC1.5 UE (HPUE) for intra-band CA in terrestrial network (TN) [1] 1.2 SAR solution Way forward: FFS on following options SAR solution with consideration the P-MPR is applicable. - Option 1: Duty-cycle solution is not considered for any HPUE scenarios in this WI - Option 2：Duty-cycle solution is considered for all HPUE scenarios in this WI - Option 3：Duty-cycle solution is considered for all HPUE scenarios in this WI except for “increasing UE transmission power limit” - Option 4: Duty-cycle solution is considered for limited scenarios, further study which scenarios to consider 2.1 Assumed UE architectures for MPR/A-MPR evaluation - Option 1: (Xiaomi, Samsung, Huawei) Architecture; Description; Indicated capability; Support UL MIMO; Applicable cases; Note. #1; 2x26 dBm PA + 2 LO with 100MHz BW; dualPA-Architecture; No; C/NC CA; No Frequency Separation limitation. #2; 2x26 dBm PA + 1 LO with 200MHz BW; TxD; Yes; C/NC CA; Frequency Separation ≤ 200MHz For NC CA, FFS whether BW Gap size＜CC1 +CC2 CBW. NOTE: support UL MIMO here doesn’t mean UE has to support the feature, it depends on UE capability reporting - Option 2: (Apple, vivo, Skyworks, Meta) Architecture; Description; Indicated capability; Support UL MIMO; Applicable cases; Note. #1; 2x26 dBm PA + 2 LO with 100MHz BW; dualPA-Architecture; No; NC CA; No Frequency Separation limitation. #2; 2x26 dBm PA + 1 LO with 200MHz BW; TxD; Yes; Contiguous CA. Way forward: Both options are considered for next meeting discussion, FFS on down selection of UE architectures - Prioritize UL contiguous CA 2.2 General consideration for MPR/PCMAX/PCMAX,c Way forward: - For PC1.5, intra-band contiguous ULCA with UL MIMO can share the MPR requirements of intra-band contiguous ULCA without UL MIMO and with dual Tx (TxD) - Study and if necessary, specify the MPR requirements for both small and large form factor UEs, i.e. minimum antenna isolation = 10 or 20dB - For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, consider PCMAX,,c limitation (i.e., 26dBm) for each component carrier, which have impact on MOP, PCMAX tolerance and MPR/A-MPR evaluation - FFS For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10)) - Other options are not precluded

In this paper, we suggest how to define the RF requirements for high power UE for intra-band contiguous CA i.e. n41C, n77C and other intra-band non-contiguous CA i.e. CA_n78(2A), CA_n77(2A).

- MPR simulation assumptions of High power intra-band CA UE

  - MPR assumptions for intra-band contiguous CA UE

In this session, we propose the MPR simulation assumptions for PC1.5 CA UE for single carrier.

This is based on the previous RAN4 agreements for high power PC2 CA UE. On top of theses parameters, just modified these assumptions for PC1.5 intra-band contiguous CA UE as follows in Table 1.

Table 1. MPR simulation assumption for intra-band Contiguous CA

MPR simulation assumptions for intra-band contiguous CA - 2CC and 100MHz CBW per CC is assumed. - 2x26dBm PA + 1LO up to 200 MHz aggregated CBW are the baseline for MPR/A-MPR evaluation. - 2x26dBm PA + 2LO also can be considered as vendor implement aspect. - PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR. - 26dBm/antenna: 29dBm at 31dB ACLR - ULTR-ACLR is not considered. - Modulation order: QPSK/16QAM/64QAM/256QAM - Carrier leakage: 28 dBc for the allocated CC and non-allocated CC - IQ image: 28 dBc for the allocated CC and non-allocated CC - EVM: Apply per CC in Table 6.4A.2.1.1-1 in TS38.101-1 - SEM: Apply Table 6.5A.2.2.1-1 in TS38.101-1 for both CCs are active - SE: Follow Table 6.5.3.1-2 in TS38.101-1 - Back-off is relative to 29dBm at the antenna. - Equal PSD and Equal back-off power split - Measurements is used where two PA are coupled on the output recreating the 10dB antenna isolation assumption for HH UE. - For FWA UE, RAN4 can consider 20dB antenna isolation - Emission requirements (ACLR/SEM/spurious emissions) are checked by summing the power of the two transmit paths. - Since simulation are not available, at least worst-case corners are evaluated for different modulation order.

In Table 2, we propose the detail MPR simulation assumptions for PC1.5 intra-band non-contiguous CA UE as follows:

Table 2. MPR simulation assumption for intra-band Non-Contiguous CA

MPR simulation assumptions for intra-band Non-Contiguous CA - 2CC and 100MHz CBW per CC is assumed. - 2x26dBm PA + 2LO up to 200 MHz aggregated CBW are the baseline for MPR/A-MPR evaluation. - 2x26dBm PA + 1LO also can be considered as vendor implement aspect. - PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR. - 26dBm/antenna: 29dBm at 31dB ACLR - ULTR-ACLR is not considered. - Modulation order: QPSK/16QAM/64QAM/256QAM - SEM: Apply section 6.5A.2.2.2 in TS38.101-1 for both CCs are active. - SE: Apply section 6.5A.3.1 in TS38.101-1 for both CCs are active. - Variable Frequency separation will be considered according to BCS of the intra-band NC CA band. - Back-off is relative to 29dBm at the antenna. - Equal PSD and Equal back-off power split - Measurements is used where two PA are coupled on the output recreating the 10dB antenna isolation assumption. - For FWA UE, RAN4 can consider 20dB antenna isolation - Emission requirements (ACLR/SEM/spurious emissions) are checked by summing the power of the two transmit paths. - Since simulation are not available, at least worst-case corners are evaluated for different modulation order.

Based on the above detail RF simulation parameters, RAN4 can derive the MPR requirements for PC1.5 intra-band contiguous CA UE and PC1.5 intra-band non-contiguous CA UE.

Proposal 1: The proposed MPR simulation assumptions in Table 1 &2, RAN4 can derive the detail MPR requirements for PC1.5 intra-band CA UE.

  - RF architectures and SAR solution of PC1.5 intra-band contiguous CA UE

In TS38.101-1 in Rel-18, RAN4 defined the MPR requirements with dual Tx for single carrier as shown in Table 3 (original Table 6.2D.2-2 in TS38.101-1). The same RF architecture with separate PA and separate antenna RF architectures will be assumed for intra-band contiguous CA with dual Tx for CA_n41C, CA_n77C, CA_n78C and CA_n79C.

Table 3: Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Based on the same RF architecture, RAN4 need evaluate the MPR requirements to support wide intra-band contiguous CA up to 200MHz.

Proposal 2: RAN4 consider separate PA (2x26dBm) +1LO RF architecture as baseline which was proposed in Table 1 to derive the MPR/A-MPR requirements for those example intra-band contiguous CA combinations.

For the SAR regulation requirements, RAN4 can reuse ‘the max uplink dutycycle limitation’ of the single carrier SAR solution for intra-band contiguous CA operation. It means that intra-band contiguous CA UE will be operated by synchronous operation between 2CCs as like simultaneously transmit or reception to comply the SAR limitation.

Proposal 3: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band contiguous CA combinations UE.

  - RF architectures and SAR solution of PC1.5 intra-band non-contiguous CA UE

In PC2 high power intra-band non-contiguous CA in Rel-17, RAN4 considered 4 candidate RF architectures to support different PA capability combinations and the limitation of supporting channel Bandwidths.

However, RAN4 can only consider 2x26dBm PA + 2LO as baseline architecture to support up to 200MHz CBW and the baseline RF architecture will be considered to derive MPR/A-MPR requirements for PC1.5 intra-band non-contiguous CA operation with 2Tx.

Proposal 4: RAN4 can consider separate PA (2x26dBm) + 2LO RF architecture as baseline which was proposed in Table 2 to derive the MPR/A-MPR requirements for those example intra-band non-contiguous CA combinations.

For the SAR regulation requirements, RAN4 can reuse ‘the max uplink dutycycle limitation’ of the single carrier SAR solution for intra-band non-contiguous CA operation. The same SAR solution will be applied for all intra-band contiguous CA and intra-band non-contiguous CA UE. It means that intra-band non-contiguous CA UE will be operated by synchronous operation between 2CCs as like simultaneously transmit or reception to comply the SAR limitation.

Proposal 5: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band non-contiguous CA combinations UE.

  - Accounting for RB allocation BW imbalance for intra-band non-contiguous CA UE

To derive MPR/A-MPR requirements with equal PSD per CC, the MPR level can be applied to equally back-off in each CC in 1st phase MPR analysis.

The following RB combinations with two 26dBm PA can be assumed to derive MPR requirements.

Table 4: PCmax versus LCRB ratio for SCS1=SCS2 and one 26dBm PA per CC [5]

| LCRB1 | 273 | 273 | 273 | 273 | 273 | 273 | 273 | 273 | 273 | 273 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB2 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 273 |
| RBBWratio | 273 | 136.5 | 68.25 | 34.13 | 17.06 | 8.53 | 4.27 | 2.13 | 1.07 | 1 |
| PCmax | 26.0 | 26.0 | 26.1 | 26.1 | 26.2 | 26.5 | 26.9 | 27.7 | 28.9 | 29.0 |

And then, RAN4 further discuss how to define Pcmax for high power intra-band non-contiguous CA UE with equal PSD per CC

Proposal 6: RAN4 can consider the above table 4 power combination to derive MPR requirements with same power density level for both CCs in 1st phase. Also, RAN4 need further discuss how to define the PCMAX for high power intra-band non-contiguous CA UE.

  - Configured transmitted power for PC1.5 intra-band contiguous CA UE

For PC1.5 intra-band contiguous CA UE, RAN4 can reuse the configured transmitted power Pcmax,c for PC2 intra-band contiguous CA UE as follows:

- Just included ΔPPowerClass,CA = 6 dB for a power class 1.5 CA UE

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA }

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in clause 6.2A.2 and 6.2A.3, respectively;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.1; otherwise ΔPPowerClass,CA = 0 dB;

-	ΔPPowerClass,CA = 6 dB for a power class 1.5 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.1; otherwise ΔPPowerClass,CA = 0 dB;

NOTE: 	UE reports ∆PPowerClass,CA when [∆PPowerClass,CA reporting capability XXX-r18 is present], dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

-	∆TRxSRS is the highest value among all serving cells c;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

For uplink intra-band contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

Proposal 7: RAN4 can update configured Transmitted power for PC1.5 intra-band contiguous CA as above session 2.5.

- Conclusions

In this contribution, we propose some RF architectures and SAR solution for intra-band contiguous CA or non-contiguous CA. Also, we propose detail MPR simulation parameters to derive MPR requirements for PC1.5 intra-band contiguous CA/non-contigous CA UE. Based on our detail proposals for the high-power intra-band CA, we proposed as follow

Proposal 1: The proposed MPR simulation assumptions in Table 1 &2, RAN4 can derive the detail MPR requirements for PC1.5 intra-band CA UE.

Proposal 2: RAN4 consider separate PA (2x26dBm) +1LO RF architecture as baseline which was proposed in Table 1 to derive the MPR/A-MPR requirements for those example intra-band contiguous CA combinations.

Proposal 3: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band contiguous CA combinations UE.

Proposal 4: RAN4 can consider separate PA (2x26dBm) + 2LO RF architecture as baseline which was proposed in Table 2 to derive the MPR/A-MPR requirements for those example intra-band non-contiguous CA combinations.

Proposal 5: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band non-contiguous CA combinations UE.

Proposal 6: RAN4 can consider the above table 4 power combination to derive MPR requirements with same power density level for both CCs in 1st phase. Also, RAN4 need further discuss how to define the PCMAX for high power intra-band non-contiguous CA UE.

Proposal 7: RAN4 can update configured Transmitted power for PC1.5 intra-band contiguous CA as above session 2.5.

Reference

[1] R4-2406583, “WF on on HPUE UE for UL CA and EN-DC,” Samsung

[2] RP-240828, “New WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,” Huawei

[3] TS38.101-1

[4] TS38.101-3

[5] R4-2404901, “On architecture aspects for PC1p5 intra-band ULCA support,” SKW