3GPP TSG-RAN WG4 Meeting #111	R4-2408032

Fukuoka, JP, May 2024

Source	:	Qualcomm Incorporated

Title	:	On LPWUR system considerations

Agenda item	:	10.14.2.1

Document for	:	Approval

# 1.	Introduction

The low-power wake-up signal and receiver belong to a class of features that bring no immediately quantifiable benefit to infra vendors, but there is an indirect benefit through lower energy consumption in the UE. This complicated relationship implies that RAN4 must have a good general understanding of how much benefit each potential deployment variant (FR1-only vs FR1 as well as FR2, idle vs connected) brings to the table. This understanding could be the basis for actually deploying this feature despite the ‘benefit’ problem.

# 2. 	Discussion

## 2.1	Scenarios to pursue

The WID outlines that the WUS design must prioritize idle mode. It however does not prioritize use cases. Since this feature does not benefit infra vendors directly, at a minimum, it would be useful for RAN4 to clearly outline the benefits to the UE for each use case. This type of exercise will help network operators to weight the net benefit of deploying such a feature. Below are system level observations on idle vs connected more, and UE benefits thereof.

The UE autonomously and privately decides whether it is in a low power mode, based on measurements. The system cost however does not reduce when a UE exits out of the low power mode, because the network would continue to set aside DL resources for LPWUS. This situation is quite unattractive for infra vendors and will reflect on the probability of deployment. In connected mode, the network is always aware of the UE’s status, so there is potential system level benefit that can be estimated by the network.

Observation 1: The gNB does not know if its LP-WUS is beneficial for idle mode UEs, but it is aware of the connected mode UEs using LP-WUS.

A second issue is energy consumption in idle vs connected modes. The long-term energy consumptions attributable to the radio component are estimated based on typical usage patterns seen in FR1 bands and UE design decisions to optimize choice of LPWUR or MR in the presence of UL and DL:

| Condition | Notes | FR1, long term energy consumption | FR2, long term energy consumption |
| --- | --- | --- | --- |
| Baseline (Rel-16 WUS) | Rel-16 WUS | X | Y |
| Rel-19 WUS, real usage | Time average of idle and connected modes | 0.94 * X | 0.88 * Y |
| Rel-19 WUS, connected-mode only | Real usage above filtered for connected mode periods, to determine additional gains from idle mode WUS functionality | (not available) | 0.87 * Y |

In the table above, Y > X as would be expected in practical UEs. (i.e. FR2 per-band power consumption is higher than that of FR1)

The first observation from our analysis is that the long-term energy consumption reduction is a greater fraction for FR2  {(1-0.88) > (1-0.94)}. This greater fraction also applies to a greater long-term energy consumption (Y > X). Together, they suggest that a stronger case for WUS and WUR can be made in FR2. Note that this result applies even under the Rel-19 restrictions (‘In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR’). The energy reduction gets more attractive yet for FR2 as these restrictions are removed in the future and more functionality is relocated from the MR to the LPWUR.

Observation 2: For typical usage patterns, UEs stand to enjoy more significant energy consumption reduction in FR2 bands than FR1.

The choice of example FR2 band can be based on current and imminent FR2 deployments. A ‘low-band’ (24-30 GHz) seems like a good representative for this purpose, and for the sake of definition, it is proposed that n258 be used as the example FR2 band.

Proposal 1: RAN4 to consider using n258 as the example FR2 band.

A second  interesting finding with energy consumption reduction over a long period of typical usage is how much accrues over real time versus how much accrues only over periods filtered for connected  mode operation (12 % vs 13%).  This shows that despite the long periods of idle time, idle mode energy consumption is so small that the connected mode energy consumption dominates the long-term average for FR2. Similar results are expected for FR1.

Observation 3: For typical usage patterns, connected mode energy consumption dominates the long-term usage energy consumption. Idle mode consumption reduction is less important.

Now, RAN4 UE RF requirements are expected to be invariant of idle or connected mode. Given the promise of better savings in connected mode, the side conditions may be better constructed to reflect both modes.

Proposal 2: RAN4 to reflect both idle and connected mode conditions in the side conditions for the LPWUR requirements.

## 2.2	Refsens specification of the LPWUR

The WID clearly outlines that the LPWUR coverage radius should equal or exceed that of msg3. This system consideration should supersede any NF and SNR based expectation if the latter gives rise to more relaxed REFSENS values. In other words: the REFSENS requirement (dBm) cannot be more relaxed than either of the two criteria:

- A NF and SNR based projection (UE feasibility consideration)

- Coverage consideration (network consideration)

Typ. DL channel with WUS

WUS

Refsens test condition

WUS

Legacy NR sub carriers

Furthermore, the REFSENS signal was agreed to be the naked LPWUS with zero power adjacent DL RBs. In the real world this is unlikely to be the deployment scenario. Consequently, the ‘coverage consideration’ in determining the requirement for REFSENS must also account for desensitization due to the legacy DL RBs

Proposal 3: The Refsens condition is determined as the more stringent on the two:

- A NF and SNR based projection (UE feasibility consideration)

- Coverage consideration including legacy DL signal in adjacent subcarriers (network consideration)

# 3.	Conclusion

Observation 1: The gNB does not know if its LP-WUS is beneficial for idle mode UEs, but it is aware of the connected mode UEs using LP-WUS.

Observation 2: For typical usage patterns, UEs stand to enjoy more significant energy consumption reduction in FR2 bands than FR1.

Proposal 1: RAN4 to consider using n258 as the example FR2 band.

Observation 3: For typical usage patterns, connected mode energy consumption dominates the long-term usage energy consumption. Idle mode consumption reduction is less important.

Proposal 2: RAN4 to reflect both idle and connected mode conditions in the side conditions for the LPWUR requirements.

Proposal 3: The Refsens condition is determined as the more stringent on the two:

- A NF and SNR based projection (UE feasibility consideration)

- Coverage consideration including legacy DL signal in adjacent subcarriers (network consideration)

# 4.	References

[1] 	R4-2404864, ‘remaining issue for power boosting feature’,  Ericsson, RAN4#110-Bis, Apr. 2024

[2] 	R4-2404929, ‘Power boosting features applicability to UL CA scenarios’, Intel, RAN4#110-Bis, Apr. 2024

[3] 	R4-2406713, ‘WF on power boosting feature in Rel-18’, Ericsson, RAN4#110-Bis, Apr. 2024