3GPP TSG-WG Meeting #111	R4-2408042

# Fukuoka, Japan,  May 20 – May 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RLM IS test-case is needed for the new requirements specified for 15 PRB and 12 PRB bandwidth |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case for RLM IS performance of 15PRB and 12 PRB bandwidth |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE functionality to support 15PRB and 12PRB BW may not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### Start of change 1

#### A.6.5.1.X	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode

##### A.6.5.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are specified in Table A.6.5.1.X.1-1. General test parameters as specified in Table A.6.5.1.2.1-2 with config 1 apply to this test for both config 1 and 2, except those specified in Table A.6.5.1.X.1-2. Cell specific test parameters as specified in Table A.6.5.1.2.1-3 apply except those specified in Table A.6.5.1.X.1-3.

The test procedure specified in A.6.5.1.2.1 applies to this test.

Table A.6.5.1.X.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 3 MHz (15PRB) |
| 2 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 3 MHz (12 PRB) |
| Note: The UE is required to pass the test with configuration 1 for every supported 3MHz band in FR1. If the UE indicates the support for band n100, the UE is also required to pass the test with configuration 2 in addition to the test with configuration 1. |  |

Table A.6.5.1.X.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 15 |
|  | Config 2 |  | 3: NRB,c = 12 |
| RMSI CORESET Reference Channel | Config 1 |  | [CR.1.X FDD] |
|  | Config 2 |  | [CR.1.Y FDD] |
| Dedicated CORESET Reference Channel | Config 1 |  | [CCR.1.X FDD] |
|  | Config 2 |  | [CCR.1.Y FDD] |
| SSB Configuration | Config 1, 2 |  | TBD |
| In sync transmission parameters | Number of Control OFDM symbols, Config 1 |  | 3 |
|  | Number of Control OFDM symbols, Config 2 |  | 2 |
|  | Aggregation level, Config 1 | CCE | 4 |
|  | Aggregation level, Config 2 |  | 2 |
|  | Mapping from REG to CCE, Config 1 |  | Distributed |
|  | Mapping from REG to CCE, Config 2 |  | Distributed |
| Out of sync transmission parameters | Number of Control OFDM symbols, Config 1 |  | 3 |
|  | Number of Control OFDM symbols, Config 2 |  | 2 |
|  | Aggregation level, Config 1 | CCE | 8 |
|  | Aggregation level, Config 2 |  | 4 |
|  | Mapping from REG to CCE, Config 1 |  | Non -Distributed |
|  | Mapping from REG to CCE, Config 2 |  | Distributed |

Table A.6.5.1.X.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |

##### A.6.5.1.X.2	Test Requirements

Test requirements specified in Clause A.6.5.1.2.2 apply to this test.

#### End of change 1