3GPP TSG-RAN WG4 Meeting #111	R4-2408074

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	10.11.2

Source:	MediaTek Inc.

Title:	Discussion on AI/ML RAN4 BM use case

Document for:	Discussion

# Introduction

In this contribution, we provide our views about AI/ML RAN4 BM use case.

# Discussion

      - 2.1 RRM core requirements

Last meeting, the potential impact on RRM requirements was proposed. Here we also provide our initial thinking on the potential impacts to AI/ML BM.

| RRM Req. Category | RRM procedure | RAN4 impact |
| --- | --- | --- |
| IDLE state mobility – Cell selection | Cell selection | No impact |
| IDLE/INACTIVE state mobility – Cell re-selection | Measurement and evaluation of serving cell | No impact |
|  | Measurements of intra-frequency NR cells |  |
|  | Measurements of inter-frequency NR cells/inter-RAT |  |
| RRC_CONNECTED state mobility - Handover | NR Handover | No impact |
|  | Handover to other RATs |  |
|  | NR DAPS Handover |  |
|  | NR Conditional Handover |  |
|  | NR Handover with PSCell |  |
| RRC_CONNECTED state mobility - RRC Connection Mobility Control | SA: RRC Re-establishment | No impact |
|  | Random access |  |
|  | SA: RRC Connection Release with Redirection |  |
| Timing | UE Transmit timing | No impact |
|  | UE Timer accuracy |  |
|  | Timing advance |  |
|  | Cell phase synchronization accuracy |  |
|  | Maximum transmission timing difference |  |
|  | Maximum receive timing difference |  |
| Signaling characteristics | SSB /CSI-RS based Radio Link Monitoring | No impact - The SSB/CSI-RS for RLM should be always transmitted by NW as legacy. |
|  | Interruption | No impact |
|  | Scell Activation/Deactivation Delay |  |
|  | UE UL carrier RRC reconfiguration delay |  |
|  | Link recovery procedures (SSB/CSI-RS based beam failure indication, L1 indication, Candidate beam detection) | No impact on beam failure detection - The SSB/CSI-RS for RLM should be always transmitted by NW as legacy. FFS on candidate beam detection |
|  | Active BWP switch delay | No impact |
|  | Active TCI state switching delay | Impact - at least known TCI state condition should be further discussed. |
|  | Uplink spatial relation switch delay |  |
|  | UE-specific CBW change | No impact |
|  | Pathloss reference signal switching delay | Impact - at least known TCI state condition should be further discussed. |
|  | Active downlink TCI switching delay for unified TCI |  |
|  | Active uplink TCI switching delay for unified TCI |  |
|  | Functionality/Model monitoring procedure | FFS new requirements |
|  | Functionality/Model selection/activation/deactivation/fallback delay |  |
|  | Active model switch delay |  |
| Measurement procedure | NR intra-frequency measurements (Number of cells and number of SSB, measurement reporting requirements, etc) | No impact - No impact on L3 measurement |
|  | NR inter-frequency measurements | No impact |
|  | Inter-RAT measurements |  |
|  | L1-RSRP measurements for reporting | FFS |
|  | Cross link interference measurements | No impact |
|  | L1-SINR measurements for reporting |  |
| Measurement performance requirements – NR measurements | Intra-frequency RSRP accuracy requirements for FR1/FR2 | No impact - no impact on L3 measurement |
|  | Inter-frequency RSRP accuracy requirements for FR1/FR2 | No impact |
|  | RSRP measurement report mapping | FFS |
|  | Intra-frequency RSRQ accuracy requirements for FR1/FR2 | No impact |
|  | Inter-frequency RSRQ accuracy requirements for FR1/FR2 |  |
|  | RSRQ report mapping |  |
|  | Intra-frequency SINR accuracy requirements for FR1/FR2 |  |
|  | Inter-frequency SINR accuracy requirements for FR1/FR2 |  |
|  | SINR report mapping |  |
|  | L1-RSRP accuracy requirements for FR1/FR2 | Depend on which metric to use to test AI/ML BM models |
|  | L1-SINR accuracy requirements for FR1/FR2 | No impact |

Specially, as AI/ML beam prediction is for the purpose of beam management, there may be some impact on beam management related requirements. For instance, after UE reports the predicted beams or RSRP to NW, NW may activate corresponding reported best-predicted beam. In RAN4, we defined TCI state switching delay requirements. Take the requirements of MAC CE based TCI state activation as an example. The TCI state switch delay is THARQ + 3ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length, where TL1-RSRP = 0 for known TCI state and TL1-RSRP equals to L1 measurement delay in FR2 for unknown TCI state. If the TCI state is unknown, the switching delay would be long. One of the conditions of known TCI state is that UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command in 1280ms. The question here is whether the predicted beam is known or unknown if a prediction report is sent recently. Next, we would like to provide our views.

As UE is the one who performs TCI state switching, when talking about known condition, the corresponding TCI state should be actually known from UE side. At least two conditions shall be met in FR2. One is that SINR of the corresponding RS cannot be too low. Another is that UE knows its Rx beam for reception (for DL TCI state activation). To meet the 2nd condition, a TCI state is known only if UE has measured the corresponding RS and reported the measured result in last 1280ms before TCI state switching command. Here if the beam to switch to does not belong to set B, even there is a prediction report but UE may not know which Rx beam to use for DL reception. Note that in the WID only DL Tx beam prediction is in scope and Rx beam prediction is not mandated as shown below.

From WID: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 1: Only DL Tx beam prediction is in scope and Rx beam prediction is not for both spatial-domain and temporal beam prediction.

Proposal 1: The TCI state QCL to an RS that is not in Set B is unknown if no L1 measurement is performed by UE within 1280ms before TCI state activation.

For the TCI state whose QCL source RS belongs to Set B, if it is spatial-domain beam prediction, the TCI state can be viewed as known if the corresponding predicted beam is reported in 1280ms. If it is temporal beam prediction, we should use the last observation occasion (T3) instead of the reported occasion as shown in Figure 3. The reason lies in that the time gap between T3 and T4 may be too long to ensure Rx beam unchanged. The report occasion may be close to T4.

Figure 3. Illustration for AI/ML temporal beam prediction

Proposal 2: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

      - 2.2 Ground Truth and Testability

Considering UE-sided model and FR2 test, the possible test procedures for AI/ML BM performance testing are shown as follows:

- Before the testing, the AI/ML models are offline trained.

- During the testing, beams in set B are transmitted from TE to the DUT, i.e., UE, in the chamber through air interface.

- DUT performs measurement on beams in set B to obtain inference data.

- DUT reports the results, e.g., predicted Top-1/Top-K beam index or predicted L1-RSRP in set A, to TE for performance verification.

- TE decides if the DUT passes the tests based on the defined metrics.

In above procedures, there are two options for data used in training, inference, and performance evaluation:

- Option 1: Measured L1-RSRP which contains measurement error

- Option 2: Ideal L1-RSRP set by TE

The data for inference are obtained by measuring the beams in set B transmitted from TE. Therefore, the measured L1-RSRP will contains measurement errors.

Observation 2: Considering the UE-sided model and FR2 test, the data for inference are obtained by measuring the beams in set B transmitted from TE. Therefore, the measured L1-RSRP will anyway contains measurement errors.

For AI/ML model training, it will be easy for training data collection if we use the ideal L1-RSRP. However, during the test or in the field, the inference input to the AI/ML model is measured L1-RSRP, not ideal L1-RSRP. The mismatch between the data for training and inference may cause performance degradation.

Observation 3: Either ideal L1-RSPR or measured L1-RSRP could be used for offline training.

Observation 4: It will be easy for training data collection if we use the ideal L1-RSRP generating from channel emulator.

Observation 5: During the test or in the field, the inference input to the AI/ML model is measured L1-RSRP, not ideal L1-RSRP. If the AI/ML model is trained with ideal L1-RSRP, it may cause performance degradation due to the mismatch between the data for training and inference.

To verify the performance for AI/ML BM, TE must have the predicted L1-RSRP reported from UE and the so-called “ground truth” to derive test metrics. It is also possible to consider both options, i.e., ideal or measured L1-RSRP, in the calculation of test metrics. For the case of measure L1-RSRP, even though the measured L1-RSRP in set A at the TE side can be obtained by the report from the UE side, it will complicate the whole testing process as UE needs to measure all beams in set A.

Observation 6: Either ideal L1-RSPR or measured L1-RSRP could be used as ground truth for performance evaluation.

Observation 7: Even though the measured L1-RSRP in set A at the TE side for performance evaluation can be obtained by the report from the UE side, it will complicate the whole testing process as UE needs to measure all beams in set A.

The ideal L1-RSPR can be obtained from current channel models in TR 38.901. However, in reality, the L1-RSRP at UE side is by measurement and will anyway include measurement error. We propose simulation results by considering ideal L1-RSRP and measured L1-RSRP. For measured L1-RSRP, we also investigate different modelling of measurement error, i.e., BB error or BB+RF error. For RF error, it is assumed that if UE uses the same Rx chain to calculate L1-RSRP over different Tx beam, then the RF error assumed to be the same. We consider the following error distribution in our simulation.

  - BB error

    - BB error is sampled from single Gaussian distribution with 95% samples in ±2dB

- BB + RF error

    - BB error is sampled from single Gaussian distribution with 95% samples in ±2dB

    - RF error (specific bias for each Rx) is sampled from single Gaussian distribution with 95% samples in ±4dB

| Figure 1. Measured L1-RSRP by consdiering BB error | Figure 2. Measured L1-RSRP by consdiering BB + RF error |
| --- | --- |

Table 1. Simulation results for ideal L1-RSRP and measured L1-RSRP in AI/ML BM

|  | Ideal L1-RSRP | Measured L1-RSRP BB error | Measured L1-RSRP BB + RF error |
| --- | --- | --- | --- |
| Top-1 prediction accuracy (%) | 71.9 | 58.6 | 48.4 |
| Top-5/1 prediction accuracy (%) | 94.2 | 91.2 | 84.2 |

Take BM-Case 1 as an example, the AI/ML model predicts set A from set B by learning the spatial domain correlation between set B and set A. However, the measurement noise will degrade the correlation among set A and set B. From our results, it shows a large difference between ideal L1-RSPR and measured L1-RSRP.

Observation 8: From our simulation results, there is a large inference performance difference between ideal L1-RSPR and measured L1-RSRP.

Proposal 3: Based on observations 2~8, we suggest RAN4 to investigate the testability of ideal L1-RSRP and measured L1-RSRP in the BM performance test.

In Table 1, it also shows non-negligible difference between two different measurement error modelling. Therefore, RAN4 should conclude the error modelling when deriving the requirements as different measurement error modelling results in different requirements.

Observation 9: From our simulation results, there is a non-negligible difference between different measurement error modelling when considering measured L1-RSRP.

Proposal 4: If RAN4 agreed to use measured L1-RSRP in BM test, RAN4 should conclude the error modelling when deriving the requirements.

During the discussion of RAN4, companies raised concerns regarding the limited number of FR2 beams that TE can support. Even if TE ensures sufficient FR2 beams for BM testing, the chamber and beam patterns from different TEs may lead to different test results. Also, different UE implementation will also result in distinct L1-RSRP measurement error. Based on our results, it also impacts the results. RAN4 define performance requirements based on the simulations with some assumptions. The most crucial thing is how to verify the assumptions used for simulation to reflect the real correlation and measurement noise for set A and set B when UE is tested in an anechoic chamber.

Observation 10: Different TE and UE implementation may impact the inference performance of AI/ML BM.

Proposal 5: RAN4 to discuss how to verify the assumptions used for simulation to reflect the real correlation and measurement noise for set A and set B when UE is tested in an anechoic chamber.

## 2.3 Test Metric

In TR38.843, there are lots of options for the test metric of AI/ML based beam management.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

We need to have a clear definition of the ground truth for the derivation of test metric. For Option 2 and Option 3, the metric calculation is based on the beam index of the Top-1/Top-K predicted beams and strongest beams. The beam index of the Top-1/Top-K predicted beams is clearly defined, which is based on the output of AI/ML model. However, the definition of the Top-1/Top-K strongest beams is not so clear as the strongest beam could be ideal strongest or measured strongest beam.

Observation 11: The definition of strongest beam in metrics for beam management requirements/tests is not clear. It could be ideal strongest or measured strongest beam.

Proposal 6: RAN4 should clearly define the ground truth in AI/ML BM testing. To discuss the strongest beam is ideal or measured strongest beam.

The Option 3 is similar to the KPI “Top-1(%) with 1dB margin” in RAN1 and the ideal L1-RSRP of Top-1 predicted beam is used.

| KPI |  | Description/Definition |
| --- | --- | --- |
| Beam prediction accuracy (%) | Top-1(%) with 1dB margin | the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam” |

In Option 3, the maximum RSRP among top-K predicted beams is used. It is also not clear the “maximum RSRP” is the maximum ideal L1-RSRP or the maximum predicted L1-RSRP of the top-K predicted beams.

Observation 12: In Option 3, the definition of maximum RSRP among top-K predicted beams is not clear. It could be the maximum ideal L1-RSRP or the maximum predicted L1-RSRP of the top-K predicted beams.

Proposal 7: RAN4 should clearly define the maximum RSRP among top-K predicted beams in Option 3. To discuss “maximum RSRP” is the maximum ideal L1-RSRP or the maximum predicted L1-RSRP of the top-K predicted beams.

## 2.4 Generalization in AI/ML BM case

There are lots of generalization aspects were evaluated in RAN1 [1].

- Scenarios - Various deployment scenarios, - e.g., UMa, UMi - e.g., 200m ISD or 500m ISD - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility (for BMCase-2 only), - e.g., 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, - e.g., UE codebook - e.g., UE antenna array dimensions - e.g., different number beams in a seen UE codebook when inference using a subset of Rx beams of training - Various gNB settings, - e.g., DL Tx beam codebook - e.g., gNB antenna array dimensions - Various Set A of beam(pairs) - Various Set B of beam (pairs)

We use two different gNB beam structures to evaluate the AI model’s generalization capability. There are 8 beams in horizontal and 4 beams in vertical for the first gNB beam structure (denoted as H8V4). For the second gNB beam structure, there are 16 beams in horizontal and 2 beams in vertical (denoted as H16V2). With different gNB beam structures during the AI/ML model training phase and testing phase, the DL Tx beam prediction performance is conducted with the following cases.

- Generalization Case 1: The training data samples of the AI/ML model are generated based on the gNB beam structures of H8V4/H16V2. The testing data samples of the AI/ML model are generated based on the gNB beam structure of H8V4/H16V2.

- Generalization Case 2: The training data samples of the AI/ML model are generated based on the gNB beam structures of H16V2/H8V4. The testing data samples of the AI/ML model are generated based on the gNB beam structure of H8V4/H16V2.

- Generalization Case 3: The training data samples of the AI/ML model are mixed data samples, which are generated based on the two different gNB beam layouts of H8V4 and H16V2. The testing data samples of the AI/ML model are generated based on the gNB beam structure of H8V4 or H16V2.

Table x: Generalization results for different gNB beam structures

| Generalization case | training and testing data samples | Top-1/-3/-5 acc [%] |
| --- | --- | --- |
| Generalization Case 1 | training data samples: H8V4 testing data samples: H8V4 | 71.2/92.2/97.1 |
|  | training data samples: H16V2 testing data samples: H16V2 | 63.7/91.3/96.6 |
| Generalization Case 2 | training data samples: H16V2 testing data samples: H8V4 | 21.6/38.4/50.7 |
|  | training data samples: H8V4 testing data samples: H16V2 | 16.4/35.6/45.9 |
| Generalization Case 3 | training data samples: H8V4 and H16V2 testing data samples: H8V4 | 69.9/91.0/95.2 |
|  | training data samples: H8V4 and H16V2 testing data samples: H16V2 | 65.0/88.8/95.5 |

From our simulation results, it shows that the performance of generalization case 2 is very worse. If the training data samples and the testing data samples are generated under different gNB beam structures, a significant performance loss is observed. Therefore, RAN4 needs to carefully discuss on which aspect to consider generalization for AI/ML BM.

Observation 13: If the training data samples and the testing data samples are generated under different gNB beam structures, a significant performance loss is observed.

Proposal 8: RAN4 to carefully discuss on which aspect to consider generalization for AI/ML BM, e.g., DL Tx beam codebook, deployment scenarios, and UE mobility.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Only DL Tx beam prediction is in scope and Rx beam prediction is not for both spatial-domain and temporal beam prediction.

Proposal 1: The TCI state QCL to an RS that is not in Set B is unknown if no L1 measurement is performed by UE within 1280ms before TCI state activation.

Proposal 2: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

Observation 2: Considering the UE-sided model and FR2 test, the data for inference are obtained by measuring the beams in set B transmitted from TE. Therefore, the measured L1-RSRP will anyway contains measurement errors.

Observation 3: Either ideal L1-RSPR or measured L1-RSRP could be used for offline training.

Observation 4: It will be easy for training data collection if we use the ideal L1-RSRP generating from channel emulator.

Observation 5: During the test or in the field, the inference input to the AI/ML model is measured L1-RSRP, not ideal L1-RSRP. If the AI/ML model is trained with ideal L1-RSRP, it may cause performance degradation due to the mismatch between the data for training and inference.

Observation 6: Either ideal L1-RSPR or measured L1-RSRP could be used as ground truth for performance evaluation.

Observation 7: Even though the measured L1-RSRP in set A at the TE side for performance evaluation can be obtained by the report from the UE side, it will complicate the whole testing process as UE needs to measure all beams in set A.

Observation 8: From our simulation results, there is a large inference performance difference between ideal L1-RSPR and measured L1-RSRP.

Proposal 3: Based on observations 2~8, we suggest RAN4 to investigate the testability of ideal L1-RSRP and measured L1-RSRP in the BM performance test.

Observation 9: From our simulation results, there is a non-negligible difference between different measurement error modelling when considering measured L1-RSRP.

Proposal 4: If RAN4 agreed to use measured L1-RSRP in BM test, RAN4 should conclude the error modelling when deriving the requirements.

Observation 10: Different TE and UE implementation may impact the inference performance of AI/ML BM.

Proposal 5: RAN4 to discuss how to verify the assumptions used for simulation to reflect the real correlation and measurement noise for set A and set B when UE is tested in an anechoic chamber.

Observation 11: The definition of strongest beam in metrics for beam management requirements/tests is not clear. It could be ideal strongest or measured strongest beam.

Proposal 6: RAN4 should clearly define the ground truth in AI/ML BM testing. To discuss the strongest beam is ideal or measured strongest beam.

Observation 12: In Option 3, the definition of maximum RSRP among top-K predicted beams is not clear. It could be the maximum ideal L1-RSRP or the maximum predicted L1-RSRP of the top-K predicted beams.

Proposal 7: RAN4 should clearly define the maximum RSRP among top-K predicted beams in Option 3. To discuss “maximum RSRP” is the maximum ideal L1-RSRP or the maximum predicted L1-RSRP of the top-K predicted beams.

Observation 13: If the training data samples and the testing data samples are generated under different gNB beam structures, a significant performance loss is observed.

Proposal 8: RAN4 to carefully discuss on which aspect to consider generalization for AI/ML BM, e.g., DL Tx beam codebook, deployment scenarios, and UE mobility.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface