3GPP TSG-RAN4 Meeting #111	R4-2408132

# Fukuoka, Japan, May 20 – 24, 2024

Agenda item:			10.1.1.3.1

Source:	vivo

Title:	Discussion on power domain enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#110bis meeting, the WF[1] has been agreed and the details about power domain enhancement for single carrier are as follows:

Way forward: - Scenarios for power domain enhancements for single carrier Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator) Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators) Scenario 2: Narrower UE channel BW within wider BS bandwidth - Prioritize scenario 1-1 and scenario 2 for initial study of power domain enhancements for single carrier in terms of relaxed requirements - FFS on sub-scenarios of scenario 2. - Scenario 1-2 will be studied after scenario 1-1 and scenario 2 - relaxed requirements from co-existence/regulation perspective - For regions where at least ACLR and SEM can be relaxed for the identified scenarios, FFS whether SE could be relaxed, or under which conditions can be relaxed. - General considerations for power domain enhancements - Only requirements relaxation should be considered for power domain enhancements in Rel-19 - only consider general requirements for the further evaluation - No power domain enhancements based on BWP is considered - Evaluation of relaxed requirements - [No relaxation of ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum for scenario 2, i.e. Narrower UE channel BW within wider BS bandwidth] - FFS whether outer, edge or inner RB allocation is prioritized for further evaluation - FFS impact on MPR by relaxing ACLR w/ or w/o relaxing SEM/Spurious Emissions - FFS whether IBE is considered instead of the relaxed ACLR/SEM/SE for the region between UE CBW and BS CBW.

- Discussion: Power domain enhancements for single carrier

## 2.1. Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue

For scenario 1-1, no adjacent in-band/out-of-band co-existence issue and only single operator in the spectrum means there is no need to consider ACLR during out-of-band emission evaluation. Based on this assumption, the simulation results of single carrier are shown in figure 1. The simulation setting sweeps the whole 20M UE CBW and collects the MPR value of different RB allocations, the waveform is DFT-s-OFDM, the modulation mode is 16QAM, and the power class is PC3.

It can be seen that the MPR reduction takes effect when ACLR is omitted but the reduced value is limited and is concentrated in the outer region. So the outer region could be regarded as a high priority for MPR reduction study.

Observation 1：MPR could be reduced to a certain extent when ACLR is omitted during OOB emission evaluation, and the reduction is concentrated in the outer RB region.

Proposal 1：In scenario1-1, ACLR could be omitted from out-of-band emission evaluation, and the outer region should be regarded as a high priority for MPR reduction study.

(a)OBO with all requirements considered        (b)OBO without ACLR considered

Figure 1: DFT-S-OFDM, 16QAM, PC3, 20M

Compared with (a), figure (b) shows that the reduction value is less than 1dB in most RB allocations. The closer the configuration is to the inner area, the smaller the MPR decreases. RB configurations with MPR reduction values greater than 1dB are concentrated in the outer region where RB starts from the edge of CBW. However, after reduction the MPR of part of the outer region could fall back to below 1dB and meet the requirement of the inner region, so omitting ACLR would cause part of the outer area to merge with the inner area, and the inner region can be expanded.

Observation 2: After reduction the MPR of part of the outer region could fall back to below 1dB and meet the requirement of the inner region, i.e., omitting ACLR would cause part of the outer area to merge with the inner area, and the inner region could be expanded.

## 2.2. Scenario 2: Narrower UE channel BW within wider BS bandwidth

The simulation assumption of scenario 2 is BS CBW=100M, UE CBW = 20M, DFT-S-OFDM waveform, QPSK, PC3. When original UE is allocated in a wider BS and the UE is far away from the BS edge, the emission requirements outside the UE CBW could be switched from stricter out-of-band requirements like ACLR to more tolerant in-band requirements like IBE.

While the IBE requirement is defined as a function of the RB offset from the edge of the allocated UL transmission bandwidth and is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. So if the applying range is expanded outside of UE CBW, the definition is changed and it should be used by a new name, e.g., IBE_BW.

So in the simulation the requirement considered between the UE CBW and BS CBW is IBE_BW, and ACLR is relaxed to apply from the BS edge and the applying width is equal to 100M. Based on these assumption, whether or not to relax SEM requirement would change the MPR reduction range.

Proposal 2: In scenario 2, the starting point for applying ACLR could be changed from the edge of UE CBW to BS CBW, and for the region between UE CBW and BS CBW the IBE_BW requirement could be applied instead.

### 2.1.1 Impact on MPR when only ACLR is relaxed

When only ACLR is relaxed, it is applied from the edge of BS while SEM keeps on applying from the edge of UE CBW, and the simulation results are shown in Figure2. It can be seen that the results are similar to scenario1-1, the MPR of some outer region falls back to less than 0 dB, and the main limit factor of further reduction is SEM.

Figure 2: DFT-S-OFDM, PC3, QPSK, SEM evaluated from edge of UE

Observation 3: If the starting point of the ACLR application range is relaxed to the edge of the BS CBW, the MPR in some outer regions will fall back below 0dB, thereby meeting the requirements of the inner region.

Combing observation 2 and 3, with relaxing or omitting ACLR the current inner region would be expanded and a new inner region should be defined instead. However, whether the new inner region range of scenario 1-1 and 2 should be the same is FFS.

Proposal 3: A new inner region should be defined for scenario1-1 and scenario 2 with ACLR relaxed and SEM/SE unchanged：

- For scenario 1-1, ACLR is omitted in MPR evaluation.

- For scenario 2, ACLR is applied from edge of BS CBW rather than UE CBW.

- FFS on whether the new inner should be same or not for both scenarios.

### 2.1.2 Discussion on SEM relaxation

The current SEM of NR is evolved from LTE. The SEM of LTE is formulated in combination with WCDMA and regional regulatory requirements such as FCC, PHS of Japan, OFCOM and so on. The template of [2] which combines various out-of-band emission mask is shown below. The LTE SEM mask (mask 100RB in figure4) can meet the emission requirements in some areas, but it does not meet the requirements of Japan PHS, etc. For those cases where the regulatory requirements does not meet with the general mask, the “Additional spectrum emission requirement” should be followed. Therefore, the current SEM mask is not evaluated based on the strictest regulation requirement, so whether or not the limit level of SEM could be relaxed could be discussed further.

The current mask refers to the out-of-band emission requirement of FCC band II in the frequency range of 5MHz <ΔFOOB < 20MHz, while FCC’s regulation requirement limitation in band II [3] is: ‘the power outside of the authorized operating frequency ranges must be attenuated below the transmitting power by…’. And in band 7 of FCC, the regulation requirement limitation[4] is: ‘the power of any emission outside a license's frequency band(s) of operation shall be attenuated below the transmitter power P…’. However, the applicable area of existing SEM described in [2] is: ‘Separate spectrum emission mask definitions are needed for each LTE RF carrier bandwidth.’ Comparing the wording, the applicable spectrum range of SEM mask in 3GPP seems stringent and the starting point of SEM should be discussed further, and there is still some room for relaxation.

The relaxation of SEM could start from two directions:

- Direction 1: Relax the applicable starting point of SEM

- Direction 2: Relax the current SEM power level

- For some regions with stricter regulatory requirements, the additional SEM can be applied to control the transmission power.

Figure 3：Regulatory mask and proposed E-UTRA masks in [2]

Proposal 4: The SEM could be relaxed to apply from the BS edge instead of UE edge. And whether the power level of SEM could be relaxed is FFS.

### 2.1.3 Impact on MPR when both ACLR and SEM is relaxed

The SEM relaxation is based on option1, i.e., SEM would also apply from the BS edge instead of UE edge. With both SEM and ACLR relaxing, the MPR reduction is more obvious and most of RB configurations would fall back to lower than 0dB, which could meet the inner region requirement. And the limit factor of further reduction is IBE_BW instead.

Observation 4: If both SEM and ACLR are relaxed to start applying from the edge of BS CBW, the MPR would basically fall back below 0dB.

Proposal 5: The MPR reduction value and the range of inner region depend on the type and quantity of relaxed out-of-band emission requirements, whether to relax or not should be judged and indicated by the BS.

Figure 4：DFT-S-OFDM, PC3, QPSK, SEM and ACLR evaluated from edge of BS

For scenario2, if both ACLR and SEM are relaxed to start applying from the edge of BS CBW, the BS CBW would be equivalent to the original UE CBW, that is, it is necessary to judge the actual RB allocations in BS rather than UE to determine whether it is in the inner or outer region.

Observation 5: For scenario 2, if both SEM and ACLR are relaxed to start applying from the edge of BS CBW, the RB allocation region division should be based on the actual RB allocations in BS rather than UE.

Proposal 6: If the starting point of the application range of ACLR and SEM is relaxed to the edge of the BS CBW, the inner and outer area ranges need to be redefined based on the relationship between RB allocation, UE CBW and the BS CBW.

ACLR considers the coexistence interference between different channels, and the BSs in adjacent frequency bands within the same operator cannot avoid coexistence problems even through scheduling, so the ACLR outside the BS cannot be relaxed.

Proposal 7: ACLR outside the BS CBW should not be relaxed even if the BS CBW is smaller than the bandwidth of the operator's holding spectrum band.

- Conclusion

Observation 1：MPR could be reduced to a certain extent when ACLR is omitted during OOB emission evaluation, and the reduction is concentrated in the outer RB region.

Proposal 1：In scenario1-1, ACLR could be omitted from out-of-band emission evaluation, and the outer region should be regarded as a high priority for MPR reduction study.

Observation 2: After reduction the MPR of part of the outer region could fall back to below 1dB and meet the requirement of the inner region, i.e., omitting ACLR would cause part of the outer area to merge with the inner area, and the inner region could be expanded.

Proposal 2: In scenario 2, the starting point for applying ACLR could be changed from the edge of UE CBW to BS CBW, and for the region between UE CBW and BS CBW the IBE_BW requirement could be applied instead.

Observation 3: If the starting point of the ACLR application range is relaxed to the edge of the BS CBW, the MPR in some outer regions will fall back below 0dB, thereby meeting the requirements of the inner region.

Proposal 3: A new inner region should be defined for scenario1-1 and scenario 2 with ACLR relaxed and SEM/SE unchanged：

- For scenario 1-1, ACLR is omitted in MPR evaluation.

- For scenario 2, ACLR is applied from edge of BS CBW rather than UE CBW.

- FFS on whether the new inner should be same or not for both scenarios.

Proposal 4: The SEM could be relaxed to apply from the BS edge instead of UE edge. And whether the power level of SEM could be relaxed is FFS.

Observation 4: If both SEM and ACLR are relaxed to start applying from the edge of BS CBW, the MPR would basically fall back below 0dB.

Proposal 5: The MPR reduction value and the range of inner region depend on the type and quantity of relaxed out-of-band emission requirements, whether to relax or not should be judged and indicated by the BS.

Observation 5: For scenario 2, if both SEM and ACLR are relaxed to start applying from the edge of BS CBW, the RB allocation region division should be based on the actual RB allocations in BS rather than UE.

Proposal 6: If the starting point of the application range of ACLR and SEM is relaxed to the edge of the BS CBW, the inner and outer area ranges need to be redefined based on the relationship between RB allocation, UE CBW and the BS CBW.

Proposal 7: ACLR outside the BS CBW should not be relaxed even if the BS CBW is smaller than the bandwidth of the operator's holding spectrum band.

- References

- R4-2406584, WF on power domain enhancements, Huawei, HiSilicon, RAN4#110bis.

- TR 36.803, User Equipment (UE) radio transmission and reception.

- FCC PART 22_PUBLIC MOBILE SERVICES  Subpart H_Cellular Radiotelephone Service Sec.  22.917 Emission limitations for cellular equipment.

- FCC PART 27_MISCELLANEOUS WIRELESS COMMUNICATIONS SERVICES Subpart C_Technical Standards Sec.  27.53 Emission limits.