# 3GPP TSG-RAN WG4 Meeting # 111	R4-2408319

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.15.1

Source: 	Ericsson

Title:	Remaining issues in MUSIM

Document for:	Discussion

# Introduction

In last meetings, RAN4 made great progress on MUSIM WI. Most of open issues were agreed[1]. In this contribution, we will discuss some remaining issues for MUSIM gaps.

- General

In legacy NR, a total of 25 MGPs are defined. To reduce the design complexity for UE side, mandatory MGPs are introduced. The UE only needs to support a subset of the MGPs mandatorily and whether UE supports other MGPs will be reported by capability. The mandatory MGPs are also useful to network scheduling. Especially, when different UE vendors may support different combination of MGPs, it’s highly impossible for network to schedule different MGPs to different UEs.

RAN4 #110b Issue 1-1-1: Mandatory MUSIM gap patterns or constraints on MUSIM gap request from UE side Proposals - P1: No need to introduce mandatory MUSIM gap patterns and constraints on MUSIM gap request from UE side (Apple oppo xiaomi Huawei MTK) - P2: Define 1 or 2 mandatory MUSIM gap patterns, as minimum the UE shall support MUSIM gap 6ms MGL and 160ms MGRP (Nokia ZTE) - P3: UE support at least one MUSIM gap pattern within a subset of MUSIM gap patterns and UE shall know the preferred MUSIM gap patterns from NW before UE requesting the MUSIM gaps.(Ericsson) - P4: For compromise, when UE requests more than one periodic MUSIM gaps, at least one MUSIM gap has a MGRP larger than x ms where x could be 1280 (vivo ZTE) - P5: Discuss whether an LS is needed to RAN5 confirming that RAN4 assumption is reasonable. (Nokia)

As RAN4 discussed before, the paging monitoring is important for MUSIM UE. Thus, it is encouraged from both NW’s and UE’s perspective to support the gap for paging monitoring. In Rel-17, if the UE requested a gap for paging but NW-A doesn’t support the gap pattern, NW-A had to reject the gap request other than change the gap pattern which will result in negative performance for MUSIM UE. Especially, considering total 29 MUSIM gap patterns, the overall signalling overhead and interaction time cannot be endured for both UE and NW side. Therefore, RAN4 shall define at least one MUSIM gap patterns which can be a guidance for both UE and NW design. When UE is in NW-A’s CONNECTED mode, to avoid unnecessary MUSIM gap request or to reduce the number of MUSIM gap requests, UE shall know the preferred MUSIM gap patterns from NW before requesting the MUSIM gaps.

Proposal 1: The UE which supports MUSIM feature shall support at least one MUSIM gap pattern within a subset of MUSIM gap patterns.

Proposal 2: UE shall know the preferred MUSIM gap patterns from NW before UE requesting the MUSIM gaps.

# NW-A’s requirement

One of the remaining issues in NW-A’s requirement is when UE performs a measurement without gap which is partially overlapping with the gap but fully overlapping with the union of the NW-A’s gap and MUSIM gaps as follow.

RAN4 #110b Issue 1-1-2: Scenarios for the case where the MO to be measured without MG have to be measured in the associated MG Proposals - P1: When UE performs a measurement without gap which is partially overlapping with the MG but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG. (Apple oppo Ericsson vivo Huawei) - P1-1: RAN4 shall clarify which gap to be used for this measurement, especially when association between MO and MG is not supported (UE doesn’t support R17 concurrent gaps). (Apple)

To resolve this issue, we suggest to follow the conclusion from Rel-17 concurrent gaps. For example, in Figure 1 below, UE had to perform the measurement within the MG1. The cell identification requirement will be max(600ms, 5* MGRP)* CSSFwithin_gap,i.

Figure 1. SMTCs are fully overlapping with the union of the gaps

Proposal 3: When UE performs a measurement without gap which is partially overlapping with the MG but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: The UE which supports MUSIM feature shall support at least one MUSIM gap pattern within a subset of MUSIM gap patterns.

Proposal 2: UE shall know the preferred MUSIM gap patterns from NW before UE requesting the MUSIM gaps.

Proposal 3: When UE performs a measurement without gap which is partially overlapping with the MG but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG.

- References

- R4-2406435, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo