3GPP TSG-RAN WG4 Meeting #111 	R4-2408340

# Fukuoka City, Fukuoka, Japan,  May 20th  – 24th  2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR NTN FR2-1 SAN demodulation and FR1 PUSCH DM-RS bundling

Agenda item:	7.16.8.1

Document for:	Information

# Introduction

As per the WF in RAN4#110bis meeting [1], new channel model and some open parameters for NTN FR2 were agreed, and aggregation level for NTN FR1 PUSCH DM-RS bundling was also agreed.

In this contribution, we share new results for NTN FR2 SAN demodulation and FR1 PUSCH DM-RS bundling.

# Simulation result

  - NTN FR2 PUSCH CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-1200 Low |
| MCS |  | MCS 2 and MCS12 in Table 1 |
| SCS |  | 120kHz |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 | 70 % | 2 | pos1 | -2.9 | -0.2 |
|  |  |  |  |  | 12 | pos1 | 6.2 | 8.9 |
|  | 2 | Normal | NTN-TDLC5-1200 | 70 % | 2 | pos1 | -6.3 | -3.6 |
|  |  |  |  |  | 12 | pos1 | -2.8 | 5.5 |

  - NTN FR2 PUSCH DFT-s-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-1200 Low |
| MCS |  | MCS 2 in Table 1 |
| SCS |  | 120kHz |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 | 70 % | 2 | pos1 | -2.7 | 0 |
|  | 2 | Normal | NTN-TDLC5-1200 | 70 % | 2 | pos1 | -6.1 | -3.4 |

  - NTN FR2 PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-1200 Low |
| MCS |  | MCS 5 in Table 3 |
| SCS |  | 120kHz |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 1% BLER |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 | 1% | G-FR2-A3A-7 | Pos1 | -13 | -10.3 |
|  | 2 | Normal | NTN-TDLC5-1200 | 1% | G-FR2-A3A-7 | Pos1 | -16.5 | -13.8 |

  - NTN FR2 Normal PUCCH

    - PUCCH format 0

Table 2.4.1-1:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |
| Test metric | DTX to ACK <1% ACK missed <1% |

Table 2.4.1-2: simulation results for PUCCH format 0, 120 kHz SCS and 50 MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | ideal | impairment |
| 1 | 1 | NTN-TDLC5-1200 Low | 2.2 | 4.9 |
|  | 2 | NTN-TDLC5-1200 Low | -1.2 | 1.5 |

    - PUCCH format 1

Table 2.4.2-1:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | DTX to ACK<1% NACK to ACK <0.1% ACK missed <1% |

Table 2.4.2-2: Simulation results for PUCCH format 1, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NACK to ACK |  | ACK miss |  |
|  |  |  |  | ideal | impairment | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | -4.8 | -2.1 | -3.8 | -1.1 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | -9.2 | -6.5 | -8.2 | -5.5 |

    - PUCCH format 2

Table 2.4.3-1   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK <1% ACK missed <1% |

Table 2.4.3-2: Simulation results for PUCCH format 2, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | 2.8 | 5.5 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | -2.5 | 0.2 |

Table 2.4.3-3:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER<1% |

Table 2.4.3-4: Simulation results for PUCCH format 2, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | 1 | 3.7 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | -4.6 | -1.9 |

    - PUCCH format 3

Table 2.4.4-1:  Test parameters of PUCCH format 3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Test metric | BLER <1% |

Table 2.4.4-1: Simulation results for PUCCH format 3, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | Additioan DM-RS configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -0.9 | 1.8 |
|  |  |  |  | Additional DM-RS | -1.1 | 1.6 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -4.5 | -1.8 |
|  |  |  |  | Additional DM-RS | -5.4 | -2.7 |

    - PUCCH format 4

Table 2.4.5-1:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER<1% |

Table 2.4.5-2: Simulation results for PUCCH format 4, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | Additioan DM-RS configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ideal | impairment |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -0.6 | 2.1 |
|  |  |  |  | Additional DM-RS | -1 | 1.7 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -4.4 | -1.7 |
|  |  |  |  | Additional DM-RS | -5.2 | -2.5 |

  - NTN FR2 PRACH

| Burst format | SCS (kHz) |  | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- | --- |
| B4, C2 | 120 |  | 69 | 0 | 0 |

Time error tolerance: 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | NTN-TDLC5 |
| B4, C2 | 120 | 0.07 us | 0.13us |

Table 2.5-1 Simulation results for PRACH miss detection for 120 kHz SCS

| SCS (kHz) | Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format B4 |  | Burst format C2 |  |
|  |  |  |  |  | ideal | impairment | ideal | impairment |
| 120 | 1 | 1 | AWGN | 0 | -16.6 | -13.9 | -11.7 | -9 |
|  |  |  | NTN-TDLC5-1200 | 3000 Hz | -8 | -5.3 | -6 | -3.3 |
|  | 1 | 2 | AWGN | 0 | -19.1 | -16.4 | -14.3 | -11.6 |
|  |  |  | NTN-TDLC5-1200 | 3000 Hz | -13.9 | -11.2 | -11.3 | -8.6 |

  - NTN FR1 PUSCH with DM-RS bundling

Table 2.6-1: Test parameters for testing PUSCH with DM-RS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example TDD UL-DL pattern (Note 1) |  | FDD |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 2) | 0, 0, 0, 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| pusch-TimeDomainWindowLength |  | 4 for 15kHz SCS 8 for 30kHz SCS |
| Frequency domain resource assignment | RB assignment | 6 RBs in the middle of test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Table 2.6-2: Simulation results for PUSCH with 70% of maximum throughput, MCS 4, 6 PRBs channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | Aggregation Factor | Mapping Type | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Ideal | Impair |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -3.74 | -1.44 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -3.77 | -1.47 |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -7.09 | -4.79 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -7.10 | -4.80 |

Table 2.6-3: Simulation results for PUSCH with 70% of maximum throughput, MCS 4, 6 PRBs channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | Aggregation Factor | Mapping Type | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Ideal | Impair |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -6.41 | -4.11 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -6.40 | -4.10 |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -9.35 | -7.05 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -9.34 | -7.04 |

# Conclusions

Simulation results for Rel-18 NR NTN FR2 SAN and NTN FR1 PUSCH DM-RS bundling demodulation results are delivered.

# References

[1]	R4-2406024, “WF on NR_NTN_enh_SAN_UE_demod”, Huawei, HiSilicon, RAN4#110bis.

[2]	R4-2405146, “Simulation assumption on demodulation requirements for NR NTN enhancements”,  Huawei, HiSilicon, RAN4#110bis.

[3]	R4-2405543, “Simulation results for NR NTN enhancement SAN demodulation requirements”, Ericsson, RAN4#110bis.