# 3GPP TSG-RAN WG4 Meeting # 111                                    R4-2408351

# Fukuoka, Japan, May. 20th – 24th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On R19 PC1.5 Intra-band contiguous and non-contiguous UL CA

# Agenda item:	10.1.1.2.1

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, the new WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to TN high power UE for CA are listed as below:

High power UE (HPUE) for CA in terrestrial network (TN) - Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO - Example band combinations: - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO - Example band combinations: - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA - Focus on the SAR solution - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE - Identify and update the requirements if necessary - Only PC3 is considered for LTE FDD in EN-DC - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified - Introduce the signaling to support the above objectives, if needed. - Consider release independency, if needed

It can be seen that there are three following topics:

1. PC1.5 for NR TDD intra-band UL CA (2Tx)

2. Handheld and/or FWA UE related HPUE band combination

3: Increasing UE transmission power limit

In last meeting, some agreements/open issues are included in the WF[2]. In this contribution, we give some further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA.

# 2	Discussion

- The agreements related to 2Tx PC1.5 for NR TDD intra-band UL contiguous and non-contiguous CA in the WF[2] are extracted in the following for convenience.

# General aspect ## 1.1 New proposed BC configurations for intra-band CA which are not included in the WI objectives Agreement based online and further clarification in AH meeting: - For the lower order power class cases (PC2 and PC3) of same PC1.5 band combination - Have the Rel-19 basket WIs to cover the necessary requirements - FFS on whether to have independent basket WIs to cover the requirements in RAN plenary - For the new UL intra-band CA configurations for PC1.5 - Have the Rel-19 basket WIs to cover the necessary requirements - FFS on whether to have independent basket WIs to cover the requirements in RAN plenary - Discuss the requirements for those new UL intra-band CA configurations for PC1.5 after the general requirements are finalized in this WI ## 1.2 SAR solution Way forward: FFS on following options SAR solution with consideration the P-MPR is applicable. - Option 1: Duty-cycle solution is not considered for any HPUE scenarios in this WI - Option 2：Duty-cycle solution is considered for all HPUE scenarios in this WI - Option 3：Duty-cycle solution is considered for all HPUE scenarios in this WI except for “increasing UE transmission power limit” - Option 4: Duty-cycle solution is considered for limited scenarios, further study which scenarios to consider # 2. PC1.5 for intra-band contiguous and non-contiguous UL CA NOTE 1: Intra-band contiguous UL CA includes cases with and without UL MIMO, Intra-band non-contiguous UL CA includes only w/o UL MIMO NOTE 2: Unless otherwise stated, the issues discussed below are applicable for both contiguous and non-contiguous CA ## 2.1 Assumed UE architectures for MPR/A-MPR evaluation - Option 1: (Xiaomi, Samsung, Huawei) Architecture; Description; Indicated capability; Support UL MIMO; Applicable cases; Note. #1; 2x26 dBm PA + 2 LO with 100MHz BW; dualPA-Architecture; No; C/NC CA; No Frequency Separation limitation. #2; 2x26 dBm PA + 1 LO with 200MHz BW; TxD; Yes; C/NC CA; Frequency Separation ≤ 200MHz For NC CA, FFS whether BW Gap size＜CC1 +CC2 CBW. NOTE: support UL MIMO here doesn’t mean UE has to support the feature, it depends on UE capability reporting - Option 2: (Apple, vivo, Skyworks, Meta) Architecture; Description; Indicated capability; Support UL MIMO; Applicable cases; Note. #1; 2x26 dBm PA + 2 LO with 100MHz BW; dualPA-Architecture; No; NC CA; No Frequency Separation limitation. #2; 2x26 dBm PA + 1 LO with 200MHz BW; TxD; Yes; Contiguous CA. Way forward: Both options are considered for next meeting discussion, FFS on down selection of UE architectures - Prioritize UL contiguous CA ## 2.2 General consideration for MPR/PCMAX/PCMAX,c Way forward: - For PC1.5, intra-band contiguous ULCA with UL MIMO can share the MPR requirements of intra-band contiguous ULCA without UL MIMO and with dual Tx (TxD) - Study and if necessary, specify the MPR requirements for both small and large form factor UEs, i.e. minimum antenna isolation = 10 or 20dB - For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, consider PCMAX,,c limitation (i.e., 26dBm) for each component carrier, which have impact on MOP, PCMAX tolerance and MPR/A-MPR evaluation - FFS For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10)) - Other options are not precluded ## 2.3 MOP tolerance Agreement: +2/-3 dB, in addition adding note into MOP table to reflect PC1.5 is achieved by dualPA-Architecture and/or TxD.  - PCMAX tolerance Way forward: The following values as starting point - TLOW=3dB - THIGH=2dB ## 2.5 ACLR Agreement: NR ALCR as 31dB, UTRA ALCR not needed for the example combos

It can be clearly seen that no Rx RF requirements are impacted.

MPR/A-MPR

In current specification, the MPR requirements for both UE indicating IE dualPA-Architecture and UE not indicating IE dualPA-Architecture are defined for PC2 and PC3 intra-band contiguous CA and intra-band non-contiguous CA. For PC2 TDD intra-band CA, there are already commercial 26dBm PAs in the market, which means there exists several architectures can be used for TDD intra-band CA, such as 1Tx with one 26dBm PA(200MHz, cc1+cc2), 2Tx with two 2*23dBm(100MHz per cc) or 2Tx with 23+26dBm(per cc) and so on.

However, for PC1.5 TDD intra-band CA band combination, 2PA (i.e. 2*26dBm) shall be used due to curretly there is no commercial 29dBm single PA, as mentioned by many companies in last meeting. Thus even for PC1.5 single carrier, 2*26dBm PA via TxD is the implementation. As discussed in company paper [4], PC1.5 single carirer via 2*26dBm must be supported for PC1.5 TDD intra-band C/NC CA, but meanwile it should also consider the separation BW and frequency gap limitation for NC CA.

For PC1.5 intra-band C CA, due to the 29dBm must be achieved by 2*26dBm, and also power sharing between the two carriers should also be taken into account, it is suggested that only to consider the architecture of 2x26 dBm PA + 1 LO with 200MHz BW.

For PC1.5 intra-band NC CA. In our understanding, either ‘2x26 dBm PA + 1 LO with 200MHz BW’ or  ‘2x26 dBm PA + 2LO with 100MHz BW’ could be feasible. However, to consider more flexibility for the separation BW and frequency gap, only consider ‘2x26 dBm PA + 2LO with 100MHz BW’ would be fine.

Proposal 1:

-To adopt ‘dualPA-Architecture(2x26 dBm PA + 2 LO with 100MHz BW)’ for PC1.5 intra-band NC CA

-To adopt ‘2x26 dBm PA + 1 LO with 200MHz BW’ for PC1.5 intra-band C CA

For A-MPR, it is related to the additional emission mask requirements, signalled by CA_NS_XX for intra-band contiguous CA and CA_NC_NS_XX for intra-band non-contiguous CA. In current spec, CA_NS_04 and CA_NC_NS_04 are used to for PC2/PC3 n41C and n41(2A), respectively. In addition, CA_NC_NS_55/57 are used for CA_n77(2A) for different frequency range associated with CA_NC_NS_01, but there are no CA_NS_XX defined for CA_n77C. Moreover, there are no CA_NS_XX or CA_NC_NS_XX value for n78 intra-band CA. All of these can be applied to PC1.5 intra-band CA.

- Observation 1:  For n78, there are no CA_NS_XX or CA_NC_NS_XX values for intra-band CA, and for n77, general CA_NC_NS_01 is used for intra-band non-contiguous CA but no CA_NS_XX value defined for intra-band contiguous CA. For n41, CA_NS_04 and CA_NC_NS_04 are used to for PC2/PC3 n41C and n41(2A),

Proposal 2: No A-MPR requirements are defined for PC1.5 n77/n78 intra-band CA, while the A-MPR requirements are needed to be revisited for PC1.5 n41 intra-band CA.

- SAR solution

For the SAR issue, from specification aspect we think the P-MPR solution is the baseline solution, similar with other HPUE topics. On top of the P-MPR approach, for PC2 TDD band/intra-band CA, another way, a.k.a. duty cycle scheme is introduced in the specification.

According to the following general descriptions for intra-band contiguous and non-contiguous CA in sub-clause 4.2:

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PCell and SCells for NR SA.

The above text are described in power class agnostic manner, so it is nature to be applicable for PC1.5 intra-band contiguous and non-contiguous CA. Due to the component carries in the UL intra-band CA are synchronized, it is feasible reuse the capability for single carrier case.

- Proposal 3: For duty cycle based SAR solution, reuse the PC1.5 single CC capability for PC1.5 TDD UL intra-band contiguous and non-contiguous CA.

Pcmax requirements/telorance

- For Pcmax requirements, it is related to the duty cycle based SAR solutions when the preconditions are not meet which cause the maximum output power fallback from 29dBm to PC2/PC3, due to only the descriptions of ΔPPowerClass,CA = 3 with maximum fallback from 26dBm to 23dBm are included in the current spec, so both ΔPPowerClass=3dB and ΔPPowerClass=6dB associated with the descriptions related to duty cycle are needed to be defined.

- Proposal 4: Pcmax requirement update with ΔPPowerClass =3/6 associated with the descriptions related to duty cycle based SAR solution are needed for PC1.5 TDD UL intra-band contiguous and non-contiguous CA

- For the Pcmax tolerance, it is unclear for the TLOW/THIGH values associated with the Pcmax range in the WF. To be more clear, the TLOW/THIGH values for Pcmax range should be [23, 29]dBm, i.e. upscoping the range from [23, 26]dBm to [23, 29]dBm

- Proposal 5: To be more clear, the PCMAX tolerance for uplink intra-band contiguous/non-contiguous CA are:

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 < PCMAX ≤ 29 | 3 | 2 |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

# 3 Conclusion

In this contribution, we give further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA. The proposals and conclusions are:

Proposal 1:

To adopt ‘dualPA-Architecture(2x26 dBm PA + 2 LO with 100MHz BW)’ for PC1.5 intra-band NC CA

To adopt ‘2x26 dBm PA + 1 LO with 200MHz BW’ for PC1.5 intra-band C CA

- Observation 1:  For n78, there are no CA_NS_XX or CA_NC_NS_XX values for intra-band CA, and for n77, general CA_NC_NS_01 is used for intra-band non-contiguous CA but no CA_NS_XX value defined for intra-band contiguous CA. For n41, CA_NS_04 and CA_NC_NS_04 are used to for PC2/PC3 n41C and n41(2A),

Proposal 2: No A-MPR requirements are defined for PC1.5 n77/n78 intra-band CA, while the A-MPR requirements are needed to be revisited for PC1.5 n41 intra-band CA.

- Proposal 3: For duty cycle based SAR solution, reuse the PC1.5 single CC capability for PC1.5 TDD UL intra-band contiguous and non-contiguous CA.

- Proposal 4: Pcmax requirement update with ΔPPowerClass =3/6 associated with the descriptions related to duty cycle based SAR solution are needed for PC1.5 TDD UL intra-band contiguous and non-contiguous CA

- Proposal 5: To be more clear, the PCMAX tolerance for uplink intra-band contiguous/non-contiguous CA are:

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 < PCMAX ≤ 29 | 3 | 2 |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

# 4	Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

[2] R4-2406583, WF on HPUE UE for UL CA and EN-DC, Samsung

[3] R4-2405226, Discussion on R19 FR1 TN HPUE requirements, ZTE

[4] R4-2404901, On architecture aspects for PC1p5 intra-band ULCA support, Skyworks Solutions Inc.