# 3GPP TSG-RAN WG4 Meeting # 111                               R4-2408361

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

# Title: 	Discussion on UE Capability/UE behavior and network signaling for Type 4a Type 4b EN-DC/NR-CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	10.7.2.2

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, A new work item on support of intra-band non-collocated EN-DC/NR-CA was approved[1].

The core part objectives of the new WID is as below.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

This contribution shares our views about type 4a and type 4b.

- Discussion

Some agreements were reached in RAN4 #110bis meeting and some issues need further discussion [4].

< Issue 2-3-1: New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) > Way Forward: - Continue further discussion on the following two options in the next meeting. - Option 1 - Add new UE capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) support indication. - Option 2 - Reuse intraBandNR-CA-non-collocated-r18 and maxNumberMIMO-LayersPDSCH=4 for FR1 NR intra-band CA and interBandMRDC-WithOverlapDL-Bands-r16 and maxNumberMIMO-LayersPDSCH=4 for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands. < Issue 2-3-3: Whether to support Type 2 capabilities by UE having Type 4 capability > Way Forward: - Continue further discussion on the following proposal in the next meeting. - If Type-4 capability is indicated, Type-2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. < Issue 2-4-1: New NW Signaling to switch between Type 1 and Type 4 > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on NW signalling/UE behavior in the next meeting. < Issue 2-4-2: UE behavior between Type 1, Type 2 and Type 4 with new NW Signaling > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on NW signalling/UE behavior in the next meeting. < Issue 2-4-3: When to inform RAN2 the demand on new UE capability(s) and new NW signaling(s) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on NW signalling/UE behavior in the next meeting.

The maxNumberMIMO-LayersPDSCH=4 capability for band n77 and n78 is mandatory and cannot be used for distinguish UE capability between type 2 and type 4a/4b

Proposal 1. To introduce new UE capability signaling for type 4a/4b.

With regard to the type 2 capability of a UE supporting type 4a/4b, a reasonable assumption is that the type 2 capability is supported by default if the type 4a/4b capability is reported.

The architectures of type 2 UE and type 4a/4b are shown in figure 2-1.

Figure 2-1a Architecture of type 2

Figure 2-1b Architecture of type 4a(left) and type 4b(right)

We can know from figure 2-1 that the architectures between type 2 and type 4a/4b are similar. The only difference between them is that type 4a/4b UE has more Rx antenna than type 2 UE. Therefore, we believe that UE supporting type4a/4b can also support type2.

Proposal 2. UE that supports type 4a/4b capability supports type 2 by default.

With regard to type switching between type 1 and type 4a/4b, the type switching between type 2 and type 1 can be a reference and a relevant agreement was reached last meeting.

- UE having Type 4 capability support the following capabilities (Set 1) when operating in non-collocated scenarios. - Maximum 4 MIMO layers on each CC - Maximum power imbalance of 25dB - UE having Type 4 capability support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios. - Maximum 8 MIMO layers on each CC - It may be optionally supported - Maximum PSD imbalance of 6dB

From the above, we can find that type 4a/4b UE may operate in Set 1 or Set 2. There is a chance to configure a type 2 capability UE in type 1 transmission. Therefore a signaling may be needed to ensure a unified understanding about the operation is achieved. For example, a new intra-band CA NW signaling (nonCollocatedTypeNR-CA) is introduced to configure the transmission for a type 2 UE. Similar to the type 2 UE discussion, new Rel-19 BS signaling is needed for type 4a/4b intra-band CA and inter-band ENDC to give NW a chance for to configure/inform a UE in co-located operation.

Proposal 3. New Rel-19 BS signaling is needed for type 4a/4b intra-band CA and inter-band ENDC.

The following lists the agreed UE type related signaling which are used for UE type reporting by UE and UE type indicating by BS.

interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements.

requirementTypeIndication-r18 Indicates whether the UE supports network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16. The UE supports this feature shall also indicate support of interBandMRDC-WithOverlapDL-Bands-r16.

intraBandNR-CA-non-collocated-r18 Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. This capability is only supported for band n77/n78.

nonCollocatedTypeMRDC-r18 ENUMERATED { type1 } OPTIONAL, -- Need R nonCollocatedTypeNR-CA-r18 ENUMERATED { type1 } OPTIONAL, -- Need R

When type 4a/4b is introduced, the signaling and UE behaviour need to be discussed. Table 2-2 summarizes the UE types for combinations of different UE capability and BS configuration signaling for NRCA.Table 2-1 summarizes Rel-18/19 UE capability reporting and BS configuration signaling for NRCA.

Table 2-1 UE behaviour for Rel-19 non-collocated intra-band NR CA

| Index | UE indicates type 4a/4b? | UE indicates type 2? | Rel-19 NW signaling provided? | UE type |
| --- | --- | --- | --- | --- |
| 1 | × | × | × | Type 1 |
| 2 | × | ○ | × | Type 2 |
| 3 | ○ | × | × | Type 4a/4b |
| 4 | ○ | × | ○ | Type 1 |
| 5 | ○ | ○ | × | Type 4a/4b? |
| 6 | ○ | ○ | ○ | Type 1? |

In our understanding, the similar approaches as UE behaviour for Rel-18 non-collocated intra-band NR CA can be applied to R19 non-collocated intra-band NR CA. For Rel-19 non-collocated intra-band NR CA, we think the UE behaviour types including type 2 and type 4a/4b are needed to be taken into account.

For the case of both Type 2 UE and Type 4a/4b signalings are reported, in our understanding, it is not expected that UE reports both two signalings, since we think a UE supports type 4a/4b capability can support type 2 by default. Nevertheless, it depends on UE itself and it may not proper to mandate UE not to report both signalings. We assume the RF requirements are the same for Type 2 and Type 4a/4b UE, then the difference for Type 2 and Type 4 are the supported MIMO layer. For a UE report Type 4 signaling, it means it can support up to 4 layer per cc, which can also incorporate the Type 2. Hence, it seems it is enough to only consider Type 4 UE requirements.

With the above consideration, we have the following proposals:

Observation 1. It is not expected that a UE reports both type 2 capability and type 4a/4b capability.

Proposal 4:

When Rel-19 NW signaling is provided, Type 1 UE requirements are applied

When Rel-19 NW signaling is not provided:

- Type 2 UE requirements are applied if only Type 2 UE capability is reported.

- Type 4 UE requirements are applied if only Type 4 UE capability is reported.

- Type 4 UE requirements are applied if both Type 2 UE and Type 4 capabilities are reported.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Observation 1. It is not expected that a UE reports both type 2 capability and type 4a/4b capability.

Proposal 1. To introduce new UE capability signaling for type 4a/4b.

Proposal 2. UE that supports type 4a/4b capability supports type 2 by default.

Proposal 3. New Rel-19 BS signaling is needed for type 4a/4b intra-band CA and inter-band ENDC.

Proposal 4:

When Rel-19 NW signaling is provided, Type 1 UE requirements are applied

When Rel-19 NW signaling is not provided:

- Type 2 UE requirements are applied if only Type 2 UE capability is reported.

- Type 4 UE requirements are applied if only Type 4 UE capability is reported.

- Type 4 UE requirements are applied if both Type 2 UE and Type 4 capabilities are reported.

# 4	Reference

- RP-240838, Rel-19 WID_Intra-band non-collocated EN-DC and NR-CA, Apple

- RP-240101, Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Release 19, KDDI, etc.

- R4-2316951, LS on frequency separation for Type 2 UE NR-CA PDSCH demodulation requirements, TSG RAN WG4

- R4-2406627, WF on NonCol_intraB_ENDC_NR_CA, KDDI