3GPP TSG-RAN4 Meeting #111	R4-2408402

Fukuoka, Japan, 20th – 24th May, 2024

Title:	Text proposal on duplexing mode for TR 38.922

Source:	Qualcomm Incorporated

Agenda item:	10.3.1

Document for:	Approval

# Introduction

In this document we provide our text proposals for the duplexing mode for the 4400-4800 MHz and 7125-8400 MHz frequency ranges following WF captured in R4-2406615.

******************Begin changed sections******************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-240787: “Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”.

[3]	R4-2400333: “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”.

[4]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”, V18.5.0, April 2024.

[5]	3GPP TR 38.858: “Study on Evolution of NR Duplex Operation”, V18.1.0, April 2024.

[6]	RP-240789, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

# 4	4400 - 4800 MHz frequency range

## 4.1	General parameters

### 4.1.1	Duplex mode

The 4400 – 4800 MHz frequency range is part of an 3GPP-defined n79 band (4400 – 5000 MHz), which according to [4], is a TDD band. TDD is widely used in commercial NR deployments, where time domain resources are split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency, and reduced capacity. 3GPP has been investigating since Rel-16 the dynamic allocation of downlink and uplink resources in TDD deployments. An enhancement of TDD duplex operation via allowing the simultaneous existence of downlink and uplink subband at the gNB side within a TDD carrier in a conventional TDD band (subband non-overlapping full duplex), was studied in Rel-18 [5]. Currently 3GPP is specifying in Rel-19 SBFD operation at the gNB side within a TDD carrier, gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation, and requirements for SBFD operation at gNB [6]. To provide timely response to WP5D regarding the requested RF parameters, RAN4 considered TDD as a current candidate duplexing for the 4400 – 4800 MHz frequency range. The requirements and conformance aspects for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].

# 5	7125 - 8400 MHz frequency range

## 5.1	General parameters

### 5.1.1	Duplex mode

There is no defined 3GPP band for the 7125 - 8400 MHz frequency range, however, it is adjacent to existing TDD band n104 (6425 – 7125 MHz). Similar to the 4400 – 4800 MHz frequency range, SBFD can be a candidate duplexing method for this frequency range. TDD is widely used in commercial NR deployments, where time domain resources are split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency, and reduced capacity.To provide timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a candidate duplexing for the 7125 – 8400 MHz frequency range. The requirements and conformance aspects for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].

# 6	14800 - 15350 MHz frequency range

## 6.2	General parameters

### 6.2.1	Duplex mode

There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 7125 - 8400 MHz frequency range, SBFD can be a candidate duplexing method for this frequency range. To provide timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a candidate duplexing for the 14800 - 15350 MHz frequency range. The requirements and conformance aspects for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].

******************End changed sections******************