3GPP TSG-RAN4 Meeting #111	R4-2408439

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Qualcomm Incorporated

Title: 	view on R19 RRM enhancements WID

Agenda item:	10.6.2

Document for:	Discussion

# Introduction

In this paper, we are presenting view for R19 RRM enhancement WI.

# Discussion

## Topic #2-1 L3 measurement delay reduction by optimizing Rx BSF for UE supporting multiple-Rx simultaneous reception

Issue 2-1-1: applicable scenarios of L3 measurement delay reduction by optimizing Rx BSF WF for next meeting: - FFS: - L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided the target frequency layer to be measured is the only one in the single FR2-1 band and UE is configured [with one FR2-1 band]. - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The condition here means in which case/condition/use-case/mode UE can apply the L3 measurement delay reduction by optimizing Rx BSF. WF for next meeting: FFS：Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features - FFS: UE’s mobility status, e.g., whether HST is precluded or not - FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - FFS: Power consumption issue - FFS: UE has prior knowledge on the cell to be measured - FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - FFS: Other conditions: cell-centre UE or cell-edge UE - FFS: Simultaneous operation between multi-Rx simultaneous reception (L1) and L3 measurement delay reduction by optimizing Rx BSF. Issue 2-1-3: UE measurement procedures to use L3 measurement delay reduction by optimizing Rx BSF WF for next meeting: FFS：Scenarios to use L3 measurement delay reduction by optimizing Rx BSF: - FFS: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: Handover - FFS: PSCell addition - FFS: RRC Re-establishment/RRC Connection Release with Redirection - FFS: SCell activation - FFS: SCG activation - FFS: CGI identification - FFS: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Before moving forward the detail discussion, we first want to clarify the definition and implication of “For UE supporting multiple-Rx simultaneous reception”. From our understanding, this means UE supporting ‘simultaneous reception of multiple SSBs from different directions inside a SMTC window. But it does not mean UE can process multiple SSBs in parallel.

Proposal: RAN4 first agree the definition and implication of UE supporting multiple-RX simultaneous reception for L3 delay enhancement.

- “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

Since L3 RRM measurement is widely used in different scenarios, depends on scenario, different conditions may be required to perform enhanced L3 RRM measurement. Since the WI objectives are very broad, RAN4 shall discuss the target scenario and required conditions as a package. Without consideration of target scenario, same conditions are applied for different scenario.

Observation : L3 RRM measurement is widely used in different scenarios. Depending on target scenario, fast beam sweeping may or may not be applied and different conditions may be required. Without considering target scenario and conditions as a package, same conditions are applied for different scenarios.

Proposal : RAN4 shall consider target scenario with applicable conditions as a package to evaluate whether/how UE to perform enhanced L3 measurement with fast beam sweeping.

One example for neighbor cell measurement, UE is not always required to perform enhanced L3 measurement. If UE perform enhanced L3 measurements, it means UE just can fast measurement and do better than legacy L3 measurement requirement. It does not mean, new requirement is required. Moreover, we don’t see any different NW behavior by getting information that UE can perform enhanced L3 measurement at certain point because SMTC is periodically configured.  NW may not need to know whether UE is performing fast L3 measurement.

Observation: If fast beam sweeping is applied to L3 measurement, the fast beam sweeping may not always be needed for UE who perform neighbor cell L3 measurement.

Observation : L3 measurement is performed within SMTC and SMTC is periodically configured, NW behavior may not be changed by getting information that UE is performing enhanced L3 measurement.

Proposal : RAN4 discuss whether NW needs to know whether UE is performing fast beam sweeping.

Since L3 measurement is performed within SMTC, L3 measurement with fast beam sweeping and L1 measurement with fast beam sweeping is independent. RAN4 may consider similar mechanism or concept from R18 multi-rx for L1 measurement enhancement. But supporting multi-rx for fast L1 beamsweeping does not mean UE to support L3 measurement with fast beam sweeping. Moreover, L3 fast beam sweeping may need different condition and activation mechanism compared to L1 fast beam sweeping.

Observation: L1 fast beam sweeping and L3 fast beam sweeping are independent feature. Supporting R18 L1 fast beam sweeping does not mean UE can perform L3 fast beam sweeping.

Proposal: R19 L3 measurement enhancement for both fast beam sweeping and CSSF optimization is independent to R18 multi-rx feature. There are no features to be considered from previous release.

## Topic #2-2 L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization Agreement: Rel-19 discussion on CSSF optimization starts for the case UE is not capable of Rel-18 multi-Rx simulaeous reception, further discuss whether/how it can be applied to the case UE is capable of Rel-18 multi-Rx simulaeous reception but work in single-Rx currently. Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement: - Only consider CSSF outside MG case. Proposal for further discussion: - Scenarios to use L3 measurement delay reduction by optimizing CSSF: - SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter and TSSB_measurement_period_inter

From our understanding, CSSF optimization WI is about solutions for how to reduce number of carriers when scale target frequency measurement. One possible approach is to group among intra-frequency layers and UE to prioritize one frequency layer in each groups.

Observation: CSSF optimization is about how to reduce CSSF value.

Proposal: NW can group among intra-frequency layers from configured MO. UE can prioritize to measure one frequency layer in each group.

# Conclusions

Proposal: RAN4 first agree the definition and implication of UE supporting multiple-RX simultaneous reception for L3 delay enhancement.

- “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

Observation : L3 RRM measurement is widely used in different scenarios. Depending on target scenario, fast beam sweeping may or may not be applied and different conditions may be required.

Proposal : RAN4 shall consider target scenario with applicable conditions as a package to evaluate whether/how UE to perform enhanced L3 measurement with fast beam sweeping.

Observation: If fast beam sweeping is applied to L3 measurement, the fast beam sweeping may not always be needed for UE who perform neighbor cell L3 measurement.

Observation : L3 measurement is performed within SMTC and SMTC is periodically configured, NW behavior may not be changed by getting information that UE is performing enhanced L3 measurement.

Proposal : RAN4 discuss whether NW needs to know whether UE is performing fast beam sweeping.

Observation: L1 fast beam sweeping and L3 fast beam sweeping are independent feature. Supporting R18 L1 fast beam sweeping does not mean UE can perform L3 fast beam sweeping.

Proposal: R19 L3 measurement enhancement for both fast beam sweeping and CSSF optimization is independent to R18 multi-rx feature. There are no features to be considered from previous release.

Observation: CSSF optimization is about how to reduce CSSF value.

Proposal: NW can group among intra-frequency layers from configured MO. UE can prioritize to measure one frequency layer in each group.

# References

[1] RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo, RAN #103

[2] R4-2406392	Way Forward of [110bis][230]NR_RRM_Ph5, Apple, RAN4 #110bis