3GPP TSG-RAN WG4 Meeting #111	R4-2408459

Fukuoka, Japan, 20th May – 24th May 2024

Source:	Ericsson, Telstra, Skyworks, Murata, Qualcomm

Title:	TP for 37.718-11-11 to include UL CA_n3B

Agenda item:			6.9.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-11-11 to add to include UL CA_n3B based on the agreement as captured in R4-2406681, “WF on MSD test point selection for intra-band uplink CA” at the RAN4 110bis meeting.

# 2. Text Proposal

---Start of changes---

## 5.x	CA_2DL_n3B_2UL_n3B

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3 | n3 |

Table 5.x.1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n3B | - | 5 | 15, 20, 25, 30 |  |  |  |  | 60 | 0 |
|  |  | 10 | 10, 15, 20, 25, 30 |  |  |  |  |  |  |
|  |  | 15, 20, 25, 30 | 5, 10, 15, 20, 25, 30 |  |  |  |  |  |  |
|  | CA_n3B | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  | 40 | 1 |
|  | - | See n3 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  |  | 75 | 4 and 5 |

### 5.x.2	UE maximum output power for Intra-band contiguous CA

Table 5.x.2.1-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3B |  |  |  |  | 23 | +2/-2 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

### 5.x.3	UE additional maximum output power reduction for CA

No additional A-MPR is needed for this configuration.

### 5.x.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation with shared spectrum channel access as identified in Table 5.2-1.

### 5.x.5	Reference sensitivity power level for Intra-band contiguous CA

The MSD value used is based on an agreement at RAN4 #110-bis inputs based on the inputs from Qualcomm, Murata and Skyworks.

Table 5.x.5-1: Intra-band contiguous CA uplink configuration for reference sensitivity.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3B5 | 15/15 | 20MHz + 20MHz | 25 (RBSTART = 0) | 25 (RBSTART = 81) | 25.1 | 12.4 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 5: Applicable only to BCS 1 |  |  |  |  |  |  |  |

### 5.x.6	In-band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

### 5.x.7	Out-of-band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

### 5.x.8	Narrow band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

### ---End of changes---