3GPP TSG-RAN WG4 Meeting #111	R4-2408499

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source:	Samsung

Title:	Simulation results on Rel-18 MIMO UE demodulation performance and CSI requirements

Agenda item:	7.19.3.1

Document for:	Discussion

- Introduction

In the RAN plenary RAN#98 meeting, the WID [1] for Rel-18 NR MIMO evolution for downlink and uplink has been approved. RAN4 work plan for this topic including RF, RRM and performance part was endorsed in [2]. In RAN4#108bis meeting, initial discussion on UE demodulation performance and CSI reporting parts started and overall work scope endorsed [3]. In RAN4#109 meeting, there are some agreements on general scope, initial simulation assumptions for the feasibility study of Type II Doppler, Type II CJT codebooks and Rel-18 DMRS [4]. In last RAN4#110bis meeting, further discussion and agreements on the test assumptions are achieved [6]. In this contribution, we provide our simulation results and observations on Type II Doppler, Type II CJT codebooks and Rel-18 DMRS separately.

- TypeII-Doppler-r18

For the TypeII-Doppler-r18 codebook introduced in Rel-18, which is UE-side prediction, UE predicts the channel at furture time slots based on past CSI-RS measurements and reports the future PMIs calculated from the predicted channel. Simulation cases for FR1 FDD in this section are based on Figure 2-1 in which CSI request periodicity is 10ms. Simulation cases for FR1 TDD in this section are based on Figure 2-2 in which CSI request periodicity is 10ms.

Figure 2-1 FR1 FDD CSI configuration for TypeII-Doppler-r18 simulation

Figure 2-2 FR1 TDD CSI configuration for TypeII-Doppler-r18 simulation

  - Simulation assumptions

For the further simulation assumptions of feasibility study, we get some agreements in RAN4#109, RAN4#110 and RAN4#110bis meetings, these agreements are summarized into below Table 2-1.

Table 2-1. Simulation assumptions of TypeII-Doppler-r18

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | FDD: 10 TDD: 40 |
| Subcarrier spacing |  | kHz | FDD: 15 TDD: 40 |
| Duplex Mode |  |  | FDD and TDD |
| Propagation channel |  |  | For FR1 FDD: TDLA30-20 TDLA30-30 For FR1 TDD: TDLA30-10 TDLA30-20 TDLA30-30 |
| Antenna configuration |  |  | XP Medium 16 x 2 XP Medium 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
|  | number of NZP CSI-RS resources (K) |  | 4 |
|  | separation between two consecutive CSI-RS resources |  | 2 for FDD 1 for TDD |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| CSI request |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration for Rel-16 cases | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Codebook configuration for Rel-18 cases | Codebook Type |  | typeII-doppler-r18 |
|  | paramCombination-Doppler-r18 |  | 7 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-Doppler-r18) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-Doppler-RI-Restriction-r18) |  | 0010 |
|  | N4 |  | 1 |
|  | DD unit duration | slot | 1 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8ms for FDD 7ms for TDD |
| Maximum number of HARQ transmission |  |  | 4 |
| MCS |  |  | MCS13 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4) for FDD or slot#(n-7) for TDD, this reported PMI cannot be applied at the gNB downlink before slot#(n+4) for FDD or slot#(n+7) for TDD. Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

In RAN4#110 meeting, we get the agreement to define PMI reporting requirements with ‘typeII-Doppler-r18’, and the test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler  is X % of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook. At the same time, companies still have interest in the performance comparison between ‘typeII-Doppler-r18’ and Rel-16 Type II codebook. Therefore, in this document, we focus on comparing the throughput performance of Rel-16 Type II codebook, ‘typeII-Doppler-r18’ codebook and random precoding based on Single Panel Type I codebook.

  - Simulation results for FR1 FDD

PMI reporting performance comparison with 2Rx for cases with Rel-16 Type II codebook, ‘typeII-Doppler-r18’ codebook and random precoding based on Single Panel Type I codebook are as Figure 2-3 – Figure 2-4.

Figure 2-3. 2Rx performance comparison with Doppler 20Hz

Figure 2-4. 2Rx performance comparison with Doppler 30Hz

From the performance comparsion results on Figure 2-3 and Figure 2-4, compared with Rel-16 Type II codebook, there are some benefits of ‘typeII-Doppler-r18’ codebook with N4=1 cases with Doppler 20Hz, while no performance gain even worse with Doppler 30Hz.

For 4Rx cases, we run more simulation cases to check the performance of ‘typeII-Doppler-r18’ codebook with N4 equals 1, even the performance comparison with random precoding based on Single Panel Type I codebook. The simulation results are on Figure 2-5 – Figure 2-6. From these results, we have similar observations as 2Rx cases.

Figure 2-5. 4Rx performance comparison with Doppler 20Hz

Figure 2-6. 4Rx performance comparison with Doppler 30Hz

Observation 1: For both FR1 FDD 2Rx and 4Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook with N4=1 vs Rel-16 Type II codebook, obvious benefit is observed for TDLA30-20 cases.

Observation 2: For both FR1 FDD 2Rx and 4Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook with N4=1 vs Rel-16 Type II codebook, no performance gain could be observed for TDLA30-30 cases.

In order to display the performance gain more clearly, we summarize the performance gain on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook in below Table 2-2 and Table 2-3. In these tables, Test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler is 90 %  of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook.

Table 2-2. 90 % SNR point for FR1 FDD, TDLA30-20 and TDLA30-30 with MCS13 cases

| case |  | SNRtypeII-r16 | SNRtypeII-doppler-r18 N4=1 |
| --- | --- | --- | --- |
| 2Rx | TDLA30-20 | 6.9 | 5.9 |
|  | TDLA30-30 | 6.8 | 7.0 |
| 4Rx | TDLA30-20 | 1.0 | 0.4 |
|  | TDLA30-30 | 1.0 | 1.0 |

Table 2-3. Test metric γ1 for FR1 FDD, TDLA30-20 and TDLA30-30 with MCS13 cases

| case |  | Rel-16 Type II | Rel-18 Doppler N4=1 |
| --- | --- | --- | --- |
| 2Rx | TDLA30-20 | 2.35 | 2.69 |
|  | TDLA30-30 | 2.37 | 2.30 |
| 4Rx | TDLA30-20 | 3.38 | 3.94 |
|  | TDLA30-30 | 3.33 | 3.31 |

Observation 3: For Doppler 20Hz with MCS13 FR1 FDD case, the 90% SNR working point of Rel-18 Doppler codebook could get about 1dB gain over Rel-16 codebook for 2Rx cases, and about 0.6dB gain for 4Rx cases.

Observation 4: For Doppler 20Hz with MCS13 FR1 FDD case, the test metric γ1 on 90% SNR working point of Rel-18 Doppler codebook is about 2.69 for 2Rx case, and 3.94 for 4Rx case.

  - Simulation results for FR1 TDD

PMI reporting performance comparison with 2Rx for cases with Rel-16 Type II codebook, ‘typeII-Doppler-r18’ codebook and random precoding based on Single Panel Type I codebook are as Figure 2-7 – Figure 2-9.

Figure 2-7. 2Rx performance comparison with Doppler 10Hz

Figure 2-8. 2Rx performance comparison with Doppler 20Hz

Figure 2-9. 2Rx performance comparison with Doppler 30Hz

From the performance comparsion results on Figure 2-7 - Figure 2-9, compared with Rel-16 Type II codebook, there are benefits of ‘typeII-Doppler-r18’ codebook with N4=1 cases with Doppler 10Hz, 20Hz, 30Hz, and the most obvious benefits is achieved at Doppler 30Hz.

For 4Rx cases, we run more simulation cases to check the performance of ‘typeII-Doppler-r18’ codebook with N4 equals 1, even the performance comparison with random precoding based on Single Panel Type I codebook. The simulation results are on Figure 2-10 – Figure 2-12. From these results, we have similar observations as 2Rx cases.

Figure 2-10. 4Rx performance comparison with Doppler 10Hz

Figure 2-11. 4Rx performance comparison with Doppler 20Hz

Figure 2-12. 4Rx performance comparison with Doppler 30Hz

Observation 5: For both FR1 TDD 2Rx and 4Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook with N4=1 vs Rel-16 Type II codebook, obvious benefits could be observed for TDLA30-10, TDLA30-20, TDLA30-30 cases, and the most obvious benefit is achieved at Doppler 30Hz.

In order to display the performance gain more clearly, we summarize the performance gain on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook in below Table 2-4 and Table 2-5. In these tables, Test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler is 90 %  of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook.

Table 2-4. 90 % SNR point for FR1 TDD, TDLA30-10, TDLA30-20 and TDLA30-30 with MCS13 cases

| case |  | SNRtypeII-r16 | SNRtypeII-doppler-r18 N4=1 | SNR diff |
| --- | --- | --- | --- | --- |
| 2Rx | TDLA30-10 | -0.5 | -0.9 | 0.4 |
|  | TDLA30-20 | 1.3 | -0.1 | 1.4 |
|  | TDLA30-30 | 3.9 | 1.5 | 2.4 |
| 4Rx | TDLA30-10 | -3.3 | -3.6 | 0.3 |
|  | TDLA30-20 | -2.3 | -3.1 | 0.8 |
|  | TDLA30-30 | -0.9 | -2.3 | 1.4 |

Table 2-5. Test metric γ1 for FR1 TDD, TDLA30-10, TDLA30-20 and TDLA30-30 with MCS13 cases

| case |  | Rel-16 Type II | Rel-18 Doppler N4=1 |
| --- | --- | --- | --- |
| 2Rx | TDLA30-10 | 20.74 | 26.45 |
|  | TDLA30-20 | 7.86 | 16.42 |
|  | TDLA30-30 | 3.50 | 7.34 |
| 4Rx | TDLA30-10 | 22.50 | 28.91 |
|  | TDLA30-20 | 10.47 | 17.26 |
|  | TDLA30-30 | 5.42 | 10.12 |

Observation 6: For Doppler 10Hz, 20Hz and 30Hz with MCS13 FR1 TDD cases, the 90% SNR working point of Rel-18 Doppler codebook could get about 0.4dB, 1.4dB, 2.4dB gain over Rel-16 codebook separately for 2Rx cases, and about 0.3dB, 0.8dB, 1.4dB gain for 4Rx cases.

For the Test metric γ1 in Table 2-5, although the higher test metric could be reached at lower Doppler, the high test metric mainly comes from the low SNR working point.

Observation 7: For Doppler 30Hz with MCS13 FR1 TDD case, the test metric γ1 on 90% SNR working point of Rel-18 Doppler codebook is about 7.3 for 2Rx case, and 10.1 for 4Rx case.

- TypeII-CJT-r18

For CJT schemes which is introduced in Rel-18, all the PDSCH/DMRS ports are jointly transmitted from multiple TRPs and signals from multiple TRPs are combined coherently, hence multiple geographically separated TRPs or RRHs for CJT are assumed to be well synchronized in time and frequency as well as the phase and amplitude of their antenna arrays are calibrated. In RAN4#110 meeting, we agreed to introduce PMI reporting requirements for ‘typeII-CJT-r18’ codebook (FR1 FDD only) based on below test metric [5].

Issue 1-1-5: clarify test metric for PMI reporting requirements with ‘typeII-CJT-r18’ codebook Agreement: - For the PMI reporting requirements with typeII-CJT-r18, define the test metric as γcjt=ttypeII-CJTtrnd, where ttypeII-CJT is [90] % of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook per TRP.

  - Simulation assumptions

According to the agreements of simulation assumptions for Type II CJT codebook in sub-topic 2-2 [4][5] and sub-topic 1-2 [6], the simulation assumptions could be summarized as below Table 3-1.

Table 3-1. Simulation assumptions of TypeII-CJT-r18

| Parameter |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Duplex mode |  |  |  |  | FDD |  |  |
| Bandwidth |  |  |  | MHz | 10 |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |
| Active DL BWP index |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) High XP 8 x 4 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| CSI-RS resources K |  |  |  |  | 2 |  |  |
| Number of TRPs NTRP |  |  |  |  | 2 |  |  |
| restrictedCMR-Selection |  |  |  |  | ON |  |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |  |
|  | k0 |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000, 1001 |  |  |
|  | TCI state |  |  |  | TCI State #1 |  |  |
|  | DMRS Type |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  |  |
|  |  |  | QCL Type |  | Type A |  |  |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A |  |  |
|  |  |  | QCL Type |  | N/A |  |  |
| Resource allocation |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  | 4 |  |  |
|  |  | CDM Type |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  | Not configured |  |  |
| Sub-band Size |  |  |  | RB | 4 |  |  |
| csi-ReportingBand |  |  |  |  | 1111111111111 |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  | typeII-CJT-r18 |  |  |
|  |  | CodebookMode |  |  | Mode2 |  |  |
|  |  | paramCombination-CJT-L-r18 |  |  | 7 {4, 4} |  |  |
|  |  | paramCombination-CJT-r18 |  |  | 4 (pν=14and18, β=12) |  |  |
|  |  | R(numberOfPMI-SubbandsPerCQI-Subband-CJT-r18) |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  | 0x 7FF FFFF FFFF FFFF FFFF |  |  |
|  |  | RI Restriction |  |  | 00000010 |  |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  | R.PDSCH.1-6.3 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with subband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs, where both TRxPs will send the same PDSCH data Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. For each TRP, use random type I SP PMI precoding independently. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-7)], this reported PMI cannot be applied at the gNB downlink before slot#[(n+7)]. Note 4: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |  |  |  |

  - Simulation results

paramCombination-CJT-L-r18 = 7 and paramCombination-CJT-r18 = 4

In last RAN4#110bis meeting, companies agreed to use MCS 20 and Rank 2 for CJT codebook simulation assumptions. For paramCombination-CJT-L-r18 = 7 {4, 4} and paramCombination-CJT-r18 =4 (pν=14and18, β=12), with MCS 20, the absolute TP of following TypeII-CJT-r18 codebook vs random Type I codebook and the relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook are as Figure 3-1.

Figure 3-1. performance of following TypeII-CJT-r18 codebook vs random Type I codebook with MCS20

Accordingly, the SNRtypeII-CJT  at Z% of the maximum TP by using TypeII-CJT-r18 and related test metric  γ3=ttypeII-CJTtrnd are summarized in below Table 3-2, where ttypeII-CJT is 90% of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook.

Table 3-2 SNRtypeII-CJT and test metric  γ3 of paramCombination-CJT-L-r18 = 7 and paramCombination-CJT-r18 = 4

| MCS | Rx | SNRtypeII-CJT | γ3 |
| --- | --- | --- | --- |
| MCS20 | 2 Rx | 9.1 | 6.56 |
|  | 4 Rx | 6.8 | 5.7 |

Observation 8: For FR1 FDD cases with MCS20 and Rank 2, the test metric γ1 on 90% SNR working point of Rel-18 CJT codebook is about 6.56 for 2Rx case, and 5.7 for 4Rx case.

- Rel-18 DMRS

In RAN4#108bis meeting, it is agreed to introduce PDSCH demodulation requirements for Rel-18 enhanced DMRS for SU-MIMO scenario [3], and in RAN4#109, RAN4#110 and RAN4#110bis meetings, some agreements about simulation assumptions are achieved [4][5][6]. In last RAN4#110bis meeting, companies agreed to use new simulation results for Rel-18 DMRS Rank 4 cases and reuse requirements of Test 2-1 of Clause 5.2.2.1.1, 5.2.2.2.1 for Rel-18 DMRS Rank 2 cases, we’d like to provide our simulation results of Rel-18 DMRS for alignment and results with impairment.

  - Simulation assumptions

Table 4-1. Simulation assumptions of R18 enhanced DMRS

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping inter-leaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports indexes |  | {1008, 1009} for Rank 2 {1000, 1001, 1008, 1009} for Rank 4 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | FDD: 4 for 2Rx FDD: 8 for 4Rx TDD: 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | FDD: 2 TDD: Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

  - Simulation results

For FDD and TDD with 2Rx and 4Rx, the PDSCH demodulation performance results are as Figure 4-1 – Figure 4-4.

Figure 4-1. FDD PDSCH demodulation performance with 2Rx Rank 2

Figure 4-2. TDD PDSCH demodulation performance with 2Rx Rank 2

Figure 4-3. FDD PDSCH demodulation performance with 4Rx Rank 4

Figure 4-4. TDD PDSCH demodulation performance with 4Rx Rank 4

The SNR point without impairment at 70% fraction of maximum throughput could be summarized as below Table 4-1 and Table 4-2. And the results considering impariment for Rank 4 cases are as below Table 4-3.

Table 4-1 SNR point at 70% fraction of maximum throughput for FDD cases

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | R15 DMRS | R18 DMRS |
| 1-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 15.8 | 15.8 |
| 1-2 | R.PDSCH.1-2.4 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 11.6 | 11.5 |

Table 4-2: SNR point at 70% fraction of maximum throughput for TDD cases

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | SNR(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | R15 DMRS | R18 DMRS |
| 1-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 15.4 | 15.4 |
| 1-2 | R.PDSCH.2-2.4 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 11.9 | 12.0 |

Table 4-3: Rank 4 Simulation results for SNR at 70% @maximum throughput with impairment

| Cases | SNR (dB) |
| --- | --- |
| FDD Rank4 | 13.5 |
| TDD Rank4 | 14.0 |

- Conclusion

In this contribution, we provide our simulation results and observations on UE demodulation performance and CSI requirements for Rel-18 NR MIMO enhancements for downlink.

The observations could be summarized as:

Observation 1: For both FR1 FDD 2Rx and 4Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook with N4=1 vs Rel-16 Type II codebook, obvious benefit is observed for TDLA30-20 cases.

Observation 2: For both FR1 FDD 2Rx and 4Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook with N4=1 vs Rel-16 Type II codebook, no performance gain could be observed for TDLA30-30 cases.

Observation 3: For Doppler 20Hz with MCS13 FR1 FDD case, the 90% SNR working point of Rel-18 Doppler codebook could get about 1dB gain over Rel-16 codebook for 2Rx cases, and about 0.6dB gain for 4Rx cases.

Observation 4: For Doppler 20Hz with MCS13 FR1 FDD case, the test metric γ1 on 90% SNR working point of Rel-18 Doppler codebook is about 2.69 for 2Rx case, and 3.94 for 4Rx case.

Observation 5: For both FR1 TDD 2Rx and 4Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook with N4=1 vs Rel-16 Type II codebook, obvious benefits could be observed for TDLA30-10, TDLA30-20, TDLA30-30 cases, and the most obvious benefit is achieved at Doppler 30Hz.

Observation 6: For Doppler 10Hz, 20Hz and 30Hz with MCS13 FR1 TDD cases, the 90% SNR working point of Rel-18 Doppler codebook could get about 0.4dB, 1.4dB, 2.4dB gain over Rel-16 codebook separately for 2Rx cases, and about 0.3dB, 0.8dB, 1.4dB gain for 4Rx cases.

Observation 7: For Doppler 30Hz with MCS13 FR1 TDD case, the test metric γ1 on 90% SNR working point of Rel-18 Doppler codebook is about 7.3 for 2Rx case, and 10.1 for 4Rx case.

Observation 8: For FR1 FDD cases with MCS20 and Rank 2, the test metric γ1 on 90% SNR working point of Rel-18 CJT codebook is about 6.56 for 2Rx case, and 5.7 for 4Rx case.

- Reference

[1] RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung

[2] R4-2301929, Work plan for Rel-18 WI on MIMO evolution, Samsung

[3] R4-2316920, WF on NR_MIMO_evo_DL_UL_demod, Samsung

[4] R4-2321141, WF on [109][327] NR_MIMO_evo_DL_UL_demod, Samsung

[5] R4-2402868, WF on [110][323] NR_MIMO_evo_DL_UL_demod, Samsung

[6] R4-2406111, WF on [110bis][328] NR_MIMO_evo_DL_UL_demod, Samsung