3GPP TSG RAN WG4 Meeting #111	R4-2408524

Fukuoka City, Japan, May 20th – May 24th, 2024

Agenda Item:	7.14.2.2

Source: 	Ericsson

Title:	RRM core requirements for Rel-18 Further NR mobility enhancement

Document for:	Discussion

# Introduction

From RAN4 #110-bis meeting WF on Rel-18 mobility enhancement performances part are listed in [1]. In this paper we provide our view on certain open issues.

Background: WID objective #7

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including:​

- Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and​.

- Definition of corresponding RRM requirements [RAN4]; and​

- If necessary, based on RAN4 outcome, definition of corresponding signaling support [RAN2].​

Note 5: RAN4 will coordinate in due course with RAN2 to start the work.​

Note 6: R4-2220415 serves as baseline for future work in RAN4.​

Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.​

# Discussion

## Improvement on Scell/SCG setup delay

Issue 2-2: test configuration/procedure for solution based on existing measurement for validity check. - Tentative Agreement: - Introduce test case to verify FG 39-x1 and 39-x2. - Candidate option: use different power levels before X window starts plus some time margin. Issue 2-3: test scope of solution based on existing measurement for validity check. - FFS: - Proposal 1: Define test cases to support Rel-18 early measurement reporting of both EMR and cell-reselection measurements. (Nokia) - Proposal 2: Define also a test case where X is not configured to verify the accuracy of the reported measurements. (Nokia)

The main difference between eEMR and IMR are:

- eEMR measurement duration is upon network configuration which is controlled by T331 timer with value specified in (measIdleDuration) which starting from RRC_release and ending can be either before or after paging.

- IMR measurement duration is starting from RRC_Release and ending in paging.

The validity duration is the X value counting back from msg1, this is the same procedure for both eEMR and IMR.

Even though eEMR and IMR are two independent UE capabilities 39-8 and 39-9, with smart configuration of the T331 timer, we still see the possibility to have one test case to verify the UE behavior for both UE capabilities.

Proposal 1: Introduce one single test case for Rel-18 Idle/Inactive measurement for CA/DC setup enhancement to cover both UE capability 39-8 eEMR (Measurement validation based on EMR measurement) and 39-9 IMR (Measurement validation based on non-EMR measurement).

Regarding how to verify the validity duration, the existing Rel-16 EMR test case, TS38.133 A.6.6.9 Idle Mode CA/DC Measurements, already has certain procedure can be reused. The difference is that cell reselection measurement and procedure was also considered for the Rel-16 EMR test case.

##### …… ##### A.6.6.9.1.2 Test Requirements The UE behaviour during time durations T2 and T3 shall be as follows: During the time period T2 the UE is in Idle mode and the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4. At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2. The UE shall send early measurement report to the PCell. After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in Section 10.1.4B for SS-RSRP and in Section 10.1.8B for SS-RSRQ and test ends. The rate of correct events observed during repeated tests shall be at least 90%. ……

Due to the reason that validity timer is up to network configuration, the UE behavior for eEMR and IMR can be different so in total there are 4 scenarios for Rel-18 to verify:

| Scenario | measReselectionValidityDuration configured? | measIdleValidityDuration configured? |
| --- | --- | --- |
| 1 | YES | YES |
| 2 | YES | NO |
| 3 | NO | YES |
| 4 | NO | NO |

One thing we noticed is that to be able to verify both IMR and eEMR capability in one case, the configuration of how many neighboring cells is depended on how many capabilities UE reports, when UE reports both eEMR and IMR, always need 3 cells to be configured regardless of the validity timer configuration.

When only one of the eEMR and IMR capabilities is claimed, then only need to configure 2 cells.

Observation 1: The different UE capabilities of 39-8 and 39-9 can be verified in one case with configure 2 neibouring cell.

For scenario 1: when both IMR and eEMR capabilities are claimed and both validity timer is configured, the correct UE behavior is to have measurement based on the network configured carrier, then during certain time period the signal level of neighboring cells are changed, then the UE shall observe, and report correct RSRP value.

For scenario 2:

- If UE claim both capabilities, verify the UE report behavior based on the signal level change without performing cell-reselection. The eEMR report behavior shall follow the legacy Rel-16 UE behavior.

- If only IMR capability is claimed, the UE report behavior based on signal level change without performing cell-reselection.

- If only eEMR capability is claimed, even though this is a false network configuration, the correct UE behavior shall follow the legacy Rel-16 UE behavior.

For scenario 3: It should be rather similar test procedure as scenario 2, only difference is to guarantee the T331 timer duration end after the paging message.

- If UE claim both capabilities, verify the eEMR behavior on UE report behavior based on the signal level change without performing cell-reselection. The correct IMR UE behavior shall be UE report all the measurement results higher than accuracy requirement on the configured neighboring cell during the Idle period.

- If only IMR capability is claimed, this is a false configuration however the UE shall report all the measurement results higher than accuracy requirement on the configured neighboring cell during the Idle period.

- If only eEMR capability is claimed, verify the eEMR behavior on UE report behavior based on the signal level change without performing cell-reselection.

For scenario 4: When both validity timer are not configured, it will depend on the capability UE reports.

- If UE claim both capabilities:  the correct behavior shall be UE report all the measurement results higher than accuracy requirement on the configured neighboring cell#1 during the Idle period. The neighboring cell #2 shall be reported follow the legacy EMR behavior.

- If only IMR capability is claimed, the correct UE behavior shall be UE report all the measurement results higher than accuracy requirement on the configured neighboring cell#1 during the Idle period.

- If only eEMR capability is claimed, The UE shall report measurement results follow the legacy EMR behavior.

Proposal 2: The different UE capabilities of 39-8 and 39-9 can be verified in one case by configuring 2 neibouring cells. When both 39-8 and 39-9 capabilities are claimed, configure 3 cells (1 serving and 2 neighboring cells) for the test case, when only one of the capabilities is claimed, configure 2 cells (1 serving and 1 neighboring cell) for the test case.

To verify the UE behavior of validity check, we think the change target neighboring cell signal level from legacy can be reused here. However, from implementation of the test case point of view, as the X duration is counting back from UE initiated msg1 which is very hard for network to control the timeline for changing the power level of the neighboring cell.

As the paging message to msg1 usually have very small-time difference typically in ms range, the easiest way to implement the test case is to base the validity duration x counting back from paging message as the paging message is up to network and can be implemented in the test environment. Details of our proposal can be seen from figure 1.

Figure 1

Test case timeline defined as:

T1: UE is in connected mode in serving cell during T1. The moment when UE receives RRC_release message defined as the starting point of T2.

T2: During T2, the 2 neighboring cells become detectable however no cell reselection is being performed. Signal level of both neighboring cell is set to level 1. T2 duration shall allow UE at least finishes one measurement period for two neighboring cells.

T3: The starting point of T3 shall be X seconds before TE send the paging message to UE. X seconds value: X= measIdleValidityDuration or measReselectionValidityDuration. During T3, the signal level of the 2 enamouring cells shall be set to level 2.  The time when TE receiving the msg 1 from the UE defines as the starting point of T4.

T4: UE shall send measurement report based on different scenarios defined and the time when TE receives UE measurement report defines as the ending point of T4.

Proposal 3: Newly introduced test case shall have 4 scenarios with 2 or 3 cells configured based on UE capability. Validity timer can be verified based on timeline with changing the neighboring cell signal levels.

# Summary and Conclusion

In this contribution we have provided our views on performance part of Mobility enhancement. The following proposals are made:

Observation 1: The different UE capabilities of 39-8 and 39-9 can be verified in one case with configure 2 neibouring cell.

Proposal 1: Introduce one single test case for Rel-18 Idle/Inactive measurement for CA/DC setup enhancement to cover both UE capability 39-8 eEMR (Measurement validation based on EMR measurement) and 39-9 IMR (Measurement validation based on non-EMR measurement).

Proposal 2: The different UE capabilities of 39-8 and 39-9 can be verified in one case with configure 2 neibouring cell. When both 39-8 and 39-9 capabilities are claimed, configure 3 cells (1 serving and 2 neighboring cells) for the test case, when only one of the capabilities is claimed, configure 2 cells (1 serving and 1 neighboring cell) for the test case.

Proposal 3: Newly introduced test case shall have 4 scenarios with 2 or 3 cells configured based on UE capability. Validity timer can be verified based on timeline with changing the neighboring cell signal levels.

# References

- R4-2406346 “WF for RRM performance requirements of R18 Further NR mobility enhancement”, Apple

- R2-2401990 CR to TS38.3331 for eEMR and IMR, Nokia