3GPP TSG RAN WG4 Meeting #111		 R4-2408637

Fukuoka City, Fukuoka, Japan, 20th - 24th May, 2024

Agenda Item:	10.7.2.2

Source: 	MediaTek Inc.

Title: 	Continue discussion on intra-band non-collocated UE capability

Document for:	Discussion

1. Introduction

In RAN4 #107, WF [1] has been agreed and the options on how UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2 were down select with two options. We continue discuss them in this contribution.

2. Discussion

In previous RAN4 meeting, the WF[1] was agreed and issue left from last RAN4 meeting are copied below,

< Issue 2-3-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) >

Way Forward:

- Continue further discussion on the following two options in the next meeting.

  - Option 1

    - Add new UE capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) support indication.

  - Option 2

    - Reuse intraBandNR-CA-non-collocated-r18 and maxNumberMIMO-LayersPDSCH=4 for FR1 NR intra-band CA and interBandMRDC-WithOverlapDL-Bands-r16 and maxNumberMIMO-LayersPDSCH=4 for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands.

Issue 2-3-2: UE having Type 4 capabilities in non-collocated/collocated scenarios

Agreement:

- UE having Type 4 capability support the following capabilities (Set 1) when operating in non-collocated scenarios.

  - Maximum 4 MIMO layers on each CC

  - Maximum power imbalance of 25dB

- UE having Type 4 capability support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios.

  - Maximum 8 MIMO layers on each CC

    - It may be optionally supported

  - Maximum PSD imbalance of 6dB

In our view since type 4 UE  is new type of UE supporting 4-layer MIMO on each non-collocated CC, and in the following discussion, UE may optionally support 8 MIMO layers on each CC when operating in collocated scenarios. It is necessary for type 4 UE to report correct capability on whether it can support 8 MIMO layers on each CC in collocated scenarios. New UE capability is needed to make sure both UE and network have same understanding on DL MIMO layer configurations. With the reason, we propose option 1 in Issue 2-3-1.

Proposal 1: We propose add new UE capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) support indication. The new capability shall also include whether the type 4 UE can support 8 DL MIMO layer on each CC that is optional capability, up to UE implementation.

As previous RAN4 discussion, UE needs indication on collocation/non-collocation from network to avoid mis-alignment between UE and gNB. UE may change its configuration to type 1 UE accordingly. This will need legal time gap reserved for UE to handle such configuration change as well as protocol change including different timing alignment configurations.

Observation 1: UE need collocation/non-collocation indication from network to config itself for type 4 UE.

According to existing specs in 38.101-1/3, there are BS signalling indications “nonCollocatedTypeNR-CA-r18” and “nonCollocatedTypeMRDC-r18” when SCell is activated. For type 4a/4b UEs, same indicators can be re-used for BS to inform UE to configure corresponding receiver hardware settings.

Proposal 2: The existing BS signalling indications“nonCollocatedTypeNR-CA-r18” and “nonCollocatedTypeMRDC-r18” can be re-used to configure type 4a/4b UE’s DL receiving hardware configuration when SCell is activated.

3. Conclusion

Proposal 1: We propose add new UE capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) support indication. The new capability shall also include whether the type 4 UE can support 8 DL MIMO layer on each CC that is optional capability, up to UE implementation.

Observation 1: UE need collocation/non-collocation indication from network to config itself for type 4 UE.

Proposal 2: The existing BS signalling indications“nonCollocatedTypeNR-CA-r18” and “nonCollocatedTypeMRDC-r18” can be re-used to configure type 4a/4b UE’s DL receiving hardware configuration when SCell is activated.

Reference:

- R4-2406627 WF on NonCol_intraB_ENDC_NR_CA, RAN4#110-bis