3GPP TSG-RAN WG4 Meeting #111                                                           R4-2408742

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.11.2.2

Source:	Ericsson

Title:	On MOBD test case selection

Document for:	Discussion

# 1	Introduction

In this contribution, we provided our view on:

- Selection of test case with modulation order blind detection

- Left open issues related to the test case design

# 2	Discussion

## 2.1 Test case with modulation order blind detection

According to the WF [1], the test case for rank 1+1 with 2T2R should be down-selected from the following three candidates:

Test setting for when UE is not indicated Modulation order (DCI index 6 is indicated) - For Rank 1+1 with 2T2R, down-select among the following cases: - Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Case26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE - Case 20: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE (as priority for requirement definition)

From our simulation results [2], all three candidates have shown enough gain (>3dB). Our preference on the test case selection would be on Case 26 so that different MCS combinations can be covered comparing with the case without MOBD of similar assumptions.

Observation 1: All three candidates have shown enough gain (>3dB).

Proposal 1: Select case 26 for rank 1+1 with 2T2R scenario.

The test case for rank 2+2 with 4T4R (if introduced) should be down-selected from the following two candidates:

- Option 1A (Case 32): Orthogonal precoding, TDLA30-10, XP medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Option 1B (Case 31): Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

According to our observation, the result of Case 32 and Case 31 has shown 1.4dB and 1.1dB gain respectively. Our preference is to select Case 32 as there is more gain comparing with the result of Case 31 shown not only on ours but also on most other companies’ results.

Observation 2: The result of Case 32 and Case 31 has shown 1.4dB and 1.1dB gain respectively.

Proposal 2: Select case 32 for rank 2+2 with 4T4R scenario.

As for rank 1+1 with 2T4R, the test case is supposed to be down-selected from the following candidates:

- Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Case 29: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE - Case 23: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

From our simulation results [2], it’s observed that all candidates showed enough gain over the baseline receiver. From the assumption point of view, our preference is to select Case 29 as it differs from the one chosen for test without MOBD.

Observation 3: All candidates for rank 1+1 with 2T4R showed enough gain over the baseline receiver.

Proposal 3: Select Case 29 for rank 1+1 with 2T4R scenario.

## 2.2 Others

RRC assistant information configuration on the MCS table

For UEs not supporting modulation order blind detection, it’s still undetermined whether there is a need for the Network to send the MCS table. Followings are the candidate options:

RRC assistant information configuration on the MCS table - For UEs not supporting modulation order blind detection: - Option 1: No need for the network to inform such information to the UE - Option 2: Signalled regardless of whether the UE supports MO BD - Option 2A: 256QAM MCS Table - Option 2B: 64QAM MCS table

As the Network is not aware of the capability of doing modulation order blind detection, in the real deployment, it’s straightforward the Network will send the MCS table information to all UEs. While considering the test, there is no harm (no additional cost in the real Network) to inform such information to all UEs. To keep the testing consistency, we agree with option 2A. Thus, we propose the following:

Proposal 4: Signal the 256QAM MCS table regardless of the capability of MOBD.

Applicability rule

There was a discussion on whether to introduce applicability rule to skip tests with modulation order indicated. Candidate options are listed as follows:

- Option 1: Introduce applicability rule to skip tests with modulation order indicated for UEs capable of BD MO - Option 2: Do not introduce applicable rule skip tests with modulation order indicated

In our view, the tests with modulation order indicated can be skipped only if those tests are with the same parameter assumptions as that of the tests with MOBD. Because in such case, the requirement is expected to be the same, which means that the performance of using MOBD should not be worse than that of without MOBD. Thus, we propose the following:

Proposal 5: Introduce applicability rule to skip tests without MOBD only if those tests are with the same parameter assumptions as that of the tests with MOBD

# 3	Summary

In this contribution, we provided our view on MCS table configuration for both with and without modulation order blind detection scenarios. Our proposals are summarized as follows:

Observation 1: All three candidates have shown enough gain (>3dB).

Proposal 1: Select case 26 for rank 1+1 with 2T2R scenario.

Observation 2: The result of Case 32 and Case 31 has shown 1.4dB and 1.1dB gain respectively.

Proposal 2: Select case 32 for rank 2+2 with 4T4R scenario.

Observation 3: All candidates for rank 1+1 with 2T4R showed enough gain over the baseline receiver.

Proposal 3: Select Case 29 for rank 1+1 with 2T4R scenario.

Proposal 4: Signal the 256QAM MCS table regardless of the capability of MOBD.

Proposal 5: Introduce applicability rule to skip tests without MOBD only if those tests are with the same parameter assumptions as that of the tests with MOBD

# References

[1]	R4-2406114: WF on the advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #110bis, Changsha, China

[2]	R4-2408744: Simulation results for cases with MOBD, Ericsson, RAN4 #111, Fukuoka City, Fukuoka, Japan