3GPP TSG-RAN WG4 Meeting #111                                                            R4-2408744

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.11.2.2

Source:	Ericsson

Title:	Additional simulation results for cases with MOBD

Document for:	Discussion

# 1	Introduction

In this contribution, we provided our simulation results for being the reference of making further down-selection on the test cases for UE is not indicated Modulation order (DCI index 6 is indicated) [1].

# 2	Discussion

## 2.1 Simulation assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

Following assumptions are assumed:

- Maximum transmission bandwidth configuration for all UEs

- Assume all scheduled DMRS ports have same QCL assumptions.

- Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s)

- Same CDM group for rank 1+1, different CDM groups for rank 2+2

The following table 2.1-1 is showing the general parameter configurations, which are basically using what were agreed in the previous meetings.

Table 2.1-1 General parameter configurations

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Number |  |  | 1 | 1 |
| Channel Bandwidth/SCS |  | MHz/kHz | 10/15 for FDD |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13/17 | QPSK/16QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R, 4T4R with different correlations |  |
| CDM groups without data |  |  | Same for rank 1+1 / different for rank 2+2, 1+1+1 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | Orthogonal |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100 for 2T2R and 2T4R cases TDLA30-10 for 4T4R cases |  |

## 2.2 Simulation results

Table 2.2-1 FDD results for cases with modulation order blind detection

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | R-ML | E-IRC | Gain |
| Rank 1+1 with 2T2R |  |  |  |  |  |  |  |  |  |
| 20 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 14.1 | 18.9 | 5 |
| 20a | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA low | 13.2 | 15.7 | 3.3 |
| 26 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | 21.1 | N/A | Inf |
| Rank 1+1 with 2T4R |  |  |  |  |  |  |  |  |  |
| 23 | 10 / 15 | MCS 13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA medium | 12.2 | 18.5 | 6.2 |
| 29 | 10 / 15 | MCS 17 | 16QAM | TDLC300-100 | 1+1 | 2x4, ULA medium | 20.6 | 25.7 | 5.2 |
| 23a | 10 / 15 | MCS 13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA low | 3.8 | 4.6 | 1.0 |
| Rank 2+2 with 4T4R |  |  |  |  |  |  |  |  |  |
| 31 | 10 / 15 | MCS 13 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA low | 10.3 | 11.7 | 1.1 |
| 32 | 10 / 15 | MCS 13 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | 12.7 | 13.3 | 1.4 |

Table 2.2-2 TDD results for cases with modulation order blind detection

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | R-ML | E-IRC | Gain |
| 34 | 40 / 30 | MCS 13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 15.6 | 21.7 | 6.8 |
| 37 | 40 / 30 | MCS 13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA medium | 14.7 | 21.6 | 8.3 |
| 40 | 40 / 30 | MCS 17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | 22.7 | N/A | Inf |
| 43 | 40 / 30 | MCS 17 | 16QAM | TDLC300-100 | 1+1 | 2x4, ULA medium | 22.4 | N/A | Inf |
| 34a | 40 / 30 | MCS 13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA Low | 12.3 | 29.1 | 1.8 |
| 37a | 40 / 30 | MCS 13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA Low | 10.6 | 28.7 | 3.6 |
| 45 | 40 / 30 | MCS 13 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 12.6 | 13.9 | 1.1 |
| 46 | 40 / 30 | MCS 13 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | 13.8 | 15.9 | 1.9 |

# 3	Summary

In this contribution, we provided our simulation results for the test cases with MOBD for the reference to make further down-selection in this meeting.

# References

[1]	R4-2406114: WF on the advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #110bis, Changsha, China.