3GPP TSG-RAN WG4 Meeting #111                                                            R4-2408752

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.25.1

Source:	Ericsson

Title:	Simulation results for Rel-18 DSS enhancement demodulation

Document for:	Discussion

# 1	Introduction

In the last RAN4#110bis meeting, test cases for TDD were slightly modified. In this contribution, we provided our simulation results for newly agreed test case to assist the completion of defining requirement.

# 2	Discussion

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters

| Parameters | Values |  |
| --- | --- | --- |
| TDD UL-DL pattern | FDD | TDD |
|  | / | FR1.15-1 |
| SCS | 15 kHz |  |
| Bandwidth | 10 MHz |  |
| Channel model | TDLA30-10 and TDLC300-100 |  |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |  |
| CRS | 4 port CRS |  |
| DCI payload (excluding CRC) | 39 bits |  |
| Interleaving | Non-interleaved |  |
| Precoding | Precoder cycling per REG bundle |  |
| REG bundle size | 6 |  |
| Shift index | 0 |  |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |  |
| CORSET RB | 48 |  |
| AL | 4, 8 |  |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |  |

## 2.2 Simulation results

Table 2.2-1 FDD Simulation results

| Test case | SCS(KHz)/CBW (MHz) | Duplex | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR(dB) @1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | FDD | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -2.7 |
| 2 | 15/10 | FDD | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -3.1 |
| 3 | 15/10 | TDD | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | -3.2 |
| 4 | 15/10 | TDD | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | -2.8 |

# 3	Summary

In this contribution, we provided our simulation results based on the agreed test cases listed in the WF [1].

# References

[1]	R4-2406019: Way forward on [110bis][333] NR_DSS_enh, Ericsson, RAN4 #110bis, Changsha, China.