3GPP TSG-RAN WG4 Meeting #111	R4-2408775

Fukuoka, Japan, 20 May – 24 May 2024

Agenda Item:	7.21.2.1

Source:	Ericsson

Title:	Simulation results of UE demodulation requirements for RedCap enhancements

Document for:	Information

# 1	Introduction

RAN4#110bis agreed with the way forward on UE demodulation and CSI reporting requirements for RedCap enhancements (eRedCap) [1]. This contribution provides our updated simulation results.

# 2	PDSCH demodulation

## 2.1	Test cases for UE not supporting eRedCapNotReducedBB-BW-r18

Table 1	FDD/HD-FDD SCS=15kHz, 1Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput. (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 1.0 | 3.0 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 9.1 | 11.1 |
| MCS19 in Table 1 | 64QAM, 0.50 | Rank 1 | 12 | 9992 | TDLA30-10 | 13.4 | 15.4 |

Table 2	FDD/HD-FDD SCS=15kHz, 2Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput. (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | -3.1 | -1.1 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 4.8 | 6.8 |
| MCS19 in Table 1 | 64QAM, 0.50 | Rank 2 | 6 | 9480 | TDLA30-10 | 15.4 | 17.4 |

Table 3	TDD SCS=30kHz, 1Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 1.0 | 3.0 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 9.3 | 11.3 |
| MCS19 in Table 1 | 64QAM, 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 | 13.4 | 15.4 |

Table 4	TDD SCS=30kHz, 2Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | -2.9 | -0.9 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 5.1 | 7.1 |
| MCS19 in Table 1 | 64QAM, 0.50 | Rank 2 | 6 | 4736 | TDLA30-10 | 16.0 | 18.0 |

## 2.2	Test cases for UE supporting eRedCapNotReducedBB-BW-r18

Table 5	FDD/HD-FDD SCS=15kHz, 1Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 0.5 | 2.5 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 9.3 | 11.3 |

Table 6	FDD/HD-FDD SCS=15kHz, 2Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | -3.3 | -1.3 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 4.9 | 6.9 |

Table 7	TDD SCS=30kHz, 1Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 0.5 | 2.5 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 8.8 | 10.8 |

Table 8	TDD SCS=30kHz, 2Rx.

| Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput (Alignments) | SNR to achieve 70% of the maximum throughput (Impairments) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | -3.1 | -1.1 |
| MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 4.8 | 6.8 |

# References

- R4-2406052, “Way Forward for [110bis][330] NR_redcap_enh_demod”, Ericsson.