# 3GPP TSG- RAN WG4 Meeting # 111	R4-2408797

# Fukuoka, Japan, 20th–24th May, 2024

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

# Agenda Item:	10.10.2

# Document for:	Approval

- Introduction

In RAN#103 meeting, a new Work Item [1] was approved on NR channel BW less than 5MHz for FR1 Phase 2 with the following RAN4 objective. In this contribution, we want to share some further views on UE RF requirements for inter-band NR CA/DC with 3MHz CBW.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

- Discussion

  - 2.1 Power class for UL inter-band CA

- Issue 2-4: Power class for UL inter-band CA Agreement (Wed online discussion): - Define PC3 inter-band CA/DC combinations - FFS on whether to define PC1 inter-band CA/DC combinations

The above is way forward in last meeting [2]. We agreed to define PC3 inter-band CA/DC combinations. The following is UE power classes supported by NR band n100 and n101, it has been defined PC1 and PC3 for these two bands. Theoretically, it is possible to support PC1 inter-band CA/DC combinations. The UE PC1 requirements for band n100 and n101 single carrier are targeted for public safety scenario. Whether there is a request to support PC1 inter-band CA/DC combinations needs input from operators.

Proposal 1: Whether to support PC1 inter-band CA/DC combinations needs input from operators.

Table 2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n100 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |

  - 2.2 Signalling aspects

- Issue 2-6: Signalling aspects Agreement - FFS whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth

Firstly, when introducing 5G NR for FRMCS in band n100 of 2x5.6MHz FDD, it was specified that FRMCS and GSM-R will have to be in operation in parallel. In order to maintain the network quality level of GSM-R  while reducing the available bandwidth or number of GSM-R channels, only limited transmission bandwidth can be converted from GSM-R to FRMCS, and 12 RB transmission bandwidth for FRMCS would be possible. Hence, if we define a new CA/DC band combination of 3MHz in band n100 and 5MHz or 10MHz in band n101, 12 RB transmission bandwidth should be considered. Given that 5MHz bandwidth for n100 isn’t included in the band combination, 20 RB transmission bandwidth will be considered after RAN plenary agrees to add it in the WID.

Subsequently, in Rel-18, only additional sync raster was defined for single carrier of 12 RB transmission bandwidth. Moreover, it was established that the requirements for 12 RBs transmission bandwidth are identical to those for 15 RBs transmission bandwidth. In other words, RF requirements are agnostic of transmission bandwidth configurations for single carrier with 3MHz CBW. And in last meeting, we also achieved an agreement that define RF requirements agnostic of transmission bandwidth configurations for 3MHz CBW for the new new CA/DC band combination. From RF perspective, there is no necessity to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth. However, we have no idea whether RRM requirements will have difference for 12 and 15 RB transmission bandwidth. Whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth needs to wait for RRM requirements for CA.

Proposal 2: Whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth needs to wait for RRM requirements for CA.

  - 2.3 Reference sensitivity: MSD requirements

- Issue 2-12: Reference sensitivity: MSD requirements Agreement - FFS on MSD requirements

For inter-band CA_n100-n101, sensitivity degradation needs to be analyzed, and the harmonic and IMD products can be calculated by the table shown in Annex Table 5.1. As can be seen in the table, no harmonic and IMD products needs to be discussed. In addition, we adopt the approach of detecting potential cross-band isolation MSD for up to the ACLR5 range, which is proposed in [3]. It can be observed that there is no overlap of the band n100 ACLR range with the band n101 DL up to order 5 and there is enough rejection of the transmitter noise floor at band n101, so no cross-band MSD is needed. Similarly, there is no overlap of the band n101 ACLR range with the band n100 DL up to order 5 and there is enough rejection of the transmitter noise floor at band n100, so no cross-band MSD is required. Thus, we can conclude that no cross-band MSD is needed for inter-band CA_n100-n101.

Proposal 3: For inter-band CA_n100-n101, no MSD issue needs to be discussed.

- Conclusion

In this contribution, we want to share some views on UE RF requirements for inter-band NR CA/DC with 3MHz CBW and the proposals are made as following:

Proposal 1: Whether to support PC1 inter-band CA/DC combinations needs input from operators.

Proposal 2: Whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth needs to wait for RRM requirements for CA.

Proposal 3: For inter-band CA_n100-n101, no MSD issue needs to be discussed.

- Reference

RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo

R4-2406623, WF on RF requirements for less than 5MHz band combinations, Intel

R4-2404250, On cross-band isolation MSD analysis, Skyworks Solutions Inc., Nokia

- Annex

Table 5.1: Harmonic and IMD products analysis of CA_n100A-n101A with n100 and n101 UL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 874.4 | 880 | 1900 | 1910 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1020 | 1035.6 | 2774.4 | 2790 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 161.2 | 140 | 2920 | 2945.6 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3648.8 | 3670 | 4674.4 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2071.2 | 2040 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 713.2 | 740 | 4820 | 4855.6 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5548.8 | 5580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4523.2 | 4550 | 6574.4 | 6610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765.6 | 6720 | 1620 | 1587.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8474.4 | 8520 | 5397.6 | 5430 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3981.2 | 3940 | 1160 | 1196.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7448.8 | 7490 | 6423.2 | 6460 |

Table 5.2: Cross-band isolation analysis of CA_n100A-n101A with n100 and n101 UL

| Bands3 | n100 |  | n101 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 874.4 | 880 | 1900 | 1910 |
| fDL (MHz) | 919.4 | 925 | 1900 | 1910 |
| CBW (MHz)2 | 3 | 3 | 5 | 10 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 871.4 | 883 | 1890 | 1920 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 865.4 | 886 | 1880 |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 862.4 | 889 | 1870 |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 859.4 | 892 | 1860 |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 856.4 | 895 | 1850 |  |
| Analysis | There is no overlap of the band n100 ACLR range with the band n101 DL up to order 5 and there is enough rejection of the transmitter noise floor at band n101, so no cross-band MSD is needed. |  | There is no overlap of the band n101 ACLR range with the band n100 DL up to order 5 and there is enough rejection of the transmitter noise floor at band n100, so no cross-band MSD is needed. |  |