3GPP TSG-RAN WG4#111	R4-2408842

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	10.1.1.4.4

Source: 	Qualcomm France

Title: 	6Rx SRS IL Imbalance issue

Document for:	Approval

- Introduction

Analysis and considerations on SRS IL imbalance are provided in this contribution.

- WID description and WF in RAN4#110bis

WID [1] includes the following content for 6Rx:

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

General for all the areas

- Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible.

WF [2] agreed in RAN4#110bis says the following for SRS IL Imbalance

# Topic 4:	SRS IL imbalance issue

## Sub-topic 4-1:	General considerations for SRS IL imbalance issue

Way forward: Further discuss the topic of SRS IL imbalance issues with the following proposals used as a starting point. Whether a solution is agreeable, depends on further discussion of the following candidate solutions and/or identified issues.

-	Proposal 1: Study the issues of insertion loss imbalance across SRS ports.

-	Proposal 2: Further discuss SRS IL for 6RX type of devices taking into consideration:

-	SRS insertion loss requirements for 6RX UEs

-	SRS IL impact on performance

-	SRS insertion loss compensation

-	UE assistance on SRS insertion loss (power imbalance)

-	Others

-	Proposal 3: Companies should, independent of their view, provide constructive technical inputs on SRS TX power imbalance topic.

## Sub-topic 4-2:	Candidate solutions for the SRS IL imbalance issue

Way forward: Further discuss the following options considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

-	Option 1: Specify UE behaviour and requirements for scenarios, when UE has sufficient power to compensate the power imbalance (Case 2) and require UE to perform SRS IL compensation up to the maximum power capabilities.

-	Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX_L,f,c. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed. Per discussion during RAN4 meetings there seem to be no clear understanding whether all existing UEs do perform the SRS IL compensation under these conditions. So, it is recommended to consider specific requirements / conformance requirements in future to guarantee proper UE implementations.

-	Case 2 (near max Tx power): In this scenario the actual required SRS transmission power is higher than PCMAX_L,f,c, but is still below the max transmission power PCMAX_H,f,c. Based on the interpretation of TS 38.101-1 above, in this case the UE behaviour is undefined and UE may or may not perform SRS IL compensation.

-	Case 3 (max Tx power): In this scenario the actual required SRS transmission power is equal to PCMAX_H,f,c. (i.e. hypothetical scenario). In this case UE is not capable to perform any SRS IL compensation.

-	Option 2: At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

-	Option 3: Not specifying reporting of SRS IL offsets due to IL imbalance.

-	Option 4: IL imbalance reporting mechanism for SRS AS including both static reporting and dynamic reporting.

-	Static reporting is up to UE implementation, and UE needs to indicate the power compensation behaviour to NW if UE reports statically.

-	Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

-	Option 5: IL imbalance reporting mechanism for SRS AS including both the configured maximum output power per SRS resource and the power headroom per SRS resource.

-	The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission.

-	Other options are not precluded.

Consider some clarifications according to the RAN1 and RAN4 specs.

-	RAN1 specification on power control doesn’t require or imply that UE shall apply the same PCMAX for all AS-SRS transmission occasions

-	Under the minimum requirement ∆TRxSRS defined in RAN4 specification, it is up to UE implementation whether to apply exact value for corresponding AS-SRS transmission occasions.

- Discussion

Sub-topic 4: SRS Imbalance issue

This topic was discussed at length during 8Rx both in RAN4 and in RAN1. The first submissions seen in RAN4#110bis basically repeat the same discussion with exactly same challenges due to quite opposite views. Even the views among the initial supporters on how to do the indication differs a lot.

Ideally, providing gNB the exact information of the transmitted power from each SRS port (or the said delta) should help gNB in channel estimation. The whole topic started from a simple indication, but then during the work when more analysis was done in both in 3GPP and internally, a lot of uncertainties rose beyond how to do the indication if the whole concept of SRS imbalance indication is agreed.

For instance if/when UE fully compensates the additional SRS IL then it transmits the SRS at uncompromised power meaning there is no delta to report or that the delta is “0dB”. However, even in this case there is a lot of uncertainty on the actual transmitted power and ideally UE should report that it transmitted SRS at same power with SRS#0 so that gNB would be sure of the power UE used. If UE does not compensate the power even it could do so, due to the relaxed power tolerance at lower power levels there is some uncertainty in the actual transmitted power unless UE has a lot more accurate power setting capability than specified by 3GPP. In cases where UE cannot compensate the extra SRS IL, the benefit of the indication is small.

Observation 1: While there are some performance gains in SRS Imbalance indication, there are a lot of uncertainties and assumptions which impact the actual gain

Observation 2: Probably the largest gain is achieved in case where UE does not compensate the additional SRS IL delta even there would be power reserve remaining

As conclusion, we are leaning towards not specifying SRS IL imbalance indication. However, we will participate the discussion in Rel-19 to see is a generic solution to improve SRS is found.

- Conclusion

# SRS IL Imbalance issue was discussed with the following observations:

Observation 1: While there are some performance gains in SRS Imbalance indication, there are a lot of uncertainties and assumptions which impact the actual gain

Observation 2: Probably the largest gain is achieved in case where UE does not compensate the additional SRS IL delta even there would be power reserve remaining

- Reference

[1] RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei

[2] R4-2406585, “WF on requirements for 6Rx”, AT&T