3GPP TSG-RAN WG4#111	R4-2408857

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	6.18.2

Source: 	Qualcomm France

Title: 	n71(2A) PC2 MSD

Document for:	Approval

# Introduction

MSD for CA_n71(2A) is discussed in this contribution.

- Background

TP for 1TX PC2 CA_n71(2A) MSD was approved in RAN4#110 [1]. However, there was an another contribution [2] in Athens, which proposed simplification of PC3 requirements as well as test points for PC2. In Changsha, some companies further evaluated [2] and provided analysis results for such test points. Our paper [3] was based on approved [1].

In this contribution we evaluate test points proposed by [2].

- Discussion

The following has been specified for PC3 n71(2A):

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |

In [2], the following simplification for PC3 was considered, with the proposal to specify only 20+5 and 25+5MHz cases.

Table 4: CA_n71(2A) REFSENS one UL test point candidates.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNC (dB) | Candidate | BCS | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10.0 | 5 (RBstart = 2) | 22.2 | #1: Legacy worst-case MSD (Obs 1.1) | All | near full overlap IMD3 |
|  |  | 20MHz + 5MHz | 10.0 | 20 (RBstart = 0) | FFS | #2: Alternative to legacy (Obs.1.3) |  | IMD3 full overlap |
|  |  | 25MHz + 5MHz | 5.0 | 20 (RBstart = 8) | FFS | #3: Rel-18 UEs that support symmetrical UL/DL CBW (Obs. 1.4) | 4/5 only |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band (Obs. 1.2) |  |  |  |  |  |  |  |  |

Our views is that for PC3 no new test points applicable for BCS0/1 should be specified. We are fine with reducing the number of test points, but we’d like to keep one legacy test point for BCS0/1 in 15+10MHz case. In addition, case 25+5MHz could be specified in addition.

Proposal 1: Specify the following MSD test points for PC3 n71(2A):

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
|  |  | 25MHz + 5MHz | Wgap = 5.0 | 20 (RBstart = 8) | TBD |  |

We did analyse MSD for new PC3 test case, as well as MSD 1TX PC2 case and 2TX PC2 case. Please note that in the parameters were tuned to match the MSD in the current specifications.

Proposal 1: Specify PC3 n71(2A) MSD as follows

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
|  |  | 25MHz + 5MHz | Wgap = 5.0 | 20 (RBstart = 8) | 23.6 |  |

Proposal 2: Specify 1TX PC2 and 2TX PC2 CA_n71(2A) MSD as follows:

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap =10.0 | 5 (RBstart=2) | 25.2 | 29.6 |  |
|  |  | 20MHz + 5MHz | Wgap = 10.0 | 20 (RBstart = 0) | 26.6 | 30.9 | FDD |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |

- Conclusion

MSD for PC2 CA_n71(2A) was discussed with the following proposals:

Proposal 1: Specify PC3 n71(2A) MSD as follows:

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
|  |  | 25MHz + 5MHz | Wgap = 5.0 | 20 (RBstart = 8) | 23.6 |  |

Proposal 2: Specify 1TX PC2 and 2TX PC2 CA_n71(2A) MSD as follows:

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap =10.0 | 5 (RBstart=2) | 25.2 | 29.6 | FDD |
|  |  | 20MHz + 5MHz | Wgap = 10.0 | 20 (RBstart = 0) | 26.6 | 30.9 | FDD |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |

- Reference

[1] R4-2402466, “TP for TR38.850: FDD PC2 for n71 with DL CA_n71(2A)”, T-Mobile USA

[2] R4-2400365, “PC2 CA_n71(2A) MSD”, Skyworks

[3] R4-2405449, “n71(2A) PC2 MSD”, Qualcomm