# 3GPP TSG-RAN WG4 Meeting #111	R4-2408901

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38761 | CR | 0009 | rev |  | Current version: | V18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Formal CR for 38.761 on Lab 7 noise impact evaluation for n28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It was agreed that the noise impact should be evaluated for low bands to implement proper attenuators for probes in the MIMO OTA chamber. Lab7 completed the corresponding evaluation for n28 in RAN4 #108bis. And the evaluation result should be captured in TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add noise impact evaluation for n28 of Lab 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR does not capture comprehensive lab validation information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The corresponding draft CR is endorsed in RAN4#110bis meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES >

7.2	Noise impact in MPAC on MIMO performance

This clause includes analysis and experimental measurement results of the impact of noise in MPAC on FR1 MIMO OTA performance.

During Rel-18, several companies studied the impact of noise in MPAC on FR1 MIMO OTA performance, conducted experiments, and obtained similar observations. It was found that by adding suitable attenuators after the amplifiers in MPAC system, the impact of noise can be eliminated and the MIMO OTA measurement results can be corrected, as shown in Figure 7.2-1.

Figure 7.2-1: Measurement setup for correcting FR1 MIMO OTA test results with MPAC

The measurement results (12 orientations average in FS DMP mode) at bands n28 (780.5MHz) and n41 (2593MHz) obtained by Lab 1 are show in Figure 7.2-2. The same DUT were measured with different attenuation values. It can be observed that, as the attenuation value increases, the measurement results first get better and then become stable. At the lower frequency, the effect of the attenuation value is more obvious, and it requires a lager attenuation to make the measurement results stable. The reason is that at lower frequency, the path loss from the amplifier output to the chamber centre is smaller, and when the amplifier noise arrives at the chamber centre, it has higher power than that at higher frequency.

Adding attenuators after the amplifiers can reduce the noise and make it neglectable for FR1 MIMO OTA testing. A suitable attenuation value can be selected, e.g., for band n28, the measurement results with 20dB-attenuation and 30dB-attenuation are almost the same, but the measurement result with 10dB-attenuation is worse, which means 20dB-attenuation is enough for band n28 in Lab 1’s MPAC system.

(a) Band n28 (780.5MHz)                                                  (b) Band n41 (2593MHz)

Figure 7.2-2: Measured MIMO OTA performance with different attenuation values (in Lab 1)

Figures 7.2-3, 7.2-4 and 7.2-5 present the measurement results at band n28 (780.5MHz) from Lab 2, Lab 4 and Lab 7. Similar phenomenon can be observed.

Figure 7.2-3: Measured MIMO OTA performance at band n28 (780.5MHz)  with different attenuation values (in Lab 2)

Figure 7.2-4: Measured MIMO OTA performance at band n28  (780.5MHz) with different attenuation values (in Lab 4)

Figure 7.2-5: Measured MIMO OTA performance at band n28  (780.5MHz) with different attenuation values (in Lab 7)

Table 7.2-1 presents the measurement results at band n28 (780.5MHz) from Lab 6, RS EPRE difference between FR1 MIMO OTA system with and without 10dB attenuators are listed.

Table 7.2-1: Measured RS EPRE difference between FR1 MIMO OTA system with and without 10dB attenuators at band n28  (780.5MHz) in Lab 6

| Percentile of max TP | RS EPRE between FR1 MIMO OTA system with and without 10dB attenuators |
| --- | --- |
| @ 50% throughput | 0.59 dB |
| @ 70% throughput | 0.66 dB |
| @ 90% throughput | 0.67 dB |

Based on the observations, RAN4 reached the following agreements:

-	Labs should first exclude the impact of noise before performing MIMO OTA measurements, especially for frequency bands <1GHz.

-	Perform a low-band lab alignment activity at Band n28, to cross-validate the measurement results from different labs and ensure the validity of the data pool for requirements definition.

<END OF CHANGES >