3GPP TSG-RAN WG4 Meeting #111	R4-2408976

# Fukuoka, Japan, 20 - 24 May, 2024

Title: 	Simulation results on UE demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.8.2

Document for:	Discussion

Background

During RAN4#110b meeting, WF [1] on NR_NTN_enh_SAN_UE_demod was approved. In this contribution, we provide our update simulation results on UE demodulation requirements for NR NTN enhancements.

Simulation

PDSCH

| Case number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of HARQ Processes | SNR@70% of maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-600 | 16 | 0.8 |
| 1-2 | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-600 | 16 | 8.6 |
| 1-3 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-600 | 32 | 0.7 |
| 1-4 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-600 | 16 (4 with feedback disabled) | 1.4 |

PDCCH

| Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleave size | Shift index | DCI format | Payload (without CRC) | Propagation Condition | Antenna configuration and correlation Matrix | SNR@1% of Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200 | 120 | 132 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_0 | 42 | NTN-TDLC5-1200 | 1x1 Low | 3.1 |
| 200 | 120 | 132 | 1 | 16 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | NTN-TDLC5-1200 | 1x1 Low | 1.7 |

Proposals

In this contribution, we provide our initial simulation results on UE demodulation requirements for NR NTN enhancements.

Reference

R4-2406024, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110b, Huawei, HiSilicon