3GPP TSG-RAN WG4 Meeting #111	  R4-2409001

Fukuoka, Japan, May 20 – 24, 2024

Agenda Item:	10.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on testability and interoperability issues for beam management

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #110bis [1], we continue to discuss the remaining issues related to the test for AIML enabled beam management.

# Test metrics for beam management

[1] Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes. [2] TR 38.843: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%): the percentage of “the Top-1 strongest beam is Top-1 predicted beam” - Top-K/1 (%): the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams” - Top-1/K (%): the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams” - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

The definition of RSRP accuracy is related to the side where the AIML model is located.

- NW-sided model

For NW-sided beam prediction, the UE is required to report Top-K L1-RSRP of beams within Set B. The requirement of RSRP accuracy is the measurement accuracy which reuses RAN4 legacy.

Proposal 1: For NW-sided beam prediction, the requirement of RSRP accuracy is the measurement accuracy which reuses RAN4 legacy.

- UE-sided model

For UE-sided beam prediction, the UE is expected to either report the ranking of the predicted Top-K beam index within Set A, or report the predicted Top-K beam index along with associated predicted L1-RSRP. The content to be specified is still ongoing in other WGs.

If the UE only reports the ranking of the Top-K beams, there are two options to define the requirement:

- Option a: Do not define the requirement of RSRP accuracy, only check whether the genie-aided Top-1 beam is within the predicted Top-K beam IDs.

- Option b:

  - If the genie-aided Top-1 beam ID is within the predicted Top-K beam IDs, there is no need to check the RSRP accuracy.

  - If the genie-aided Top-1 beam ID is not within the predicted Top-K beam IDs, check the RSRP difference between the RSRP of the genie-aided Top-1 and the measured RSRPs of the predicted Top-K beams. The requirement of RSRP accuracy is defined by considering this RSRP difference.

    - In this context, the measured RSRPs of the predicted Top-K beams can be provided by measuring the RSRPs of these K beams at DUT.

If the UE supports to report the predicted Top-K beam index along with associated predicted L1-RSRP, there are two options to define the requirement:

- Option a: Do not define the requirement of RSRP accuracy. Instead, only check whether the genie-aided Top-1 beam is within the predicted Top-K beam IDs.

- Option b: Define the requirement of RSRP accuracy, where

  - Option b1: The RSRP accuracy is defined by considering the RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K

  - Option b2: The RSRP accuracy is defined by considering the RSRP difference between the RSRP of the genie-aided Top-1 beam and the predicted RSRP (predicted by DUT) of the predicted Top-K

  - Option b3: The RSRP accuracy is defined by considering the RSRP difference between the measured RSRP of the predicted Top-K beams and the predicted RSRP (predicted by DUT) of the predicted Top-K

The aforementioned Option a is exactly the Option 2 in [2], which is not reasonable since the beam measurement accuracy tolerance is not considered. For example, if the predicted Top-1 is not the genie-aided Top-1, while the difference between the RSRP (measured RSRP) of the predicted Top-K beams and the genie-aided strongest beam is within the pre-defined tolerance, it is not fair to say the DUT fails the test.

Observation 1: The test metric in Option 2 is not applicable if the beam measurement accuracy tolerance is not considered.

Proposal 2: For UE-sided beam prediction, deprioritize the test metric of beam prediction accuracy in Option 2.

Note that the main intention of AIML enabled BM is not to verify whether the RSRP is predicted with pre-defined accuracy compared with the measured RSRP of the same beam (i.e., the difference between the predicted RSRPs of the predicted Top-K beams and measured RSRPs of the predicted Top-K beams) but to verify whether the best Top-K beams can be figured out instead. From this perspective, the aforementioned Option b2 and Option b3 are not suitable.

Proposal 3: For UE-sided beam prediction, the RSRP accuracy is defined by considering the RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K.

# Test setup for AI-BM

It is observed that the performance of the model may be impacted by a set of factors, including the gNB-side beampattern (which is gNB-side proprietary information), the channel conditions (e.g., the scatter in the physical environment), the receiving gain at UE (i.e., different UE implementation, UE rotation). Therefore, whether and how to define a common training/testing dataset is an open issue.

If the training dataset is not specified, how to ensure the consistency between training and testing needs to be studied. One possible solution is to let the DUT to collect training dataset and complete the model training, followed by performing model test. However, according to R18 study outcome, the training procedure is not considered in RAN4 test due to many reasons, such as test cost/time.

Even if RAN4 revisits the training procedure, how to ensure the following principle which identified in R18 is also an open issue:

The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field. [2]

This is due to the reason that the parameters considered in the test set up which limit the model generalization may be totally different from real deployment. In this case, a DUT has an AI model to pass RAN4 test does not mean it can work well in the field.

Proposal 4: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to ensure that a UE can pass the test but perform poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

After achieving clear answers of the aforementioned questions, we can start to discuss how to define the setup, where the exiting FR2 OTA test set up should be considered as the baseline. Any enhancements on top of existing fancy enough framework should be justified.

Proposal 5: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For NW-sided beam prediction, the requirement of RSRP accuracy is the measurement accuracy which reuses RAN4 legacy.

Observation 1: The test metric in Option 2 is not applicable if the beam measurement accuracy tolerance is not considered.

Proposal 2: For UE-sided beam prediction, deprioritize the test metric of beam prediction accuracy in Option 2.

Proposal 3: For UE-sided beam prediction, the RSRP accuracy is defined by considering the RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K.

Proposal 4: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to ensure that a UE can pass the test but perform poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

Proposal 5: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

# References

- R4-2406617, “WF on AI/ML”, 3GPP RAN4 #110bis, 15th – 19th April, 2024.

- RP-233133, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, 3GPP TR 38.843, V2.0.0, December 11-15, 2023.